3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240081

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	Evaluation and conclusion on KI#6

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	zhengshaowen@chinamobile.com

1. Introduction

This paper provides evaluation and conclusion for KI #6: XR EAS selection based on N6 delay measurement.

2. Reason for Change

The evaluation and conclusion for KI #6 is needed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change (all new text) * * * *

9.3.X	Key Issue #6 solutions evaluation

Key issue #6 studies the XR EAS selection enhancement for reducing service latency. There are three solutions addressing the related issues: solution #10, solution #12 and solution #X.

-	Solution #10 proposes that the ECS obtains predicted application session connectivity information, and the ECS determines EES considering the closeness of the EES endpoint and the predicted application session connectivity information.

-	Solution #12 proposes to add EAS UPF information (i.e. UPF information related to the EAS) to EAS profile. The EES selects EAS based on the EAS related UPF information and the UE serving UPF information.

-	Solution #X proposes to utilizes N6 delay measurement mechanism which has already been supported by SA2. After candidate EAS(s) discovery is performed, the EES sends request to 5GC for N6 delay measurement for each pair of candidate XR EAS and UPF related to DNAI of the XR EAS. The EAS is selected based on the measured N6 delay.

Solution #12 and solution #X propose to enhance the EAS discovery procedure, while solution #10 proposes to enhance service providing procedure.

In order to reduce the service latency, solution #12 proposes to identify the related UPF for each EAS to select the potential optimal EAS, while solution #X proposes to request N6 delay measurement. The service latency may be varied, which causes that the UPF with the lowest service latency to a certain EAS may not be always the same one. Therefore, the service latency should be measured, and the measurement of the service latency or the UPF information with the lowest service latency to each candidate EAS should be provided to the EES.

* * * Next Change (all new text) * * * *

10.X	Conclusions of key issue #6

It is agreed to adopted the following principles in the normative work:

- The EES selects XR EAS based on service latency (e.g. N6 delay), which can be quantitatively measured.

- The EES obtains the following information per XR EAS: the measurement of the service latency to each UPF, or the UPF information with the lowest service latency to the XR EAS.

- Prediction information may be used for XR EAS selection if needed. The detail of such information and how to obtain such information should be defined in normative work.

* * * End of Change(s) * * * *