3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240104

10th – 18th July 2024	(revision of S6a240xxx)

Source:	China Mobile

Title:	Pseudo-CR on Architecture of TS23_392

Spec:	3GPP TS 23.392 v0.0.0

Agenda item:	9.12

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes text for TS 23.392 Architecture. The text is based on clause 8.1 of TR23.700-92.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.0.0.

* * * First Change * * * *

6.	Architecture

Editor's Note: This clause provides the architecture of MMTelAPP Enabler.

6.1	General

This  clause specifies the architecture of eMMTel Enabler.

6.2	eMMTel Enabler architecture

Figure 6.2-1 shows the eMMTel Enabler architecture.

The eMMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: eMMTel Enabler architecture

The eMMTel functional entities on the UE and the server sides are eMMTel client and eMMTel Enabler server respectively.

The eMMTel client communicates with the eMMTel Enabler server over the eMMTel-1 reference point to support the necessary application layer interactions needed in eMMTel service.

NOTE 1	The interaction between eMMTel Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The eMMTel client also communicates with the DCMTSI client specified in 3GPP TS 26.114 [26114] over the eMMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the eMMTel Enabler Server.

The Application Server communicates with the eMMTel Enabler server over the eMMTel-2 reference point to invoke the eMMTel service related capabilities provided by eMMTel Enabler server,

The Controlling Application Server communicates with the eMMTel Enabler server over the eMMTel-3 reference point to manage the necessary information needed in eMMTel service.

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

NOTE 2:	The functionalities of the Application Server, Application Client and the App-1 reference point are out of scope of the present document.

6.3	Functional entities

6.3.1	General

The functional entities for eMMTel Enabler are described in the following subclauses.

Editor's note:	 The functionalities of the eMMTel client and the eMMTel Enabler server are FFS and to be updated per solution.

6.3.2	eMMTel Enabler Server

The eMMTel Enabler Server provides the server side functionalities corresponding to the eMMTel service.

The eMMTel Enabler Server supports:

-	the Application Server and Controlling Application Server to discover, to authenticate and to get authorization the MMTel/eMMTel service APIs provided by the eMMTel Enabler server;

-	the interaction with Controlling Application Server to configure the data channel application profile;

-	the integration of the capabilities provided by the underlying network, including the 3GPP core network and IMS. The eMMTel Enabler Server re-exposure of these capabilities via MMTel/eMMTel service APIs to the Application Server and Controlling Application Server. The capabilities provided by the underlying network include:

a)	the APIs provided by DCSF via DC3/N33 reference point;

b)	the APIs provided by DCSF via DC4 reference point; and

c)	the APIs provided by MF/MRF via MDC3/MDC4 reference points;

-	the communication with the corresponding functions in the underlying network on behalf of the Application Server and Controlling Application Server when the capabilities provided by the underlying network are needed to be invoked;

-	the interaction with eMMTel client for the necessary application layer interactions needed in eMMTel service,

-	providing MMTel/eMMTel specific value added services which can be used by the Application Server in the mmtel communication between UE and Application Server;

-	the interaction with SEAL functionalities to use the corresponding SEAL services in eMMTel service, e.g. to use  SEAL notification management service to update of data channel application profiles to UE;

-	the storage of the application layer information associated with the eMMTel services;

-	the media function to support the necessary media handling/translation, e.g. acts as a media gateway of WebRTC or if supported media type of application is different with the UE; and

-	the HTTP Proxy to support the establishes a Bootstrap data channel and application data channel, e.g. if the address type of application is different with the UE, e.g. IPv4 and IPv6, etc.

Editor's note:	Whether other functionalities, e.g. function to handle eMMTel service related Signalling, are needed in the eMMTel Enabler Server is FFS.

6.3.3	eMMTel Client

The eMMTel Client provide the client side functionality needed in eMMTel service.

The eMMTel Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the data channel application profiles provided by the eMMTel Enabler Server.

NOTE 1:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications.

NOTE 2	The interaction between eMMTel Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

6.3.4	Application Server

The Application Server communicates with the eMMTel Enabler server over the eMMTel-2 reference point to invoke the eMMTel service related capabilities to communicate with the UE. It may also communicate with Application Client to provide Application specific logic.

NOTE:	The functionalities of the Application Server is out of scope of the present document.

6.3.5	Controlling Application Server

The Controlling Application Server is a specific Application Server that only used by the eMMTel service provider, i.,e. MNO, to manage the necessary information, e.g. data channel application profiles, needed in eMMTel service. It communicates with the eMMTel Enabler server over the eMMTel-3 reference point to invoke the eMMTel service related capabilities and does not interacts with Application Client on the UE directly.

NOTE:	The functionalities of the Controlling Application Server is out of scope of the present document.

Editor's note:	 Whether the controlling AS functionalities can be provided by OAM or not is FFS and may require coordination with SA5.

6.3.6	Application Client

The Application Client is an entity in the application layer to implement and perform the application service logic for its own service.

NOTE:	The Application Client can be the data channel application client. The functionalities of the Application Client is out of scope of the present document.

6.4	Reference points

6.4.1	General

The reference points for eMMTel Enabler are described in the following subclauses.

Editor's note:	 The functions of the reference points are FFS and to be updated per solution.

6.4.2	eMMTel-1 (between the eMMTel client and the eMMTel Enabler server)

The eMMTel-1 reference point supports the interactions between a eMMTel client and the corresponding eMMTel Enabler server. This reference point supports the necessary application layer interactions needed in eMMTel service.

6.4.3	eMMTel-2 (between the eMMTel Enabler server and the Application Server)

The interactions between the Application Server and the underlying network are provided by the eMMTel Enabler server via eMMTel-2 reference point to ensure the security access of the underlying network. This reference point supports the Application Server to discover eMMTel service APIs, to authenticate, to get authorization and to communicate with the eMMTel service.

6.4.4	eMMTel-3 (between the eMMTel Enabler server and the Controlling Application Server)

The interactions between the eMMTel Enabler server and the Controlling Application Server are referred to eMMTel-3 reference point. This reference point supports the Controlling Application Server to manage the necessary information needed in eMMTel service.

6.4.5	App-1 (between the Application Client and the Application Server)

The App-1reference point supports the communication between  Application Client and Application Server to provide the Data Channel Application specific logic.

NOTE:	The functionalities of the App-1 reference point is out of scope of the present document.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[26114]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".