3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240112

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Samsung

Title:	Pseudo-CR on adding solution#2 evaluation

Spec:	3GPP TR 23.700-01 V1.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

Adding solution evaluation.

2. Reason for Change

Adding solution evaluation for Solution#2 - Enhancement for on board EES(s) and service provisioning and other minor edits.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V1.0.0.

* * * First Change * * * *

### 7.2.2	Solution #AE2: Enhancement for on board EES(s) and service provisioning

#### 7.2.2.1	Solution description

This solution proposes to address the following open issues in KI#5,

-	How the EES(s) are placed on board the Satellite.

-	Whether and how the service provisioning is impacted.​

It is proposed to add the Satellite assistant information in the EES profile to indicating the EES(s) are placed on board the satellite, so that, the ECS could do the service provisioning based on those Satellite assistant information.

##### 7.2.2.1.1	General

When EES is deployed in the satellite, it is proposed to add the Satellite assistant information in the EES Profile to indicating the EES is deployed on-board. The Satellite assistant information could be the satellite ephemeris information(e.g., time slot and spatial location). So that the ECS could obtain the EES’s Satellite assistant information during the EES registration.

Table 7.2.2.1.1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EDN information | O | EDN information where the EES resides. |
| > DNN | M | Data network name to identify the EDN. |
| > DNAI(s) | O | DNAI(s) associated with the EDN. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| >Allowed MNO information | O | Information of the allowed operator as described in EAS profile clause 8.2.4, Only subscribers from these operators can consume the EES services. |
| List of EAS bundle information | O | List of EAS bundles per EASID to which the EAS belongs and related bundling requirements. |
| > Bundle ID (NOTE 2) | O | A bundle ID as described in clause 7.2.10. |
| > List of EASIDs (NOTE 2) | O | List of EASIDs associated with the EAS bundle. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (NOTE 1) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability (e.g. EAS bundle information) of handling bundled EAS ACR. |
| Satellite assistant information | O | Assistant information indicating the EES is on-board, and could be used to calculate the satellite's position and movement. It could be the statistic satellite ephemeris information (e.g., signal quality metrics and orbital elements) |
| NOTE 1: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". NOTE 2: At least one of the IEs shall be present if EAS bundle information is provided. |  |  |

After that, when UE requests the service provisioning, the following changes may apply.

Pre-condition:

1.	The ECS support the availability information calculation.

Figure 7.2.2.1.1-1: Service provisioning

-	In step2, the ECS calculates the EES availability information based on the Satellite assistant information, and determines the EES based on the EES availability information so that the EES is accessible to the UE. The EES availability information indicates when and how the EES is available for the requesting EEC. It could be a sequence of time durations for each grid point where each time duration includes an indication of coverage availability or unavailability as discussed in Annex Q of 3GPP TS 23.501 [5].

-	In step3, if the EES meet the requirement so that the on-board EES is selected by the ECS, the ECS should send the EES availability information to the EEC. The EEC can select one or more such EES(s) to perform EAS discovery based on EES availability information.

#### 7.2.2.2	Architecture Impacts

This solution has no architecture impacts to EDGEAPP. Under the existing EDGEAPP architecture, the service provisioning procedure is enhanced to support different cases in service continuity.

#### 7.2.2.3	Corresponding APIs

See clause 7.2.2.1.1 Table 7.2.2.1.1-1: EES Profile.

#### 7.2.2.4	Solution evaluation

This solution is for KI#5 and addresses the open issue about deployment option for EDGEAPP and related enhancements to the EDGEAPP service provisioning. The solution does not required new architecture entity in EDGEAPP.

The solution proposes ECS is deployed on ground while EAS and EES are deployed on satellite. In this solution the Satellite assistant information in EES profile is used by ECS to calculate EES availability information for the EEC requesting service provisioning. ECS then sends the EES availability information to the EEC.

The solution is feasible, for consideration to normative.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>