3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240113

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Samsung

Title:	Pseudo-CR on adding solution#3 evaluation

Spec:	3GPP TR 23.700-01 V1.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

Adding solution evaluation.

2. Reason for Change

Adding solution evaluation for Solution#3 - Support discontinuous coverage for satellite access and other minor edits.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V1.0.0.

* * * First Change * * * *

### 7.2.3	Solution #AE3: Support discontinuous coverage for satellite access

#### 7.2.3.1	Solution description

This solution addresses the KI#3: Satellite access with discontinuous coverage.

This solution shows how the application enabler obtaining the satellite related information (e.g., ephemeris) from the satellites, retrieving unavailability period for UE with satellite access then exposing to the VAL UE and the VAL server.

This solution is based on the following assumptions and principles:

-	This solution main serves for IoT services and take the SEAL as the application enabler.

-	The SEAL server has obtained the ephemeris for Satellites.

-	The SEAL server is aware of VAL UE’s location.

In this solution, it’s assumed the SEAL server has obtained the satellite related information (e.g. ephemeris) from the satellite information provider (e.g. satellite service provider or the operators) and based on these information, the SEAL server could generate the satellite coverage availability information which can be indicated to the UE whether or not coverage is available for a specific satellite RAT Type for a particular location and time. The example of coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [5]. And based on clause 5.4.13.2 of 3GPP TS 23.501 [5], the coverage availability information can be provided to a UE by an external server. That means the SEAL server could provision UE the coverage availability information if UE is using the satellite access.

From the ephemeris and Satellite coverage availability information, the SEAL server could retrieve the unavailability period (e.g., duration and start time) based on UE’s location and then provide to the VAL UE and expose to the VAL server to indicate them to consider the unavailability period when the VAL UE is using satellite access.

The clause of 7.2.3.1.1 specifies the SEAL server generates or formats the Satellite coverage availability information based on ephemeris from the satellite information provider and retrieve the unavailability period (e.g., duration and start time) based on UE’s location and expose it to the VAL UE and VAL server to guild their following actions.

##### 7.2.3.1.1	Procedure of SEAL Server generating Satellite coverage availability and unavailability period information

Pre-condition:

-	SEAL Client has registered on SEAL server.

-	The VAL server asks SEAL Server to provide VAL UE’s information (e.g. location data) periodically.

-	The SEAL server is aware of the VAL UE’s location information.

Figure 7.2.3.1.1-1: Procedure of SEAL Server generating Satellite coverage availability and unavailability period information

0. 	Assume the SEAL server has obtained the ephemeris for Satellites.

1.	The SEAL server may generate the satellite coverage availability information based on the obtained the ephemeris for Satellites and the VAL UE’s location data or shape them in a unified format in case it receives the satellite coverage availability information directly from different 3rd parties (e.g., different Satellite companies). The example of satellite coverage availability information can refer to the Annex Q of 3GPP TS 23.501 [5].

2.	As the satellite coverage availability information could indicate to the UE whether the coverage is available for a particular location and time, the SEAL server configures the satellite coverage availability information to the VAL UE/ SEAL Client via satellite coverage availability information configuration request.

3.	The SEAL Client acknowledges the SEAL server if it supports the satellite access.

4.	Based on the ephemeris for satellites and Satellite coverage availability information, the SEAL server also could retrieve the unavailability period (e.g., unavailability period duration and start time for the unavailability period) to guild UE when it could trigger the UL service flow, in case the UE wants to get the unavailability period information directly.

5.	The SEAL server sends the unavailability period information to the VAL UE/SEAL Client via unavailability period information configuration request.

6.	The SEAL Client acknowledges the SEAL server if it supports the satellite access. And the SEAL Client will operate considering the unavailability period information (e.g. not send UL messages during the unavailability period).

7.	If the VAL server asks to provide VAL UE’s information (e.g. location data) periodically, the SEAL server may notify the VAL server the unavailability period information of VAL UE which could indicate VAL server how to operate to minimize the service impact (e.g., extended data buffering, pending downlink data transferring).

##### 7.2.3.1.2	Procedure for AS handling UE unavailability period information

Pre-condition:

-	Satellite coverage availability information (SCAI) is not yet configured on the UE or not supported by the UE.

-	5GC is aware of SCAI and UE location.

Figure 7.2.3.1.2-1: Procedure for AS handling UE unavailability period information

1.	UE does not support SCAI provisioning or UE has not received SCAI configuration from the 5GC. Hence, cannot provide UE unavailability period to the AS for operating gracefully under a discontinuous coverage pattern.

2.	AS (e.g. SEAL server) sends get UE unavailability request to the 5GC, including the UE/User identity and service identity for which UE has satellite access. AS can subscribe with 5GC to loss of coverage event subscription to get notified about the unavailability period or unavailability duration when certain events occur e.g. start of unavailability period or before certain duration in a particular location, as described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

3.	5GC (e.g. NEF) checks if UE supports satellite access and then determines UE unavailability e.g. based on SCAI information and UE location.

4.	5GC sends get UE unavailability response including the UE unavailability information to the AS. Or when an event is triggered in the 5GC corresponding to the subscription in step 2, 5GC notifies to AS the unavailability period information e.g. duration/start time, as described in clause 5.4.1.4 of 3GPP TS 23.501 [5].

5.	AS (e.g. SEAL server) sends the unavailability period information to the AS (e.g. VAL server), if required.

6.	AS (e.g. SEAL server) uses the UE unavailability information for adjusting the application behaviour e.g. to trigger the downlink data delivery, bulky data exchanges can be deferred and planned during longer connectivity intervals, giving precedence to data suiting short connectivity intervals.

Editor's note:	Co-ordination may be required with SA2.

#### 7.2.3.2	Architecture Impacts

This solution has no architecture impacts to SEAL. Under the existing SEAL architecture, the SEAL procedures are enhanced to support satellite discontinuous coverage.

#### 7.2.3.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

#### 7.2.3.4	Solution evaluation

This solution is for KI#3 and addresses the open issue about leveraging SEAL architecture to obtain the satellite related information and to retrieve unavailability period for UE, for supporting satellite discontinuous coverage. The solution does not required new architecture entity in SEAL.

The solution proposes how SEAL can obtain the satellite coverage availability information and provided it to the UE. In this solution SEAL retrieves the UE unavailability information either from the UE or from the 5GS. Further the solution proposes VAL server obtaining the UE unavailability period information which indicate how to minimize the service impact or adjust the application behaviour e.g. to trigger the downlink data delivery, bulky data exchanges can be deferred and planned during longer connectivity intervals, giving precedence to data suiting short connectivity intervals.

The solution is feasible, for consideration to normative.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>