3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240129

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Ericsson

Title:	IE alignement

Spec:	3GPP TS 23.700-01 v1.0.0

Agenda item:	8.5

Document for:	Approval

1. Introduction

This paper introduces IE alignement for sol#1.

2. Reason for Change

This paper introduces IE alignement for sol#1.

3. Conclusions

This pCR proposes IE alignment.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-01.

* * * First Change * * * *

##### 7.2.1.1.4	Service provisioning  for EES on board satellite

Similar to EAS on board a MEO or LEO satellite, the EES will also be mobile when the EES is also on board a MEO or LEO satellite. Therefore considering the trajectory of the EES is important for efficient service service provisioning for selecting EDN matched with movement of the AC hosting UE during the AC’s operation schedule.

1.	Dynamic service area indication: Indicates if the service area is dynamic or not. The service area is dynamic in case of EES on board a MEO or LEO satellite and not dynamic in case of a EES on board a GEO satellite or EES on ground.

2.	Trajectory ID: to indicate the trajectory of the EES on board MEO and LEO satellites. . This trajectory ID is unique for each satellite and its trajectory.

NOTE 1:	It is up to the satellite service provider or operator to associate the Trajectory ID with the actual trajectory of the satellite. The trajectory and position of the satellite can be calculated based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris.

NOTE 2:	The generation and assignment of the trajectory ID for the EES(s) is done by the operator, the satellite service provider.

The "Expected AC Geographical Service Area" in the AC profile as in 3GPP TS 23.558 [10] clause 8.2.2 can be used by the ECS to request the external  server (by the satellite service provider or operator) with list of trajectory IDs that match the UEs planned route and its location. The ECS then uses the list of the trajectory ID for the UE to provision EES(s) by matching the trajectory IDs in the EES profile with the list of the trajectory ID determined for the UE according to its position and its planned route. Furthermore, the list of the trajectory IDs are provided in the service provisioning response to be used for further EAS discovery as per clause 7.2.1.1.3.

# *** End of Changes ***