3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240132

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Ericsson

Title:	Solve EN in sol#4

Spec:	3GPP TR 23.700-72 v1.0.0

Agenda item:	8.1

Document for:	Approval

Contact:	Wenliang Xu (wenliang.xu@ericsson.com)

1. Introduction

This pCR provides EN resolution in sol#4.

2. Reason for Change

If the reference UE or target UE is losing connectivity, there is no need to consume system resource to monitor the other UE’s activity.

The Geofencing monitoring should be suspended and resumed later accordingly, and the EN can be removed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72.

* * * First Change * * * *

### 6.4.1	Description

The following procedure is an example of LM provided service to monitor UE location in relation to Area of Interest (AoI), it supports the following additional functions comparing to clause 9.3.11.2 of 3GPP TS 23.434 [9]:

-	the AoI consists of a reference UE and proximity range from the reference UE.

NOTE 1:	The proximity range is an application defined proximity range.

-	multiple target UEs can be specified by the VAL server.

-	the VAL server can specify a temporal, connectivity and/or a spatial condition(s) for the reference UE in the monitoring location subscription. For instance, the monitoring is valid when a reference UE is served by operator A without using satellite in location area X from 7:00am to 9:00am on working days.

NOTE 2:	The VAL server can be aware of UE/user’s preference via communication with AC, which is out of the scope of 3GPP.

-	the VAL server can request LM server to suppress uninterested notifications (e.g. Notify Presence).

-	the SEAL LM server can notify the VAL server (e.g. Notify Unknown) if the target UE location information cannot be obtained (e.g. UE power off).

Figure 6.4.1-1: Dynamic Geofencing support procedure

1.	The Geofencing subscription request from the VAL server is same as step 1, clause 9.3.11.2 of 3GPP TS 23.434 [9]. For AoI related to the target UE(s), a reference UE and its proximity range are included. The VAL server may include interested notification information (e.g. Notify Absence and Notify Unknown), a temporal condition(s) (e.g. time of day and day of week), connectivity condition(s) (e.g. operator information, roaming status, access type) and/or a spatial condition(s) (e.g. location area(s)) for the reference UE. The target UE information under monitoring can be a list of UEs.

NOTE 3:	Geofencing subscription service re-uses Monitor Location subscription service.

2.	The LM server requests 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [7]) to monitor the reference UE location change, access type change and roaming status change, in order to determine spatial dynamicity in AoI related to the target UE(s). If spatial condition is received, the LM server also requests 3GPP CN (i.e. NEF service for AoI monitoring as specified in 3GPP TS 23.502 [7]) to know whether the reference UE is in the VAL server’s predetermined area (e.g. PLMN x). The LM server will process the Geofencing monitoring considering the received temporal, connectivity and/or spatial condition(s) for the reference UE.

NOTE 4:	For temporal condition with time of day and day of week, the LM server supervises the time since it is not supported by UE location service in CN.

3.	In order to know UE status (e.g. purged or detached) in 3GPP CN, the LM server requests 3GPP CN (i.e. NEF service for loss of connectivity as specified in 3GPP TS 23.502 [7]) for both target UE(s) and reference UE.

4a-4c.	Same as step 2, 3 and 4, clause 9.3.11.2 of 3GPP TS 23.434 [9]. If multiple target UEs are received from the VAL server, step 4a-4c are performed for each target UE.

5.	Same as step 5, clause 9.3.11.2 of 3GPP TS 23.434 [9].

6a. Same as step 6a, clause 9.3.11.2 of 3GPP TS 23.434 [9].

6b.	If the location information is matching as a result of step 6a, then the LM server checks if the VAL UE's current location is within the proximity range of the reference UE.

7.	Same as step 7, clause 9.3.11.2 of 3GPP TS 23.434 [9].

8.	If the location information is matching as a result of step 6a, and the target UE is not within the proximity range of the reference UE, then the LM server considers the target UE as outside from the AoI related to the target UE(s) and notifies the VAL server that the target UE's current location is outside of AoI and target VAL UE ID in "Notify Absence" message.

9.	If the location information is matching as a result of step 6a, and the target UE is within the proximity range of the reference UE, then the LM server notifies ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the target UE is within the AoI, along with the target UE's current location information.

10.	If the LM server is notified by the 3GPP CN about loss of connectivity for the reference UE or target UE(s), then the LM server notifies ("Notify Unknown" message) the VAL server indicating that the LM server does not know whether the target UE is "Present" or “Absence", therefore the Geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 5:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope. If the reference UE loses connectivity, the LM server can stop event monitoring for the target UE. If the target UE loses connectivity, the LM server can stop event monitoring for the reference UE. When the connectivity issue is solved (e.g. UE registered in 3GPP CN), the Geofencing monitoring process resumes (with resumed event monitoring for reference UE or target UE) and the VAL server will be notified accordingly with "Present" or “Absence".

NOTE 6:	Notifications in step 7-10 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown" in Geofencing.

* * * End of Changes * * * *