3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240134

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Ericsson

Title:	KI#8 overall evaluation

Spec:	3GPP TS 23.700-23 v1.0.0

Agenda item:	8.5

Document for:	Approval

1. Introduction

This paper introduces overall evaluation for KI#8.

2. Reason for Change

This paper introduces overall evaluation for KI#8.

3. Conclusions

This pCR proposes overall evaluation for KI#8.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23.

* * * First Change * * * *

### 9.3.x	Key issue #8 solutions evaluation

The open issue of KI#8 include:

-	Whether and how SEALDD or other application enabler may be enhanced to facilitate XR communication flows between application clients and application servers involved in the same application service in relation to PDU set (e.g., support for PDU set handling, QoS measurement)?

In the existing solutions, solution#4 addresses QoS measurement for PDU set on crossflow RTT. The SEALDD measurement function is enhanced to identify the 1st UL and last DL packet in a PDU set.

In the existing solutions, solution#7 addresses PDU set handling support. The SEALDD layer is enhanced with RTP packetilization and PDU set inclusion.

# *** End of Changes ***