3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240150

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Samsung

Title:	Pseudo-CR on update to solution #6, and conclusion for KI 7

Spec:	3GPP TR 23.700-21 (v1.0.0)

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR updates solution #6 by providing corresponding APIs and solutiong evaluation. Further, the pCR provides overall evaluation for KI #7 and concludes KI #7.

2. Reason for Change

It is requird to provide corresponding APIs and solutio evaluation as per the template.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 (v1.0.0).

* * * First Change * * * *

#### 7.2.6.3	Corresponding APIs

Table 7.2.6.3-1 specifies information elements for MSGin5G update stored message parameter request message.

Table 7.2.6.3-1: MSGin5G update stored message parameter request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Message ID | M | Unique identifier of the stored message for which parameters needs to be updated |
| Store and forward parameters | O | Updated parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2 of 3GPP TS 23.554 [14]. |

Table 7.2.6.3-2 specifies information elements for MSGin5G update stored message parameter response message.

Table 7.2.6.3-2: MSGin5G update stored message parameter response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE 1) | O | The reason for failure |
| NOTE 1: This IE shall only be present when the result is Failure. |  |  |

Table 7.2.6.3-3 specifies information elements for MSGin5G discard stored message request message.

Table 7.2.6.3-3: MSGin5G discard stored message request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Message ID | M | Unique identifier of the stored message for which parameters needs to be updated |
| Store and forward parameters | O | Updated parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2 of 3GPP TS 23.554 [14]. |

Table 7.2.6.3-4 specifies information elements for MSGin5G discard stored message response message.

Table 7.2.6.3-4: MSGin5G discard stored message response

| Information element | Status | Description |
| --- | --- | --- |
| result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE 1) | O | The reason for failure |
| NOTE 1: This IE shall only be present when the result is Failure. |  |  |

#### 7.2.6.4	Solution evaluation

This solution addresses key issue #7 by utilizing store and forward events from SA2, and deciding whether to update the stored and forward parameters or the MSGin5G message or discard the stored message. The solution is depends on SA2 progress. The solution is feasible solution.

* * * Next Change * * * *

### 10.4.7	Key issue #7: Usage of S&F events information for the application enablement

Solution #AE6 addresses open issues 1 and 2 of thes key issue #7. Solution #6 proposes to utilize S&F events for MSGin5G service and optimizes the behaviour by deciding to update or discard already stored message.

* * * Next Change * * * *

### 11.3.y	Conclusions of key issue #7

Solution #AE6 can be considered for normative work to update MSGin5G service. The solution depends on progress of SA2 work.

* * * End of Change * * * *