3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240152

10th – 18th July 2024	(revision of S6a240xxx)

# Source:	Huawei

# Title:	SEAL Phase4 on further enhancement for APP friendly

# Document for:	Approval

# Agenda Item:	10

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: SID of SEALPhase4 on further enhancement for APP friendly

Acronym: enhSEAL

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940024 | SEAL_Ph3 | {optional free text} |

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

3	Justification

Network capabilities exposure is one of important topics of 3GPP. Lots of network APIs were defined by SA2, SA4, and SA5 from 4G phase and beyond. In the specifications defined by these WGs, those APIs can be categorized into two types: one is the NF APIs exposed by Network functions like PCF, AMF, and MB-SMF…., and the other is the SCEF/NEF APIs exposed by network exposure function (See figure1). The NF APIs are defined in a network centric way i.e., one API can serve many use cases to simplify the implementation and save cost of Network Function. Most of the NEF APIs follow the same pattern of NF APIs, i.e., the inputs and outputs are the same as the NF APIs).

NRM

Provided third party

Network Function（e.g 5GC NFs）

NRM

Provided by MNO

Trusted domain+NEF

NEF/SCEF/EGMF

Untrusted AF

NRM service APIs

NEF APIs

NF APIs

NF APIs

Trusted AF

Network Function (e.g 5GC NFs)

Trusted AF

Untrusted AF

NEF/SCEF/EGMF

NEF APIs

NF APIs

NF APIs

Figure 1 SA2 defined Network exposure architecture

NEF service APIs

NEF

NRM service APIs

Figure 2   Core Network APIs abstraction by NRM

Since Rel16, SA6 has started the works on SEAL to ease the usage of 5G system capabilities by APP developers. As figure 2 shows, NRM as one services server in SEAL provides service Layer services aimed at providing developer-friendly APIs for APP domain to use the services exposed by network layer entities like SCEF/NEF or by PCF, AMF, MB-SMF and other Network entities (to authorized AFs). These service Layer services provide value-add functionality including abstraction, optimization, aggregation, etc. for both MNOs and 3rd parties.

There are two type of SEAL services: 1) MNO Network capabilities/resource only required services. 2) Commination of terminal capabilities/resource and MNO network capabilities/resource. The difference between them is that the  MNO Network capabilities/resource only required services requires only 4G/5G network capabilities, while other SEAL services mainly take the UE capabilities as basis, which may additionally use 4G/5G network capabilities.

Now more and more industry promotion organizations (e.g., CAMAR/GSMA Open Gateway) realize the importance of APP friendly, and start to investigate how to simplify network APIs and transform them to be more friendly APIs to APP developers. In addition, it is also an important topic in 5GACIA about how vertical applications can well use network layer exposed capabilities. MNO Network capabilities/resource only required services in NRM is exact a good technical choice to serve their motivation. However there is still some aspects needs to be improved from SA6 perspective.

- Low external visibility of the special value of NRM and other SEAL services, incorrect impression of the relationship of different SEALs services, due to all services being incorporated into SEAL specifications, lack of Usage instructions for each APIs (e.g. Valued use case and value added, Parameters mapping between 3GPP parameter and APP domain‘s parameters, APP impacted).

- Some NRM APIs are simplified compared with the original network APIs, but there are still room for further simplification and aggregation, e.g network APIs for XR and smartly factory especially newly added by Core Network after rel18 has not been covered yet. Some existed NRM services may be further improved by absorbing other network services to provide a larger granularity services. For example, One NRM service APIx1 originally transformed from NF-APIx+ NF APIy can be potential enhancement by combination of NF APIx+ NF APIy+ NF APIz.

- The content related to the option that the NRM is deployed by operators, i.e., NRM is owned by the MNO and deployed in the same trust domain of 5G network system) is not well reflected and highlighted in the TS. When NRM is deployed by MNO, it can invoke NF API from network functions without NEF, this option is absent from several services such as BDT, QoS/resource management for network-assisted UE-to-UE communication Some content in the TS may cause misleading of the potential value and different business deployment options of NRM.

Considering the XRAPP WID can potential cover the NRM enhancement as bullet b) mentioned, this WID does not include the NRM enhancement for XR.

4	Objective

Objectives of this WID include the following：

1.	Creating a new NRM dedicated TS and moving the NRM related content from TS 23.434 to the new specification.

2.	Improving the content of NRM dedicated TS and TS 23.434 to support APP developer friendly usage:

a.	Re-construct the content about the option that SEAL service server deployed by PLMN operators and wording improvement if needed.

b.	Review existed SEAL services and API parameters from APP developer perspective e.g. Valued use case and value added, Parameters mapping between 3GPP parameter and APP domain‘s parameters, APP impacted, seek the potential enhancements

c. Review the lasted version of 5G network capabilities related with Smart factory in TS 23.501.TS 23.502, TS23.503, 23.288 (e.g. TSC/TSN), update NRM services accordingly.

d.	Revisit the existed SEALAPI(s) and further optimize them when necessary (e.g., Gaps and the issues identified by industry projects like GSM OPG/Open Gateway, 5G ACIA).

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| New | TR 23.XXX | SEAL enhancement g for APP friendly usages | TSG#106 | TSG#106 |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.434 | Enhancements to SEAL Phase 3 for moving the content related to abstraction of 3GPP Network CN API(s) to new TS | SA#107(March 12, 2025) |  |

6	Work item Rapporteur(s)

Yang, Yanmei, Huawei Technologies Co.,Ltd.

7	Work item leadership

SA6

8	Aspects that involve other WGs

CT3 for stage3 implementation

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Huawei Technologies Co Ltd

Hisilicon

Samsung?

CMCC?

CATT

China Telecom

Convida Wireless LLC?

Apple?

Lenovo?

Deutsche Telekom?

Ericsson?

Nokia？

AT&T？