3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240155

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Huawei, Hisilicon

Title:	Evaluation for solution#7

Spec:	3GPP TR 23.700-22 v0.3.0

Agenda item:	8.7

Document for:	Approval

Contact:	Jian Zhang (zhangjian369@huawei.com)

1. Introduction

This paper is proposed to provide the evaluation for solution#7.

2. Reason for Change

Solution#7 solves the API based activation during service API discover procedure. For the AEF instance is already instantiated, the CAPIF core fucntion can request the API management function to influence the required service API to be active when it is inactive before.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.3.0.

* * * First Change * * * *

6.8	Solution #7: API based activation for service API discover

6.8.1	Solution description

6.8.1.1	General

This paper proposes a solution for key issue #7 on CAPIF enhancement for service API status, to introduce the API based activation in service API discover procedure.

The API can be activated during the service API discover, if the required servcie API is inactive without API interface details, and the related AEF is already instantiated.

6.8.1.2	Procedure

The following text captures the solution by describing the neccesary changes in bold font compared with TS 23.222 v18.3.0 as shown below:

* * * Enhancement based on TS 23.222 v18.3.0 * * * * #### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 1); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, Service KPIs (optional).. Shareable information; O (see NOTE 2); Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained.. NOTE 1: The interface details shall be not present, if the API status is inactive. NOTE 2: If the shareable information is not present, the service API is not allowed to be shared.. * * * Enhancement based on TS 23.222 v18.3.0 * * * * ### 8.7.3 Procedure Figure 8.7.3-1 illustrates the procedure for discover service APIs. The service API discover mechanism is supported by the CAPIF core function. Pre-conditions: 1. The API invoker is onboarded and has received an API invoker identity. 2. The CAPIF core function is configured with a discover policy information (e.g. to restrict discover to category of APIs) for API invoker(s). 3. The CAPIF core function obtains the service API information with the corresponding API status through service API publish procedure. Figure 8.7.3-1: Discover service APIs 1. The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity, and may include query information. 2. Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication). The CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, the CAPIF core function applies the discover policy and performs filtering of service APIs information retrieved from the CAPIF core function. 3-4. If the required service API is inactive, the CAPIF core function may request the AMF (i.e. API management function) to influence the require service API to be active, the CAPIF core function can obtain the required service API information including the API interface details after the API is activated by AMF. NOTE 1: This solution assumes that the AEF instance is already instantiated. 5. The API publish update procedure is performed, with the updated API status (i.e. active). 6. The CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

6.8.2	Architecture Impacts

This solution is presented based on the existing CAPIF architecture in TS 23.222.

6.8.3	Corresponding APIs

This solution is based on service API publish and service API discover, no new APIs are introduced.

6.8.4	Solution evaluation

This solution proposes the API based activation in service API discover procedure. For the AEF instance is already instantiated, the CAPIF core fucntion can request the API management function to influence the required service API to be active when it is inactive before. This solution solves the two open issues in key issue#7.