3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240159

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Huawei, Hisilicon

Title:	Architecture mapping for tethered device scenario

Spec:	3GPP TR 23.700-23 v1.0.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jian Zhang (zhangjian369@huawei.com)

1. Introduction

This paper is proposed to add the architecture mapping for tethered device sceanrio to adpat into the SEALDD architecture.

2. Reason for Change

Regarding of the tethered AR glasses with 5G relay scenario in TR 26.806, as following:

To adpat into SEALDD architecture to support tethered device scenario, there are two options:

Option#1: The SEALDD client is responsible for the closed loop measurement in tethered link. In this case, the SEALDD client and the VAL client can be located at tethered device, and the SEALDD client in 5G UE/device acts as the peer connection to perform the transmission and measurement in tethered link with tethered device, and to perform perform the transmission and measurement in 5G network with SEALDD server.

Option#2: The SEALDD client and the VAL client participate together to complete the measurement in tethered link. In this case, the VAL client and the SEALDD client are located in tethered device and 5G UE/device, respectively. The transmission and measurement in 5G network is performed between the SEALDD client in 5G UE/device and the SEALDD server.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v1.0.0.

* * * First Change * * * *

6.1.x	SEALDD architecture adaptation to support tethered device scenario

Regarding of the tethered AR glasses with 5G relay scenario as descripted in TR 26.806 [8], to adpat into SEALDD architecture to support tethered device scenario, there are the following two options:

Figure 6.1.x-1: Closed loop measurement in tethered link by SEALDD clients

For the option#1 in Figure 6.1.x-1, the SEALDD client is responsible for the closed loop measurement in tethered link. In this case, the SEALDD client and the VAL client can be located at tethered device, and the SEALDD client in 5G UE/device acts as the peer connection to perform the transmission and measurement in tethered link with tethered device, and to perform perform the transmission and measurement in 5G network with SEALDD server.

Figure 6.1.x-2: Collaborative measurement in tethered link by SEALDD client and VAL client

For the option#2 in Figure 6.1.x-2, the SEALDD client and the VAL client participate together to complete the measurement in tethered link. In this case, the VAL client and the SEALDD client are located in tethered device and 5G UE/device, respectively. The transmission and measurement in 5G network is performed between the SEALDD client in 5G UE/device and the SEALDD server.