3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240160

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Huawei, Hisilicon

Title:	update solution#4 to clarify the transmission measurement for tethered device

Spec:	3GPP TR 23.700-23 v1.0.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jian Zhang (zhangjian369@huawei.com)

1. Introduction

This paper is proposed to clarify the transmission measurement procedure for tethered device scenario.

2. Reason for Change

As discussed in S6a240159, regarding of the tethered AR glasses with 5G relay scenario in TR 26.806, there are two SEALDD architecture adaptation options to support tethered device scenarios. Based on these two architecture options, the transmission measurement procedure for tethered link should be revised to be more clearer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v1.0.0.

* * * First Change * * * *

7.4.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.7.2.1 Data transmission quality measurement Figure 9.7.2-1 illustrate the procedure for SEALDD enabled data transmission quality measurement. The SEALDD client and SEALDD server is enhanced to carry out the data transmission quality measurement. Pre-conditions: 1. The SEALDD server and SEALDD client are synchronized to the time source provided by 5GS as specified in 3GPP TS 23.501 [5]. 2. The VAL server discovers and selects the SEALDD server by CAPIF functions. Figure 9.7.2.1-1: SEALDD enabled data transmission quality measurement procedure 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate, packet loss rate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), flow(s)traffic descriptor(s), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE: The spatial and/or temporal condition can be used by SEALDD server to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time. 4. If the transmission quality measurement requirement list provided by VAL server in step 2, indicates that the latency is needed to be measured, the SEALDD server initiates the DL packet delay measurement. The SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy DL SEALDD packet generated for data transmission quality monitoring) with local time T1 when the SEALDD server sends out the DL monitoring packets. The SEALDD server considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD server stops/suspends the transmission quality measurement. NOTE: For other metrics in transmission quality measurement requirement list (e.g. bitrate), the transmission quality result can be obtained by performance detection on the SEALDD server within a period of time. 5. The SEALDD client receives the DL monitoring packet, and records the local time T2. If the enpoint for the SEALDD traffic is located at the tethered device, the SEALDD client in 5G UE measures the QoS between the SEALDD client in 5G UE and the SEALDD client (i.e. for architecture option#1) or the VAL client (i.e. for architecture option#2) in tethered device, e.g., the SEALDD client may measure the tethered link delay using the ICMP ping protocol as defined in IETF RFC792 [RFC792]). The measured delay in the tethered link shall be added to the T2 timestamp. NOTE 2: Only RTT can be measured for the link between SEALDD client and the tethered device. Editor’s Note: How the SEALDD client at the tethering UE knows the tethered device isFFS. 6. Similarly, the SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring) with local time T2 recorded in step 5 and local time T3 when the SEALDD client sends out the UL monitoring packet. 7. The SEALDD server records the local time T4 when the SEALDD server receives the UL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD server can also calculate the bitrate, jitter and packet loss rate over a certain period over a specific SEALDD connection by recording the status of the SEALDD packets carrying VAL traffic or dummy SEALDD packets generated for transmission quality measurement reports. The SEAL DD server also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. 8. The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 7 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) or flow(s)traffic descriptor(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server calculates the needed report for the VAL server. When the VAL server decides to update or unsubscribe transmission quality measurement subscription after performing step 2 and step 3, the VAL server can send data transmission quality measurement subscription update request and data transmission quality measurement unsubscribe request to SEALDD server, as specified in Table 9.7.3.9-1 and Table 9.7.3.11-1, respectively.