3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240169

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Convida Wireless LLC

Title:	Solution 9 update

Spec:	3GPP TR 23.700-21 V 1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	mladin.catalina@convidawireless.com, ly.quang@convidawireless.com

1. Introduction

This contribution proposes an update to solution #9.

2. Reason for Change

The contribution provides updates to architecture impacts, API, and evaluation for solution #9.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21.

* * * First Change * * * *

## 7.9	Solution #9: Avatar discovery procedure

### 7.9.1	Solution description

This solution maps to KI#3 and KI#5. The solution provides a mechanism for discovering avatars.

Pre-conditions:

1)	The VAL client has associated user or avatar profile information for an avatar which is authorized to discover.

Figure 7.9.1-1: Avatar discovery procedure

1.	A VAL client triggers MMEC to search for a digital avatar based on available user and/or avatar profile information.

2.	The MMEC sends an avatar discovery request to a MMES and includes the discovery criteria, e.g. avatar-associated user and/or profile parameters. For example, the request includes (corresponding to Table 7.6.3-1): "Asset type" = avatar, "Metaverse service user information" = information for the user associated with the avatar. Alternatively, the request includes (corresponding to Table 7.6.3-1): "Asset type" = avatar, "Asset type-specific profile" = criteria corresponding to Table 7.5.3.1-1.

The MMEC also includes in the request VAL user and/or MMEC identifiers.

3.	The MMES checks that avatar discovery is authorized, based on VAL user and/or MMEC identifiers. If authorized, the MMES searches an internal repository for an MDRB/avatar profile that is associated with the discovery criteria. If found, the MMES uses VAL user and/or MMEC identifiers to check that the requester is found in the "allowed user list" of the discovered avatar. The MMES can also check the current status and location of the avatar.

4.	For an authorized discovery, the MMES send a response with a status of the request, the digital avatar identifier, the current status and location of the digital avatar, and the metaverse service provider identifier.

5.	The MMEC provides the information returned in step 4 to the VAL client.

### 7.9.2	Architecture Impacts

The architectural impact for this solution is described in 6.1, Option #1. The solution may be adapted to using the A-DACM function as described in 6.2 Option #2 if the architecture is updated to support VAL clients to make requests to the A-DACM function.

### 7.9.3	Corresponding APIs

Table 7.9.3-1 shows the avatar discovery request from the MMEC to the MMES.

Table 7.9.3-1: Avatar discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Requesting avatar | M | The identifier of the avatar of the requestor. |
| Discovery criteria | M | Discovery criteria for avatar that is being discovered. |
| >Avatar identifier | O (NOTE) | An identifier for avatar that is being discovered. |
| >Avatar name | O (NOTE) | A name for avatar that is being discovered. |
| >Avatar description | O (NOTE) | A description for the avatar that is being discovered. |
| Discovery token | M | A token provided for the discovery request. |
| NOTE: At least one of the information elements must be present. |  |  |

Table 7.9.3-2 shows the avatar discovery response from the MMES to the MMEC.

Table 7.9.3-2: Avatar discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success or fail. |
| Discovery results | M | Discovery results |
| >Avatar identifiers | O (NOTE) | Identifiers of the avatar |
| >>URL | O | A URL of the service provider for where the avatar is present |
| >>Avatar status | M | Status of the avatar |
| >>Avatar descriptions | O | Description for the avatar |
| >Avatar names | O (NOTE) | Name of the avatar |
| >>URL | O | A URL of the service provider for where the avatar is present |
| >>Avatar status | M | Status of the avatar |
| >>Avatar descriptions | O | Description for the avatar |
| NOTE: At least one of the information elements must be present. |  |  |

### 7.9.4	Solution evaluation

This solution addresses Key Issues #3 and #5 by providing a mechanism for a VAL client to discover avatars and their associated status. The solution is based on the mobile metaverse application enablement layer architecture involving an enabler client (MMEC) and an enabler server (MMES) as described in 6.1.

* * * End Changes * * * *