3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240173

10 Jul 2024 - 18 Jul 2024	(revision of S6-xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-82 | CR | 0013 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update AIMLE functional architecture and solution #1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIMLAPP |  |  |  |  |  | Date: |  |  | 2024-7-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The architecture diagram does not represent the interface between AIMLEs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update AIMLE functional architecture and solution #1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIMLE architecture diagram and interface definition are not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1, 8.1.2.3, 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Change 1 * * * *

7	Application architecture for enabling AI/ML services

7.1	General

The functional architecture enhancements for the AIML enablement service are based on the generic functional model specified in clause 6.2 of 3GPP TS 23.434 [7]. The architecture enhancements are organized into functional entities to describe a functional architecture enhancement which addresses the support for AIML enablement aspects for vertical applications, as well as the representation for supporting ADAE analytics.

7.2	Application enablement architecture

7.2.1	On-Network AIML Enablement (AIMLE) Functional Architecture

Figure 7.2.1-1: On-network AIMLE functional model

Figure 7.2.1-1 illustrates the on-network functional model of AIML enablement. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The AIML enablement functional entities on the UE and the server are grouped into AIMLE client(s) and AIML enablement server(s) respectively.

The AIMLE includes of a common set of services for comprehensive enablement of AIML functionality, including federated and distributed learning (e.g., FL client registration management, FL client discovery and selection), and reference points. The AIMLE services are offered to the vertical application layer (VAL).

NOTE: The set of common services will be updated based on the TR conclusions.

The AIML enablement client communicates with the AIMLE server(s) over the AIML-UU reference points. The AIMLE client provides functionality to the VAL client(s) over AIML-C reference point. The VAL server(s) communicate with the AIMLE server(s) over AIML-S reference points. The AIMLE servers communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system. AIML-E reference point enables interactions between two AIMLE servers (e.g. central and edge AIMLE servers).

The AIMLE server interacts with the ML repository which serves as repository for ML model and ML participants over AIML-R.

* * * Change 2 * * * *

8.1.2.3	On-Network AIML Enablement Layer Functional Architecture

Figure 8.1.2.3-1: On-network AIML enablement layer functional model

Figure 8.1.2.3-1 illustrates the on-network functional model of AIML enablement layer. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The AIML enablement functional layer entities on the UE and the server are grouped into AIML enablement client(s) and AIML enablement server(s) respectively.

The AIML enablement layer includes of a common set of services for comprehensive enablement of AIML functionality, including federated and distributed learning (e.g., FL client registration management, FL client discovery and selection), and reference points. The AIML enablement services are offered to the vertical application layer (VAL).

The AIML enablement client communicates with the AIML enablement server(s) over the AIML-UU reference points. The AIML enablement client provides functionality to the VAL client(s) over AIML-C reference point. The VAL server(s) communicate with the AIML enablement server(s) over AIML-S reference points. The AIML enablement servers communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system. AIML-E reference point enables interactions between two AIMLE servers (e.g. central and edge AIMLE servers).

* * * Change 3 * * * *

8.1.3	Functional Entities Description

8.1.3.1	General

The ADAE functional entities with ML model training and management enablement function and AIML enablement layer function are described in the following subclauses.

8.1.3.2	ADAE client

The ADAE client functional entity acts as the application client supporting ML model training and may supporting management of ML model training. It interacts with the ADAE server.

8.1.3.3	ADAE server

The ADAE server functional entity provides for management and execution of ML model training supported within the vertical application layer.

8.1.3.4	AIML Enablement client

The AIML enablement client functional entity acts as the application client supporting AIML services and performs client-side operations. It interacts with the AIML enablement server.

8.1.3.5	AIML Enablement server

The AIML enablement server functional entity provides services for comprehensive enablement of AIML functionality (e.g., federated and distributed learning).

8.1.4	Reference Points Description

8.1.4.1	General

The reference points for the functional model for management and execution of ML model training and AIML enablement layer functions are described in the following subclauses.

8.1.4.2	ADAE-UU

The interactions related to ML model training and management functions between the ADAE client and ADAE server are supported by ADAE-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [8] and 3GPP TS 23.501 [5].

ADAE-UU reference point is used for VAL service signalling for VAL service data management of the VAL service.

8.1.4.3	ADAE-S

The interactions related to ML model training and management functions between the VAL server(s) and the ADAE server are supported by ADAE-S reference point.

8.1.4.4	ADAE-C

The interactions related to ML model training and management functions between the VAL client(s) and the ADAE client within a VAL UE are supported by ADAE-C reference point.

8.1.4.5	AIML-UU

The interactions related to AIML enablement layer functions between the AIML enablement client and AIML enablement server are supported by AIML-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [8] and 3GPP TS 23.501 [5].

8.1.4.6	AIML-S

The interactions related to AIML enablement layer functions between the VAL server(s) and the AIML enablement server are supported by AIML-S reference point.

8.1.4.7	AIML-C

The interactions related to AIML enablement layer functions between the VAL client(s) and the AIML enablement client within a VAL UE are supported by AIML-C reference point.

8.1.4.8	AIML-E

The interactions related to AIML enablement layer functions between different AIML enablement servers are supported by AIML-E reference point (e.g. central and edge AIMLE servers).

* * * End of changes * * * *