3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240176

10th – 18th July 2024	(revision of S6a240xxx)

Source:	Convida Wireless LLC

Title:	Solution 10 update

Spec:	3GPP TR 23.700-21 V 1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	mladin.catalina@convidawireless.com, ly.quang@convidawireless.com

1. Introduction

There are several  Editor’s Notes in clause 7.10 which need to be addressed, as well as some editorial issues to be addressed.

2. Reason for Change

The solution for metaverse digital asset synchronization with QoS control needs text in the architecture impacts and solution evaluation clauses. Another Editor’s Note is transformed into a NOTE, and editorials are provided for cleanup.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21.

* * * First Change * * * *

## 7.10	Solution #10: Metaverse digital asset synchronization with QoS control

### 7.10.1	Solution description

The following clauses specify procedures and information flows for Key Issue #3 and Key Issue #5 for QoS control for avatars as digital assets.

Preconditions:

1.	A digital asset (e.g. avatar representation) has been created (e.g. as element in MDRB) and MMEL policies have been associated.

NOTE 1:	This solution is described using terminology from clause 6.1, as well as examples or alternatives related to other solutions.

Figure 7.10.1-1: Overall Procedure

1.	A VAL server  and/or VAL client sends a digital asset synchronization session creation request to a MMES or MMEC to establish end-to-end connections for the coordinated exchange of digital asset information with required session information (e.g. QoS) between VAL client(s) and VAL server(s) for the purpose of synchronization. The request contains information pertaining to the digital asset synchronization session requirements, e.g. VAL client and VAL server endpoint information and required QoS for each end-to-end connection of the digital asset synchronization session.

NOTE 2: Alternatively, the same VAL layer request targets the SEALDD layer, and the SEALDD layer resolves the digital asset synchronization session parameters by querying MMEL. In this alternative, digital asset synchronization session policies and information are assumed to be available to both SEALDD layer and MMEL and the flow continues with step 4.

2.	The MMEC and/or MMES determine applicable digital asset synchronization session policies. This is done, in addition to the information already available in the policies established at precondition, based on the session requirements provided in step 1.

3.	The MMES interacts with the SEALDD server to determine and configure additional communication requirements, e.g.  MMES sends SEALDD connection establishment requests to SEALDD server.

4.	Based on interactions from the previous steps, the SEALDD layer creates or updates SEALDD flows to support the digital asset synchronization session based on the requirements provided.

5.	Once SEALDD flows have been established for the digital asset synchronization, MMECs and MMES begin exchanging application traffic (via the SEALDD flows) as required by the application for the purpose of digital asset synchronization.

6.	For each SEALDD flow configured with MMEL support, state information may be maintained by MMES and/or MMEC. The collected state and corresponding measurements performed are based on the configured policies or management requests.

7.	MMEL state and/or monitoring information may be used to enable notifications to be sent to the VAL layer and enable adaptive functionality in the VAL layer.

8.	When the digital asset synchronization session is no longer needed, the VAL client or server sends a request to MMEC or MMES to tear down the digital asset synchronization session and associated SEALDD flow.

### 7.10.2	Architecture Impacts

This solution is described using a new Metaverse Enabler layer (MMES/MMEC entities as described in clause 6.1 and leveraging SEALDD functionality in the SEAL layer) for support of metaverse services.

### 7.10.3	Corresponding APIs

Table 7.10.3-1 details digital asset synchronization session communication parameters received in steps 1- 2 at the SEALDD layer from the VAL Server (directly or via MMES).

Table 7.10.3-1: Digital asset synchronization session communication parameters

| Information element | Status | Description |
| --- | --- | --- |
| List of digital assets | M | A list of individual digital assets associated with this synchronization session. Each digital asset requiring a synchronization session has client-side and server-side representations. |
| > Server-side endpoint ID | M | The unique identifier (e.g., URI) of the VAL server-side endpoint of the synchronization session for this digital asset. |
| > Client-side endpoint ID | M | The unique identifier (e.g., URI) of the VAL client-side endpoint of the synchronization session for this digital asset. |
| > Hosting UE ID | M | Identifier of the UE hosting the VAL client-side of this digital asset |
| > QoS info | M | QoS requirements for the digital asset synchronization session supporting this digital asset, e.g. required bandwidth, latency, jitter, reliability, schedule. |

NOTE 1: 	Table 7.10.3-1 provides digital asset synchronization session communication parameters only. In the normative phase, additional parameters are necessary to describe the one or more messages to be detailed for steps 1-2.

### 7.10.4	Solution evaluation

This solution enables MMEL to provide digital asset synchronization services to mobile metaverse VAL applications. The procedure allows for QoS coordination and is relevant to metaverse applications requiring synchronization of digital asset information.

The solution complements solution #11 by describing how SEALDD can be leveraged. for establishment of session #1 and session #2, when they are established for synchronization purposes. It is a feasible solution which leverages existing 5GC services for QoS monitoring and QoS influence.

* * * End Changes * * * *