3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240181

10th – 18th July 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-82 | CR | 0014 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove EN in Solution#12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIMLAPP |  |  |  |  |  | Date: |  |  | 2024-07-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following EN is left in Solution#12 : Editor’s Note : Whether and how ADAE is a functional entity capable of detecting model performance degradation is FFS. An ADAE Server, as ML model consumer, uses the ML model generate analytics and collects feedback information from the analyitcs consumer (the analytics consumer has taken action(s) influenced by the provided analytics). The ADAE Server may check the performance of the ML model according to the feedback informaiton, e.g., ML model performance degradation according to the feedback information from the analytics consumer. This pCR proposes to remove the EN and discuss details on ADAES as ML model consumer for detecting model performance degradation during the normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Removed the EN left in Soltuion#12. - Added NOTE about discuss details on ADAES as ML model consumer for detecting model performance degradation during the normative phase. - Updated step 5 in clause 8.12.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is EN left in Solution#12, incomplete description of step 5 in the consumer-based ML Model performance degradation detection procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 8.12.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.12.2	Consumer-based ML Model Performance Degradation Detection

Figure 8.12.2-1: Consumer-based ML Model Performance Degradation Detection

This procedure corresponds to the step 0b in clause 8.12.1.

0.	An ADAE Server, as an AI/ML Enablement Consumer, receives trained ML model (or ML model information) from the AI/ML Enablement Server.

1.	The ADAE Server receives request from consumer for analytics, generates analytics by using the ML model provided by AI/ML Enablement Server, and responses/notifies to the consumer with the required analytics.

2.	The ADAE Server requests the consumer to feedback usage result of the analytics.

3.	The consumer uses the analytics for its operations and collects operation results. Performance degradation may be found from the operation results. The performance degradation may cause by e.g., insufficient analytics accuracy, or the current analytics cannot fulfill the changed conditions at the consumer.

4.	The consumer feedbacks the usage result of the analytics to the ADAE Server.

5.	The ADAE Server updates the analytics accuracy based on feedback information from the consumer, and checks the performance of the ML model, which is used to generate the analytics, e.g., performance degradation of the ML model according to the feedback information from the analytics consumer.

NOTE:	Details on ADAES as ML model consumer for detecting ML model performance degradation will be determined during the normative phase.

If the AI/ML Enablement Consumer is e.g. VAL Server, similar procedure as shown in Figure 8.12.2-1 be performed for detecting of ML Model performance, by replacing "analytics" with e.g., "operation instructions".

# *** End of Changes ***