3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240182

10th – 18th July 2024

Source:	Ericsson

Title:	Application Enablement Layer AIML Capabilities for Supporting XR Services

Spec:	3GPP TS 23.700-23 V1.0.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

As described in Key Issue #4, the study aspects on application enablement layer capabilities usage to supporting the XR services include:

- Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

- Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

The application enablement layer capabilities of the AIML Enablement server (studied in TR 23.700-82) can be considered to use for supporting XR services. For example, AI/ML may be required for image/object recognition (as listed in the description part of KI#4), image/video processing, etc. Use cases can be found in TR 23.874 and TR 23.876 about AI/ML support XR relevant services, e.g., split AI/ML image/object recognition, split image/video processing.

This paper proposes to use the application enablement layer capabilities provided by the AIML Enablement server to support XR services for AI/ML based XR data delivery/distribution.

2. Reason for Change

Application enablement layer capabilities provided by the AIML Enablement server (studied in TR 23.700-82) can be used to support XR services for AI/ML based XR data delivery/distribution, how the application enablement AIML capability could be used for supporting the AI/ML based XR data delivery/distribution need be studied.

3. Conclusions

This pCR proposes to use the application enablement layer capabilities provided by the AIML Enablement server to support XR services for AI/ML based XR data delivery/distribution.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 V1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS23.433: "Service Enabler Architecture Layer for Verticals (SEAL);Data Delivery enabler for vertical applications;  Stage 2".

[7]	3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[8]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

[9]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[10]	3GPP TR 22.847: "Study on supporting tactile and multi-modality".

[11]	3GPP TS 23.558:"Architecture for enabling Edge Applications".

[12]	3GPP TS 23.434:"Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[13]	3GPP TR 22.856:"Feasibility Study on Localized Mobile Metaverse Services".

[14]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[15]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[16]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[17]	3GPP TS28.538: "Management and orchestration; Edge Computing Management (ECM)".

[Y1]	3GPP TR 23.700-82: "Study on application layer support for AI/ML services".

[Y2]	3GPP TR 23.874: "Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS".

[Y3]	3GPP TR 23.876: "Study on AI/ML Model Transfer Phase 2".

[Y4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 | X |  |  |  | X |  |  | X |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 | X |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |  |
| Sol #9 |  |  |  |  | X |  |  |  |
| Sol #10 |  |  |  |  |  | X |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |
| Sol #X |  |  |  | X |  |  |  |  |

* * * Next Change * * * *

## 7.X	Solution #X: Application Enablement Layer AIML Capabilities for Supporting XR Services

### 7.X.1	General

The application enablement layer capabilities of the AIML Enablement server (studied in 3GPP TR 23.700-82 [Y1]) can be considered to use for supporting XR services. For example, AI/ML may be required for image/object recognition (as listed in the description part of KI#4), image/video processing, etc. Use cases can be found in 3GPP TR 23.874 [Y2] and 3GPP TR 23.876 [Y3] about AI/ML support XR relevant services, e.g., split AI/ML image/object recognition, split image/video processing.

The following clauses specify procedures, information flows and APIs for usage of application enablement layer AIML capabilities to support XR Services.

### 7.X.2	Procedure

Pre-conditions:

-	The AIMLE Server is known by the consumer and the SEALDD/XRApp Server.

-	The AI/ML services provided by the AIMLE Server is allowed to exposure to the consumer and the SEALDD/XRApp Server.

-	The consumer decides AI/ML based XR service, e.g. split image/video processing (distribute image/video processing tasks to multiple split endpoints for processing).

Figure 7.X.2-1: Application enablement layer AIML capabilities to support XR Services

- The consumer (e.g. VAL server, VAL client via SEALDD/XRApp client) sends request to AIMLE server for assistance information on AI/ML operations for XR service, e.g. split endpoints for split AI/ML based image/video processing. The consumer gets assistance information, e.g. split AI/ML endpoints.

- The consumer sends request to the SEALDD/XRApp server for supporting AI/ML based XR service (e.g. deliver/distribute XR data to the split AI/ML endpoints). The request message contains message as defined in table 7.X.3.2-1.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server responses to the VAL server. The SEALDD/XRApp server derives information for the request, for example, based on the message in the request to decide whether assistance from 5GC and/or AIMLE server is needed or not, and determine downstream entities and services needed based on the information derived. For example, assistance informaiton from AIMLE server is needed on:

-	Time point/time window for image/video/multi-modal data delivery/distribution with QoS requirement.

-	Achievable QoS with current configuration of session between UE and AF (e.g. the consumer).

-	Suggestion of QoS values for each flow of the session in order to satisfy application requirement.

The assistance information from 5GC may also needed on anlayitcs of data delivery/distribution time (e.g. analytics from NWDAF on E2E data volume transfer time).

- The SEALDD/XRApp server sends request to the AIMLE server for assistance information on AI/ML operations for the XR data delivery/distribution. For example, for split AI/ML image/video processing, the request contains the information of split AI/ML endpoints (from the request in step 1), data information (e.g. data type, data size) to be delivered/distributed to the split endpoints, may contain QoS requirements on the data delivery/distribution, e.g. desired data delivery/distribution time. The AIMLE server generates assistance information and responses to the SEALDD/XRApp server with the required assistance information, e.g.

-	Time point/time window for the data delivery/distribution to the split AI/ML endpoints and satisfy the required QoS. The solutions introduced in 3GPP TR 23.700-82 [Y1] on support AI/ML splitting operations in enablement layer can be reused.

-	The achievable QoS with current configuration of session between UE and AF (e.g. the consumer). The solutions introduced in 3GPP TR 23.700-82 [Y1] on inference (by using/training and then using ML model) can be reused to generate the assistance information.

-	Suggestion of QoS values for each flow of the session in order to satisfy application requirement. The solutions introduced in 3GPP TR 23.700-82 [Y1] on inference (by using/training and then using ML model) can be reused to generate the assistance information.

NOTE 1:	For generating the assistance information, the AIMLE server may collect information of the split AI/ML endpoints (i.e. VAL clients) or interact with 5GC NFs or other network entities. The procedures for data collection from UE need to take user consent into account.

NOTE 2:	The AIMLE server may interact with ADAES on analytics about image/video/multi-modal data delivery/distribution among VAL server, SEALDD/XRApp server, the SEALDD/XRApp clients (which are deployed on the VAL clients (i.e. the potential split AI/ML endpoints for the XR service request in step 1)), and VAL clients. The existing ADAE analytics introduced in 3GPP TS 23.436 [Y4] can be resued with enhancement or new ADAE analytics may need.

NOTE 3:	The workflow relevant to AIMLE server will be discussed in the normative phase.

- The SEALDD/XRApp server may request and receive assistance from 5GC, as described in step 3, e.g. assistance information from NWDAF (or via NEF) about analytics on E2E data volume transfer time.

- The SEALDD/XRApp server triggers to establish connection among the consumer, SEALDD/XRApp server, SEALDD/XRApp client(s), and VAL client(s) for the data delivery/distribution of the XR service.

NOTE 4:	The connection establishment and data delivery/distribution procedures could use the existing solutions.

- Operations are performed at the consumer, SEALDD/XRApp server, SEALDD/XRApp client(s), and VAL client(s) for the AI/ML based XR data delivery/distribution, e.g. image/video/multi-modal data delivery/distribution from the consumer to the split AI/ML endpoints.

### 7.X.3	Information flows

#### 7.X.3.1	General

The following information flows are specified for supporting AI/ML based XR service request based on clause 7.X.2.

#### 7.X.3.2	Supporting AI/ML based XR service request

Table 7.X.3.2-1 describes the information flow from the consumer (e.g. VAL Server) as a request or update request for the XR service.

Table 7.X.3.2-1: Supporting AI/ML based XR service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M (NOTE) | The identifier of the consumer (e.g. VAL server ID, SEALDD/XRApp client ID, VAL client ID). |
| VAL service ID | O | Identity of the VAL service. |
| VAL UE ID(s) | O | Identifier(s) of the VAL UE(s) for the XR service. |
| Data information | M | The information of the data needs to be delivered or distributed for the XR service. |
| >Data type | M | The type of data, could be e.g. image, video, multi-model data (e.g. audio, video, positioning, haptic data). |
| >Data size | O | The maximum/minimum/average size of the data to be deliveried/distributed. |
| QoS requirement | O | The QoS requirement, e.g. latency for data delivery/distribution. |
| NOTE: This information element shall not be updated. |  |  |

NOTE 3:	The XR service information in the request is used by the SEALDD/XRApp server e.g. derive information in step 3, request requirements on split AI/ML endpoints in step 5, check user consent for data collection.

#### 7.X.3.3	Supporting AI/ML based XR service response

Table 7.X.3.3-1 describes the information elements for the supporting AI/ML based XR service response from the SEALDD/XRApp server to the consumer.

Table 7.X.3.3-1: Supporting AI/ML based XR service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the XR service request (positive or negative acknowledgement). |

### 7.X.4	Solution evaluation

This solution addresses KI#4 and introcudes procedure on utilize of the application enablement layer AIML capabilities provided to support XR services, for the following aspects:

- Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

- Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

NOTE:	The dependency with AIMLE service will be discussed during normative phase.

# *** End of Changes ***