3GPP TSG-SA WG6 Meeting #62-ad hoc-e	S6a240196

e-meeting, 10th – 18th July 2024	(revision of S6-242xxx)

Source:	Nokia

Title:	A-DACM architecture update

Spec:	3GPP TR 23.700-21 v1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for A-DACM architecture update.

2. Reason for Change

In TR 23.700-21 v1.0.0, clause 6.2.1, the following EN is to be resolved:

Editor's Note: How VAL client(s) access the services of A-DACM function is FFS.

The following options of access are possible:

- The VAL client can use a corresponding SEAL client to access the A-DACM service which is compliant with the architecture in TS 23.434.

- The VAL client (acting as API invoker) can directly invoke the service APIs of A-DACM (acting as AEF) which is compliant with CAPIF as specified in TS 23.222.

Both approaches are to captured in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.0.0.

* * * First Change * * * *

## 6.2	Option #2: A-DACM architecture to support Metaverse services

### 6.2.1	Application enablement architecture

Figure 6.2.1-1 illustrates the A-DACM architecture to support Metaverse services using SEAL architecture specified in 3GPP TS 23.434 [8].

Figure 6.2.1-1: A-DACM architecture to support Metaverse services using SEAL architecture

The SEAL architecture includes A-DACM client and server functions which supports the VAL layer. The A-DACM server interacts with the VAL server(s) over the SEAL-S reference point. The A-DACM client interacts with the VAL client(s) over the SEAL-C reference point. The interactions between A-DACM client and A-DACM server is ove the SEAL-UU reference point. The A-DACM server may utilize the 5GC services over N33 reference point.

Figure 6.2.1-2 illustrates the A-DACM architecture to support Metaverse services using CAPIF architecture specified in 3GPP TS 23.222 [14].

Figure 6.2.1-2: A-DACM architecture to support Metaverse services using CAPIF architecture

The SEAL architecture includes A-DACM function which supports the VAL layer. The A-DACM function interacts with the VAL server(s) over the SEAL-S reference point. The A-DACM function interacts with the VAL server(s) over the SEAL-VC reference point. The service APIs offered by A-DACM function on SEAL-AC and SEAL-S to the VAL layer functions follow the service-based architecture as shown in the figure 6.2.1-3. The A-DACM function may utilize the 5GC services over N33 reference point.

Figure 6.2.1-3 illustrates the service-based SEAL architecture to support Metaverse services.

Figure 6.2.1-3: Service-based SEAL architecture to support Metaverse services

The A-DACM function exposes the APIs over the Sdacm interface to the VAL layer. The VAL functions (VAL servers) consume the services by interacting on the Sdacm interface of the A-DACM function. The CAPIF can be used by VAL functions to discover the services of the A-DACM function.

NOTE:	The A-DACM function can be realized as a new SEAL service or part of existing SEAL service.