3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240214

10th – 18th July 2024	(revision of S6a240138)

Source:	Samsung

Title:	Pseudo-CR on specification cleanup

Spec:	3GPP TR 23.700-21 v1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR clears the specification by removed template clauses, adjusting bullet numbers as per drafting rules and removing EN related to deployment models.

2. Reason for Change

It is required to remove template clauses which were added as guidance to create KI or solution. Also, some of the clauses starts with bullet 0, it is required to start with bullet number 1 – as per the drafting rules. EN related to deployment model is also removed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 (v1.0.0).

* * * First Change * * * *

* * * Next Change * * * *

* * * Next Change * * * *

#### 7.1.1.2	Procedure

Pre-conditions:

1)	Spatial anchors have been provisioned in the mobile metaverse enabler server

2)	A VAL client (e.g., an AR application) needs to display enriched localized content to a user

Figure 7.1.1.2-1: Spatial anchor discovery procedure

1.	A metaverse VAL client discover s spatial anchors. The VAL client performs spatial anchor discovery using the mobile metaverse enabler client (MMEC) of the UE. The VAL client provides spatial anchor discovery requirements to the MMEC to discover spatial anchors of interest. The discovery requirements include a VAL client identifier, a VAL client type, information on location(s) of interest, and information on service(s) of interest (e.g., service type, service provider identifier, time of day when service is requested, etc.) for the service associated with spatial anchors, and indication to enable persistent search.

NOTE 1: The requirements for spatial anchor discovery may be further defined in the normative phase.

2.	The MMEC sends a spatial anchor discovery request to the mobile metaverse enabler server (MMES). The request includes information elements defined in Table 7.1.3-1.

3.	Upon receiving the request, the MMES validates if the requestor is authorized to discover spatial anchors. If the requestor is authorized, the MMES uses the information included the spatial anchor discovery request to determine spatial anchors to include in the discovery response.

If spatial anchor discovery filters are provided in the request, the MMES determines any spatial anchors matching the discovery filters. For each determined spatial anchor, the MMES checks whether the requestor is authorized to discover the spatial anchor based on the identity and location of the requestor and the access controls and service area characteristics of the spatial anchor. To determine a location of interest to the requestor, the MMES may use location information in the request, if available, or obtain location information by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [7] and 3GPP TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [8]. Additionally, the MMES may obtain and consider data analytics, such as UE mobility prediction, during spatial anchor determination. The MMES may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [10] and 3GPP TS 23.502 [9].

4.	The MMES sends a spatial anchor discovery response to the MMEC. If the MMES has successfully determined spatial anchors, the response may include a success indication, a list of the determined spatial anchors as defined in Table 7.1.3-2. If the MMES has not determined any spatial anchor, the response may include a failure indication and a cause of failure. The response also includes the recommended list of spatial anchors in the discovery response based on the discovery filter information and stored spatial anchor information.

NOTE 2:	The logic to generate the recommended list of spatial anchor is out of scope and implementation-specific.

5.	Upon receiving the discovery response, if the response indicates success, the MMEC may store the received spatial anchor information in a spatial anchor cache. The MMEC evaluates the validity conditions of discovered spatial anchors.

6.	The MMEC provides the valid spatial anchor information to the VAL client(s) based on VAL client requirement(s). Upon receiving valid spatial anchor information, the VAL client can access the service(s) associated with each spatial anchor.

NOTE 3:	This procedure may be adapted to allow a VAL server to discover spatial anchor(s).

* * * Next Change * * * *

### 7.6.1	Solution description

The following clauses specify procedures and information flows for Key Issue #3 for support of avatars, as a type of digital asset managed using Metaverse Digital Representation Block (MDRB).

MDRB is an information block containing one or more related digital assets, information about the associated users and about the associated metaverse services. MDRB also includes MMEL policies and parameters for the handling of the set of digital representations stored and managed together as part of MDRB.

NOTE 1:	How to choose or assign the digital assets to be managed together as part of an MDRB is not in scope of this solution.

NOTE 2:	In the normative phase, MDRB may be implemented as the information block used by A-DACM (solution #7) for management of digital assets, e.g. MDRB can be renamed as DAC, and MMES can implement A-DACM or have access to an external A-DACM.

Preconditions:

1.	Digital representations are available at the VAL application layer.

Figure 7.6.1.1: MDRB support request procedure

Procedural steps:

1.	The VAL server sends a request to the corresponding MMES to enable MDRB support for the metaverse application functionality related to the digital representation created. The message(s) include information about the digital representation(s) to be supported, e.g. avatar.

NOTE 1:	In the normative phase, the step 1 request can map to one or more requests, e.g. create /update /delete.

2.	The MMES determines applicable MMEL parameters for the handling of the digital representation. The MMEL support information and parameters are associated with MDRB.

NOTE 2:	The MDRB stores information necessary for accessing and management of the associated digital assets and their exposure to/from the VAL application layer. This information is provided by the VAL layer or is derived based on the VAL layer request and policies.

Based on the information received from the VAL layer and on local policies, the MMES optionally determines to send one or more requests to the underlying 3GPP Network to determine information available in the 3GPP network for the MDRB.

NOTE 3:	The information available in the 3GPP network to be included as part of MDRB is to be determined in the normative phase and aligned with specifications from SA2, SA3 and SA4.

NOTE 4:	Depending on the architectural option chosen, this step relies on one of the following options: MMES can implement A-DACM or access an external A-DACM (e.g. as new or existing SEAL entity)..

3.	The MMES generates response and sends the message to the VAL Server.

* * * Next Change * * * *

* * * Next Change * * * *

## 8.1	General

This clause describes different deployment models to provide metaverse services to the end user.

## 8.2	Deployment model #1: Deployment of enabler for Metaverse Services as SEAL server(s)

Figure 8.2-1 illustrates deployment of the metaverse services as SEAL server(s) in a single PLMN operator domain and the VAL server(s) in the VAL service provider domain.

Figure 8.2.1-1: metaverse services as SEAL server(s)

The SEAL server(s) can be one or more SEAL services, providing enablement services to the metaverse application servers. The SEAL-S interface provides multiple service APIs towards Metaverse application server.

* * * End of Change * * * *