3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240217

10th – 18th July 2024	(revision of S6a240144)

Source:	Samsung

Title:	Pseudo-CR on update to solution #8

Spec:	3GPP TR 23.700-21 V1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah, sapan.shah@samsung.com

1. Introduction

This pCR proposes to provide different services on top of spatial map.

2. Reason for Change

Once spatial map is produced, the VAL server may required different infomration from the spatial map. Enabler service can provide value added services on top of spatial map.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-72 V1.0.0.

* * * First Change * * * *

#### 7.8.3.x	Spatial localization services

Figure 7.8.3.x-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to invoke spatial localization services. The service is provided by SEAL LM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 7.8.3.x-1: Spatial localization services

1)	The VAL server (or SEAL LMC) sends request to perform localization service in the spatial map to the SEAL LMS. The request includes identity of the map on which the action is to be performed.

Table 1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target user identity | O | Identity of the user to localize |

2)	The SEAL LMS authorizes the requestor (e.g. VAL server or SEAL LMC). If the requestor is not authorized then the SEAL LMS sends failure response. If the request is authorized:  the SEAL LMS identifies the target users along with its three dimensional localization in the spatial map. The SEAL LMS identifies the target user and also assigns unique identity within maps. If area of interest IE is present then the SEAL LMS identifies the target users within the area of interest only, otherwise, the SEAL LMS identifies the target users for whole map (according to the area associated with spatial map identifier).

3)	The SEAL LMS sends response to the VAL server (or SEAL LMC). The response includes (as shown in Table-2) the target user localization and identity.

Table 2: Spatial localization service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the action |
| User details | O | A list of users |
| > User id | O | Identity of the User |
| > User localization | O | Current localization information of the user in the spatial map |

NOTE:	Whether and how localization of objects can be realized needs further discussions.

* * * End of Change * * * *