3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240242

10th – 18th July 2024	(revision of S6a240026)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 056 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Flight route selection support API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom AG |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A procedure for flight route support (S6-241440) has been agreed in 3GPP SA6#60 without corresponding API. This CR proposes to agree API operations and description corresponding to the existing and already introduced procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to define APIs corresponding to existing procedure on flight route support. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing procedure for flight route support would not be available for API usage. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2.1, 8.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S6a240026 |  |  |  |  |  |  |  |  |

* * * First Change * * *

# 8	APIs

## 8.1	General

The following UAE capabilities are offered as APIs:

-	UAE server APIs;

The following SEAL service APIs are specified in 3GPP TS 23.434 [5]:

-	Group management server APIs;

-	Location management server APIs;

-	Configuration management server APIs;

-	Identity management server APIs; and

-	Key management server APIs.

The following SEAL service APIs are specified in 3GPP TS 23.435 [14]:

- Network slice optimization APIs based on VAL server policy,

When UAS application specific server invokes a SEAL server API directly, the UAS application specific server acting as VAL server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response as specified in 3GPP TS 23.434 [5].

Requesting specific QoS per planned flight path/waypoint is available using Policy of needed minimum QoS per UE in the SS_NSCE_PolicyManagement API, see 3GPP TS 23.435 [14] clause 9.5.4.1.

## 8.2	UAE server APIs

### 8.2.1	General

Table 8.2.1-1 illustrates the UAE server APIs.

Table 8.2.1-1: List of UAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| UAE_C2OperationModeManagement API | Manage_C2OperationMode | UAS application specific server | Request/ Response |
|  | Notify_SelectedC2Mode (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2ModeSwitching (NOTE) | UAS application specific server | Subscribe/notify |
|  | Notify_C2OperationModeManagementComplete (NOTE) | UAS application specific server | Subscribe/notify |
| UAE_RealtimeUAVStatus API | Subscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
|  | Notify_RealtimeUAVStatus | UAS application specific server | Subscribe/notify |
| UAE_ChangeUSSManagement API | Manage_USSManagement | UAS application specific server | Request/ Response |
|  | Notify_USSManagementComplete | UAS application specific server | Subscribe/notify |
|  | Manage_USSChange | UAS application specific server | Request/ Response |
|  | Notify_USSChange | UAS application specific server | Subscribe/notify |
|  | Notify_USSChangeTrigger | UAS application specific server | Subscribe/notify |
| UAE_DAASupport API | Manage_DAAManagement | UAS application specific server | Request/ Response |
|  | Notify_DAAManagementComplete | UAS application specific server | Subscribe/notify |
|  | Notify_DAAClientSupportEvent | UAS application specific server | Subscribe/notify |
|  | Manage_DAAServerSupportEvent | UAS application specific server | Request/ Response |
| UAE_UAVDynamicInfo API | Subscribe_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
|  | Unsubscribe_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
|  | Notify_UAVDynamicInfo | UAS application specific server | Subscribe/notify |
| UAE_FlightPathMonitoring API | Manage_FlightPathReportingConfiguration | UAS application specific server | Request/ Response |
|  | Notify_FlightPathReportingConfigurationComplete | UAS application specific server | Subscribe/notify |
|  | Notify_FlightPathEventReport | UAS application specific server | Subscribe/notify |
| UAE_FlightRouteSupport API | Manage_QoSFlightRoutePlan | UAS application specific server, UAE client | Request/ Response |
| NOTE: The subscribe operation for Notify_C2OperationModeManagementComplete, Notify_SelectedC2Mode and Notify_C2ModeSwitching is part of Manage_C2OperationMode |  |  |  |

* * * Second Change * * *

### 8.2.x	UAE_FlightRouteSupport API

#### 8.2.x.1	General

API description: This API enables the UAS application specific server or the UAE client to communicate with the UAE server to send information related to creating flight path for the UAS and receive response from the UAE server containing list of waypoints which support the requested QoS and service availability.

#### 8.2.x.2	Manage_QoSFlightRoutePlan

API operation name: Manage_QoSFlightRoutePlan

Description: Request the flight route for the UAS UEs based on needed QoS and service availability.

Known Consumers: UAS application specific server or UAE client.

Inputs: Refer clause 7.10.3.1.

Outputs: Refer clause 7.10.3.2.

See clause 7.10.2.1 for the details of usage of this API operation.

* * * End of Changes * * *