3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240246

10th – 18th July 2024	(revision of S6a240061)

Source:	Huawei, HiSilicon

Title:	pCR on update solution#3

Spec:	3GPP TR 23.700-92 v1.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes update the elements of DC application profile in solution#3.

2. Reason for Change

The element of Condition was missed and needed to align with the DC application profile configuration procedure in solution#4.

In the email discussion period, Rev1 adds some sub-IE of the condition and modifies the IE of condition in clause 8.4.2 accordingly, no further comments were received

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v1.0.0.

* * * First Change * * * *

### 8.3.2	Procedures

The eMMTel Client in the UE sends an application request messages to eMMTel Enabler Server to request the data channel application profile list is illustrated in Figure 8.3.2.1-2.

NOTE 1:	The eMMTel Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the data channel application profile provided by the eMMTel Enabler Server. The interaction between eMMTel Client and DCMTSI Client is internal interaction in the dialler application on the UE and implementation specific.

NOTE 2:	The DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications. To simply the procedures, the DCMTSI client is not shown in the procedures in this solution.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.2.1-2: eMMTel Client in the UE requests the Data Channel Application profile list

When the eMMTel service provider provides a Root application which specified in GSMA NG.134 [15], containing layout, and/or home page etc. of the Data Channel Application profile list, step 1, step 2 and step 3 apply: Otherwise the step 1, step 2 and step 3 are skipped.

NOTE 1:	The user experience of using the eMMTel service may significantly different in devices provided by different vendors if the layout, and/or home page etc. are all implementation specific. E.g. in some devices, the available Data Channel Applications may be selected from a menu locally implemented in the device, but in some other devices only the first Data Channel Application in the Data Channel Application list can be selected and used automatically by the device. Layout, and/or home page etc. of the Data Channel Application list provided by eMMTel service provider may reduce the difference of user experience.

1.	The eMMTel Client in the UE sends a Get Root application request to eMMTel Enabler Server to get the Root application (e.g. layout, and/or home page etc. of the Data Channel Application profile list). The request message includes information elements as specified in Table 8.3.2.1-1.

Table 8.3.2.1-1: Information elements in Get Root application request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| Root application version | O | The newest version of the Root application stored in the eMMTel Client in the UE. This IE presents if Root application is locally stored in the eMMTel Client. |
| NOTE: The eMMTel service provider can provide the eMMTel Client with different Root application based on this IE. |  |  |

2.	The eMMTel Enabler Server checks whether the Root application version included in the Get Root application request is the newest version of the Root application. If the Root application version IE is not included, the Root application version is not considered as newest.

3.	The eMMTel Enabler Server sends a Get Root application response to the eMMTel Client in the UE. The response message includes information elements as specified in Table 8.3.2.1-2.

Table 8.3.2.1-2: Information elements for Get Root application response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Get Root application request. |
| Update needed | M | Indicates whether the Root application is needed to be updated by the eMMTel Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the eMMTel Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the eMMTel service provider, e.g. layout, and/or home page etc of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the eMMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

4.	The eMMTel Client in the UE send a Get Application Profile List request to eMMTel Enabler Server to get the Data Channel Application profile list available for this user. The request message includes information elements as specified in Table 8.3.2.1-3.

Table 8.3.2.1-3: Information elements in Get Application Profile List request

| Information element | Status | Description |
| --- | --- | --- |
| eMMTel Client version (see NOTE) | M | The version of eMMTel Client |
| Begin index | O | The starting index of the first Data Channel Application required in the Data Channel Application list. This IE presents if the requested Data Channel Application is not the first one in the Data Channel Application list, e.g. when the Data Channel Application list is needed to be shown in multiple pages, this IE is needed to be included when request the pages other than the first page. The default value of this IE is 0 if this IE is absent. |
| Number of application profile required | O | The total number of Data Channel Application profiles required in this request |
| NOTE: The eMMTel service provider can provide the eMMTel Client with different Data Channel Application profile list based on this IE. |  |  |

5.	The eMMTel Enabler Server sends a  Get Application Profile List response to the eMMTel Client in the UE. The response message includes information elements as specified in Table 8.3.2.1-4.

Table 8.3.2.1-4: Information elements in Get Application Profile List response

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel Application profile list | M | A list of Data Channel Application profiles available for this user. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.3.2.1-5. |

Table 8.3.2.1-5: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Application Icon | O | Icon of the Data Channel Application |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: - Precall only:the Data Channel Application is allowed to be used before the eMMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. - Incall:the Data Channel Application is allowed to be used after the eMMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Autoload (see NOTE) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: - Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. - Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. - Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: - Service area: the application is allowed to be used in a certain area. - Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| NOTE: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. |  |  |

* * * Next Change * * * *

### 8.4.2	Procedures

The procedure of Controlling Application Server configuring DC application profile is illustrated in figure 8.4.2-1.

Pre-conditions:

1)	The Controlling Application Server has connected to the serving network successfully.

2).	Both the Controlling Application Serverand eMMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.4.2-1: configuration of DC application profile

1.	The Controlling Application Server sends a DC Application Profile configuration request to the eMMTel Enabler Server. The request message includes information elements as specified in Table 8.4.2-1.

Table 8.4.2-1: Information elements in DC Application Profile configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the eMMTel Enabler Server. |
| Number of DC Application Profiles included | M | Indicates total number of DC Application Profiles included in this request |
| Data Channel Application list | M | A list of DC application profiles configured in this request. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.4.2-2. |

Table 8.4.2-2: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| Application ID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Application Icon | O | Icon of the Data Channel Application |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: a) Precall only: the Data Channel Application is allowed to be used before the eMMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. b) Incall only: the Data Channel Application is allowed to be used after the eMMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. c) Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload (see NOTE) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peeer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: d) Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. e) Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. f) Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: - Service area: the application is allowed to be used in a certain area. - Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| NOTE: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. It also can be used in conjunction with condition fields. |  |  |

2.	Upon receiving the request, the eMMTel Enabler Server initiates authentication procedures with the Requester Identity and authorizes the Controlling Application Server if needed. If the registration is successful, the eMMTel Enabler Server  the creates and stores the DC Application Profile for each DC Application included in the DC Application Profile configuration request.

3.	The eMMTel Enabler Server sends a DC Application Profile configuration response to the Controlling Application Server. The response message includes information elements as specified in Table 8.4.2-3.

Table 8.4.2-3: Information elements for DC Application Profile configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of DC Application Profiles included | M | Indicates total number of DC Application Profiles included in this request |
| DC Application configuration response list | M | A list of Application configuration response. Each element in this list contains a response for a DC Application Profile. The detailed information is listed in Table 8.4.2-4. |

Table 8.4.2-4: Information elements in DC Application configuration response list

| Information element | Status | Description |
| --- | --- | --- |
| Configuration result | M | Indicates the success or failure of the configuration of this DC Application Profile. |
| Reason | O | Indicates the reason for failure. |

* * * End of Changes * * * *