3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240268

10th – 18th July 2024	(revision of S6a240076)

Source:	China Mobile

Title:	Sol for KI#4 Application enablement layer capabilities usage to supporting the differentiated XR service

Spec:	3GPP TR 23.700-01

Agenda item:	8.6

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#4,

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

2. Reason for Change

It is proposed to provide differentiated XR service with different QoS requirements to different XR service users, based on the interaction with NSCE server and SEALDD server.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01.

* * * First Change * * * *

7.x	Solution #x: Application enablement layer capabilities usage to supporting the differentiated XR service

7.x.1	Architecture Impacts

No architecture impact is identified.

7.x.2	Solution description

Figure 7.x.2-1 illustrates the procedure to support the differentiated XR service. The differentiated XR service provides different QoS for different users in single XR service.

Pre-conditions:

1.	The SEALDD server acts as NSaaS provider, and is authorized to get network slice related capabilities from NSCE server.

Figure 7.x.2-1: procedure of supporting the differentiated XR service

- The VAL server sends a XR resource management request to SEALDD server. The request includes Service Description, different service requirements, and corresponding different target user identifiers. When the target user corresponding to the requirement is not provided, the requirement is the default requirement. Only one requirement could be default requirement.

2a.	The SEALDD server determines whether and which slice is needed based on the service requirements and may initiate the slice related communication service lifecycle management as defined in TS 23.435, clause 9.12.2. For example, a HDLLC slice would be request for prime consumer, and eMBB slice for others.

Before triggering the network slice management service, to determine the target UE of network slice(s), the SEALDD server acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that SEALDD server has subscribed to consume 5GC services like LCS or NEF monitoring events related to UE actual location, or UE mobility analytics from NWDAF) or Location service from SEAL.

2b.	The SEALDD server determines whether the SEALDD enabled bandwidth control is needed, and may initiate the SEALDD enabled bandwidth control for different VAL users as defined in TS23.433, clause 9.8.2.

NOTE: the bandwidth control is internal behavior in SEALDD.

2c.	The SEALDD server determines whether it is needed to request a specific session. If needed, the SEALDD server initiates AF session with required QoS update as defined in TS 23.502, clause 4.15.6.6a, or Multi-member AF session with required QoS as defined in TS 23.502, clause 4.16.5.3.

NOTE: Step 2a-2c could be perform partially or fully based on the requirements. The SEALDD server decides which one or several steps from steps 2a-2c to execute. For example, when the requirements do not have clear target users, different slices in step 2a could be utilized. When the specific target users are clear, step 2c can be used. When  requirements only cover bandwidth, step 2b can be used.

3.	The SEALDD server sends the XR resource management response to the VAL server indicating the result of the request.

7.x.3	Solution evaluation

This solution addresses KI#4, the XR resource management service with different QoS requirements is provided to different users, based on the interaction with NSCE server and 5GC.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>