3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240273

10th – 18th July 2024	(revision of S6a240081)

Source:	China Mobile

Title:	Evaluation and conclusion on KI#6

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	zhengshaowen@chinamobile.com

1. Introduction

This paper provides evaluation and conclusion for KI #6: XR EAS selection based on N6 delay measurement.

2. Reason for Change

The evaluation and conclusion for KI #6 is needed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change (all new text) * * * *

9.3.X	Key Issue #6 solutions evaluation

Key issue #6 studies the XR EAS selection enhancement for reducing service latency. There are three solutions addressing the related issues: solution #10, solution #12 and solution #X.

-	Solution #10 proposes that the ECS obtains predicted application session connectivity information, and the Service provisioning is enhanced considering the closeness of the EES endpoint and the predicted application session connectivity information.

-	Solution #12 proposes to add EAS UPF information (i.e. UPF information related to the EAS) to EAS profile. The EAS discovery is enhanced based on the EAS related UPF information and the UE serving UPF information.

-	Solution #X proposes to utilize N6 delay measurement mechanism to enhance the EAS discovery. After candidate EAS(s) discovery is performed, the EES sends request to 5GC for N6 delay measurement for each pair of candidate XR EAS and UPF related to DNAI of the XR EAS.

In order to reduce the service latency, solution #X proposes to request N6 delay measurement. The service latency may be varied, which causes that the UPF with the lowest service latency to a certain EAS may not be always the same one. Therefore, the service latency should be measured, and the measurement of the service latency should be provided to the EES/EEC.  The additional delay introduced by the N6 delay measurement affects the delay of EAS discovery and will not affect the actual service delay. The exact information element and service of N6 delay measurement depend on SA2 working progress.

Solution #10 and solution #12 propose to enhance the Service provisioning and EAS discovery based on UPF information to make sure the EES and EAS selected is optimal on the topology. What UPF information could be provided is based on the progress in SA2.

* * * Next Change (all new text) * * * *

10.X	Conclusions of key issue #6

It is agreed to adopted the following principles in the normative work:

- N6 service latency can be taken into consideration during the EAS discovery.

- Editor’s Note: Whether and how the N6 delay measurement is exposed depending on the progress of SA2.

- Prediction information may be used for XR EAS selection if needed. The detail of such information and how to obtain such information should be defined in normative work.

Editor’s Note: The conclusion and overall evaluation of Key Issue #7 may be updated accordingly based on the progress of solution discussion and SA2 working progress.

* * * End of Change(s) * * * *