3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240282

10th – 18th July 2024	(revision of S6a240104 Rev1)

Source:	China Mobile

Title:	Pseudo-CR on Architecture of TS23_392

Spec:	3GPP TS 23.392 v0.0.0

Agenda item:	9.12

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes text for TS 23.392 Architecture. The text is based on clause 8.1 of TR23.700-92.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.0.0.

* * * First Change * * * *

6.	Architecture

Editor's Note: This clause provides the architecture of MMTelAPP Enabler.

6.1	General

This  clause specifies the architecture of MMTel Enabler.

6.2	MMTel Enabler architecture

Figure 6.2-1 shows the MMTel Enabler architecture.

The MMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: MMTel Enabler architecture

The MMTel functional entities on the UE and the server sides are MMTel Enabler client and MMTel Enabler server respectively.

The MMTel Enabler client communicates with the MMTel Enabler server over the MMTel-1 reference point to support the application layer interactions needed in MMTel service.

NOTE	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The MMTel Enabler client also communicates with the DCMTSI client over the MMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the MMTel Enabler Server. The DCMTSI client is specified in 3GPP TS 26.114 [26114].

The Application Server communicates with the MMTel Enabler server over the MMTel-2 reference point to invoke the MMTel service related capabilities provided by MMTel Enabler server,

The Controlling Application Server communicates with the MMTel Enabler server over the MMTel-3 reference point to manage the information needed in MMTel service.

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

6.3	Functional entities

6.3.1	General

The functional entities for MMTel Enabler are described in the following subclauses.

Editor's note:	 whether more functionalities of the MMTel Enabler client and the MMTel Enabler server are needed is FFS.

6.3.2	MMTel Enabler Server

The MMTel Enabler Server provides the server side functionalities corresponding to the MMTel service.

The MMTel Enabler Server supports:

-	the Application Server and Controlling Application Server to discover, to authenticate and to get authorization the MMTel/MMTel service APIs provided by the MMTel Enabler server;

-	the interaction with Controlling Application Server to configure the data channel application profile;

-	the integration of the multiple network layer capabilities provided by the underlying network, including the 3GPP core network and IMS into one service enabler layer API. The MMTel Enabler Server exposes these service enabler layer APIs to the Application Server and Controlling Application Server instead of exposes the network layer capabilities directly. The capabilities provided by the underlying network include:

a)	the APIs provided by DCSF via DC3/N33 reference point;

b)	the APIs provided by DCSF via DC4 reference point; and

c)	the APIs provided by MF/MRF via MDC3/MDC4 reference points;

-	the communication with the corresponding functions in the underlying network on behalf of the Application Server and Controlling Application Server when the capabilities provided by the underlying network are needed to be invoked;

-	the interaction with MMTel Enabler client for exchanging service configuration related information, e.g. the data channel application profile, needed in MMTel service,

-	providing MMTel/MMTel specific value added services which can be used by the Application Server in the mmtel communication between UE and Application Server;

-	the interaction with SEAL functionalities to use the corresponding SEAL services in MMTel service, e.g. to use  SEAL notification management service to update of data channel application profiles to UE;

-	the storage of the application layer information associated with the MMTel services;

-	the media function to support the media handling/translation, e.g. acts as a media gateway of WebRTC or if supported media type of application is different with the UE; and

-	the HTTP Proxy to support the establishes a Bootstrap data channel and application data channel, e.g. if the address type of application is different with the UE, e.g. IPv4 and IPv6, etc.

Editor's note:	Whether other functionalities, e.g. function to handle MMTel service related Signalling, are needed in the MMTel Enabler Server is FFS.

6.3.3	MMTel Enabler Client

The MMTel Enabler Client provide the client side functionality needed in MMTel service.

The MMTel Enabler Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the data channel application profiles provided by the MMTel Enabler Server.

NOTE 1:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications.

NOTE 2	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

6.3.4	Application Server

The Application Server communicates with the MMTel Enabler server over the MMTel-2 reference point to invoke the MMTel service related capabilities to communicate with the UE, e.g. establishes a MMTel session with or without IMS data channel to the UE, or establishes an application data channel to the UE. It may also communicate with Application Client to provide Application specific logic.

NOTE:	The Application Server needs to support MMTel-2 reference point, but the detailed functionalities of the Application Server is out of scope of the present document.

6.3.5	Controlling Application Server

The Controlling Application Server is a specific Application Server that only used by the MMTel service provider, i.,e. MNO, to manage the service configuration related information, e.g. data channel application profiles, needed in MMTel service. It communicates with the MMTel Enabler server over the MMTel-3 reference point to invoke the MMTel service related capabilities and does not interacts with Application Client on the UE directly.

NOTE:	The functionalities of the Controlling Application Server is out of scope of the present document.

Editor's note:	 Whether the controlling AS functionalities can be provided by OAM or not is FFS and may require coordination with SA5.

6.3.6	Application Client

The Application Client is an entity in the application layer to implement and perform the application service logic for its own service.

NOTE:	The Application Client can be the data channel application client. The functionalities of the Application Client is out of scope of the present document.

6.4	Reference points

6.4.1	General

The reference points for MMTel Enabler are described in the following subclauses.

Editor's note:	 The functions of the reference points are FFS and to be updated per solution.

6.4.2	MMTel-1 (between the MMTel Enabler client and the MMTel Enabler server)

The MMTel-1 reference point supports the interactions between a MMTel Enabler client and the corresponding MMTel Enabler server. This reference point supports the exchange of service configuration related information needed in MMTel service.

6.4.3	MMTel-2 (between the MMTel Enabler server and the Application Server)

The interactions between the Application Server and the underlying network are provided by the MMTel Enabler server via MMTel-2 reference point to ensure the security access of the underlying network. This reference point supports the Application Server to discover MMTel service APIs, to authenticate, to get authorization and to communicate with the MMTel service.

6.4.4	MMTel-3 (between the MMTel Enabler server and the Controlling Application Server)

The interactions between the MMTel Enabler server and the Controlling Application Server are referred to MMTel-3 reference point. This reference point supports the Controlling Application Server to manage the service configuration related  information needed in MMTel service.

6.4.5	MMTel-4 (between the MMTel Enabler client and the DCMTSI client)

The interaction between MMTel Enabler Client and DCMTSI Client.are referred to MMTel-4 reference point. This reference point is internal interaction in the dialler application on the UE and implementation specific.

NOTE:	The functionalities of the MMTel-4 reference point is out of scope of the present document.

6.4.5	App-1 (between the Application Client and the Application Server)

The App-1reference point supports the communication between  Application Client and Application Server to provide the Data Channel Application specific logic.

NOTE:	The functionalities of the App-1 reference point is out of scope of the present document.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[26114]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".