3GPP TSG-SA WG6 Meeting #62-ad hoc-e	S6a240296

e-meeting, 10th – 18th July 2024	(revision of S6a240193)

Source:	Nokia

Title:	Initial evaluation of KI#1

Spec:	3GPP TR 23.700-22 v0.3.0

Agenda item:	8.7

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Initial evaluation of KI#1.

2. Reason for Change

In TR 23.700-22 v0.3.0, to address KI#1, there are 3 solutions – Sol#2, Sol#3, Sol#Y (S6a240191).

An initial evaluation of these solutions for KI#1 is proposed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.3.0.

* * * First Change * * * *

# 9	Overall evaluation

## 9.1	Evaluation of key issue#1

Clause 5.1 captures the key issue on managing resource owner consent. The following open issues are listed:

1.	How consent of the resource owner can be managed through communication between the resource owner and authorization function in the CAPIF core function

2.	Whether (and how) “purpose of data processing” will be captured and where will it be stored?

3.	How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

Solution#3 specified in clause 6.4 addresses open issue#4 on finer granular access control for service API by enhancing the existing access control by introducing the resource level access.

Solution#Y specified in clause 6.Y addresses open issue#1 to manage consent of resource owner through communication between ROF and CCF (Authorization Function) using subscribe-notify mechanism for processing pending user consent requests related to one or more API invokers.

Editor’s note:	Further evaluation of KI#1 considering the related solutions is FFS.