3GPP TSG-SA WG6 Meeting #62-ad hoc-e	S6a240299

e-meeting, 10th – 18th July 2024	(revision of S6a240196)

Source:	Nokia

Title:	A-DACM architecture update

Spec:	3GPP TR 23.700-21 v1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for A-DACM architecture update.

2. Reason for Change

In TR 23.700-21 v1.0.0, clause 6.2.1, the following EN is to be resolved:

Editor's Note: How VAL client(s) access the services of A-DACM function is FFS.

The following options of access are possible:

- The VAL client can use a corresponding SEAL client to access the A-DACM service which is compliant with the architecture in TS 23.434. This limits the VAL client to access only the services/APIs exposed on SEAL-UU reference point. The services/APIs exposed over SEAL-UU reference point does not include all the services/APIs exposed over SEAL-S reference point.

- The VAL client (acting as API invoker) can directly invoke the service APIs of A-DACM (acting as AEF) which is compliant with CAPIF as specified in TS 23.222. This enables VAL client to consume the services/APIs exposed on SEAL-VC reference point. The services/APIs exposed over SEAL-VC is the same as the services/APIs exposed over SEAL-S reference point.

Both approaches are to be captured in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.0.0.

* * * First Change * * * *

## 6.2	Option #2: A-DACM architecture to support Metaverse services

### 6.2.1	Application enablement architecture

Figure 6.2.1-1 illustrates the A-DACM architecture to support Metaverse services using SEAL architecture specified in 3GPP TS 23.434 [8].

Figure 6.2.1-1: A-DACM architecture to support Metaverse services using SEAL architecture

The SEAL architecture includes A-DACM client and server functions which supports the VAL layer. The A-DACM server interacts with the VAL server(s) over the SEAL-S reference point. The A-DACM client interacts with the VAL client(s) over the SEAL-C reference point. The interactions between A-DACM client and A-DACM server is ove the SEAL-UU reference point. The A-DACM server may utilize the 5GC services over N33 reference point.

Figure 6.2.1-2 illustrates the A-DACM architecture to support Metaverse services using CAPIF architecture specified in 3GPP TS 23.222 [14].

Figure 6.2.1-2: A-DACM architecture to support Metaverse services using CAPIF architecture

The SEAL architecture includes A-DACM function which supports the VAL layer. The VAL server(s) (acting as CAPIF’s API invoker) interact with the A-DACM function (acting as CAPIF’s AEF) over the SEAL-S reference point (compliant with CAPIF-2/2e). The VAL client(s) (acting as CAPIF’s API invoker) interact with the A-DACM function (acting as CAPIF’s AEF) over the SEAL-VC reference point (compliant with CAPIF-2/2e). The service APIs offered by A-DACM function on SEAL-VC and SEAL-S to the VAL layer functions follow the service-based architecture as shown in the figure 6.2.1-3. The procedures of CAPIF as specified in 3GPP TS 23.222 [14] apply for the interactions between VAL layer and A-DACM function. The A-DACM function may utilize the 5GC services over N33 reference point.

Figure 6.2.1-3 illustrates the service-based SEAL architecture to support Metaverse services.

Figure 6.2.1-3: Service-based SEAL architecture to support Metaverse services

The A-DACM function exposes the APIs over the Sdacm interface to the VAL layer. The VAL functions (VAL servers) consume the services by interacting on the Sdacm interface of the A-DACM function. The CAPIF can be used by VAL functions to discover the services of the A-DACM function.

NOTE:	The A-DACM function can be realized as a new SEAL service or part of existing SEAL service.