3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240301

10th – 18th July 2024	(revision of S6a240187)

Source:	InterDigital Inc., Samsung

Title:	Pseudo-CR on Solution 12 update

Spec:	3GPP TR 23.700-21

Agenda item:	8.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This pCR clarifies solution 12.

2. Reason for Change

Reason 1

In Solution 12, a procedure is defined to allow a PIN Element to discover other PIN Elements(s) by requesting the PIN Management Client.

Step 2 of that procedure contains the following text:

2-	The PIN Management client authorizes the PIN client of a PIN element and discovers list of PIN elements based on provided parameters in the request.

It is not described in Solution 12 how the PIN Management Client “discovers” the list of PIN Elements(s) to be provided in the response.

Therefore a clarification is proposed in step 2.

Reason 2

The solution contains this Editor’s Note:

Editor's Note: Whether the PIN join subscribe-notification is different from this procedure is FFS.

If the requesting PIN Element has not subscribed for PIN Updates (e.g., has not obtained the PIN dynamic profile via the subscription), it will not be notified and the procedure of Solution 12 is needed.

Therefore, it is proposed to replace the EN by NOTE 1.

Reason 3

The solution contains this Editor’s Note:

Editor's Note: How PINE element will select the best PIN element to offload the task is FFS.

The PIN Element information includes limited information about the capabilities to offload a task to a PIN Element.

Therefore it is proposed to repace the EN by NOTE 2.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.0.0.

* * * First Change * * * *

#### 7.12.1.1	PIN client requests to discover another PIN element

Figure 7.12.1.1-1 illustrates procedure of PIN client requests to discover another PIN element, based on request/response model.

Pre-conditions:

1.	The UE (PIN client) has already discovered PIN as specified in clause 8.5.7 of 3GPP TS 23.542 [12];

2.	The UE (PIN client) has already joined the PIN as specified in clause 8.5.8.2 of 3GPP TS 23.542 [12]

Figure 7.12.1.1-1: discovery PIN element

1.	The PIN client of a PIN element sends request the PIN Management client to discover another PIN elements. The request includes the security credentials of the PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location, PIN ID as an identifier of the PIN and services/capabilities which is required by the requesting PIN element and time duration till when the PIN element is required.

2.	The PIN Management client authorizes the PIN client of a PIN element and discovers list of PIN elements based on provided parameters in the request and on information of PIN Element registered in the PIN. The PIN elements registered in the PIN may be known at the PIN Management Client (e.g., in the PIN dynamic profile) or otherwise may be obtained by the PIN Management Client from the PIN Server. The PIN Management client sends response back to the PIN client. The response contains list of PIN elements along with PIN client profiles for each discovered PIN elements, in case of success. The response includes failure cause in case of failure.

NOTE 1:	The information about PIN Element registered in the PIN (e.g., in PIN Dynamic profile, and in PINE registration) may be updated in the normative phase to include information about task offloading (e.g., task offloading capable, application(s), time duration, etc.).

NOTE 2:	If multiple PIN elements are discovered, then how to select the best PIN element to offload the task is up to application level implementation.

NOTE 3:	The PIN Management PINE join into PIN response as specified in clause 8.5.8.3.3 does not include list of already joined PIN elements, so the newly joined PIN element is not aware of already joined PIN elements.

* * * End of Changes * * * *

### 7.12.3	Corresponding APIs

Table 7.12.3-1 shows the request sent by a PIN client of a PIN element to a PEMC for the PIN element discovery request.

Table 7.12.3-1: PIN element discovery request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to join in. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |
| Discovery filters | M | Provides discovery filters to discover PIN elements |
| > UE location | O | The location information of the UE. The UE location is described in clause 7.2.7 of 3GPP TS 23.542 [12]. |
| > required PIN services | O | Specifies PIN services as required by the PIN element (as specified in Table 8.2.2.1-1 of 3GPP TS 23.542 [12]). |
| > time duration | O | Specifies time duration till when the service from the PIN element is required |

Table 7.12.3-2 shows the response sent by the PEMC to the requester for a PIN element discovery response.

Table 7.12.3-2: PIN element discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Discovered PIN elements List | O | List of discovered PIN element(s). Each element includes the information described below |
| > PIN client ID | M | Identifier of the PIN client. |
| > PIN client profile | O | PIN client profile |

* * * Next Change * * * *

### 10.2.6	Key issue #6: Support device discovery to offload task for metaverse services

Solution #12 addresses the key issue 6.

The solution uses PINAPP architecture and provides procedure to discover other PIN elements to offload the task.

* * * Next Change * * * *

## 11.2	Conclusions of key issue #x

The study concludes with following solution considerations for the normative work:

1.	Following individual solutions, corresponding to the key issues, will be considered as candidate solutions:

i.	for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

vi.	for Key issue #6 (Support device discovery to offload task for metaverse services) Solution #12 can be considered for normative work.

* * * End of Changes * * * *