3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240313

10th – 18th July 2024	(revision of S6a240117)

Source:	Samsung

Title:	Pseudo-CR on adding solution#8 evaluation

Spec:	3GPP TR 23.700-01 V1.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

Adding solution evaluation.

2. Reason for Change

Adding solution evaluation for Solution#8 - Leveraging S&F events information at application enablement and other minor edits.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V1.0.0.

* * * First Change * * * *

### 7.2.8	Solution #AE8: Leveraging S&F events information at application enablement

#### 7.2.8.1	Solution description

This solution maps to KI#7. This solution is about leveraging S&F events information at the application enablement for the optimized service delivery. In this scenario, AS (e.g. SEAL server) is considered to be deployed on the ground and UPF onboard the satellite as shown in Figure 7.2.8.1-1.

Figure 7.2.8.1-1: Deployment scenario

##### 7.2.8.1.1	AS subscribing to S&F events information with 5GC

Figure 7.2.8.1.1-1: Procedure for AS subscribing to S&F events information with 5GC

1.	AS (e.g. SEAL server) sends subscribe S&F information request to 5GC (e.g. NEF) e.g. for getting feeder link and service link availability information, expected data delivery time.

NOTE:	The request may include UE information if AS (e.g. SEAL server) is subscribing to events related to a specific UE.

2.	5GC then authenticates and authorizes the AS requesting to receive S&F information.

3.	5GC sends success or failure response to the AS related to the request made in step 1. If S&F information is available then the success response may also include the S&F information e.g. for getting feeder link and service link availability information, expected data delivery time.

4.	5GC monitors the occurrence of any event related to S&F.

5.	Upon occurrence of any event related to S&F, 5GC sends S&F information e.g. for getting feeder link and service link availability information, expected data delivery time towards AS.

##### 7.2.8.1.2	S&F data delivery handling at AS

Pre-condition: AS (SEAL server) has subscription to S&F information with 5GC as described in 7.2.8.1.1

Figure 7.2.8.1.2-1: Procedure for S&F data delivery handling at AS (e.g. SEAL server)

1.	 VAL server sends send data request to AS (e.g. SEAL server) including the target UE/user and service identity information.

2.	AS (e.g. SEAL server) may store the received data request. AS (e.g. SEAL server) may optimize application/service behaviour e.g. adjust the frequency/timing of sending data, adjust the data size.

3.	AS (e.g. SEAL server) provides send data response to VAL sever e.g. the estimated delivery time information, adjust the data size.

NOTE:	AS (e.g. SEAL server) may send an acknowledgement to the VAL server after the data delivery request at step 1 is eventually delivered to the target UE/user.

Editor’s note:   The S&F events information exposed from 3GPP CN depend on the progress of SA2 Rel-19 study.

#### 7.2.8.2	Architecture Impacts

This solution has no architecture impacts to SEAL. Under the existing SEAL architecture, the SEALDD procedures are enhanced to support S&F functionality during satellite connectivity.

#### 7.2.8.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

#### 7.2.8.4	Solution evaluation

This solution is for KI#7 and addresses the open issue about enabling SEAL architecture to obtain and expose the S&F related information and to leverage that information for optimised application enablement behaviour. The solution does not required new architecture entity in SEAL.

This solution proposes how SEAL server (SEALDD server) obtains UE S&F events information from 5GC and then how such information is used by the SEAL server (SEALDD server) to minimize the service interruption (e.g., adjust the DL frequency, size of data, message acknowledgement handling, ack timers). The UE S&F status events exposed from 3GPP CN (i.e. LTE) depend on the progress of SA2.

The solution is feasible.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>