3GPP TSG-SA6 Meeting #62-Ad Hoc-e	S6a240338

# Online, , 10th Jul 2024 - 18th Jul 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-82 | CR | 0001 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolutions and missing eval and conclusion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIMLAPP |  |  |  |  |  | Date: |  |  | 2024-07-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some Editors’ Notes in the TR, which are not controversial, but need to be resolved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To solve the ENs, the following changes are proposed: - In 8.12.2, the EN is converted to NOTE since this will be discussed in normative phase. - In 8.22.3, the EN is converted to NOTE since the IE details are part of the normative work. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses: 8.12.2, 8.22.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 8.12.2	Consumer-based ML Model Performance Degradation Detection

Figure 8.12.2-1: Consumer-based ML Model Performance Degradation Detection

This procedure corresponds to the step 0b in clause 8.12.1.

0.	An ADAE Server, as an AI/ML Enablement Consumer, receives trained ML model (or ML model information) from the AI/ML Enablement Server.

1.	The ADAE Server receives request from consumer for analytics, generates analytics by using the ML model provided by AI/ML Enablement Server, and responses/notifies to the consumer with the required analytics.

2.	The ADAE Server requests the consumer to feedback usage result of the analytics.

3.	The consumer uses the analytics for its operations and collects operation results. Performance degradation may be found from the operation results. The performance degradation may cause by e.g., insufficient analytics accuracy, or the current analytics cannot fulfill the changed conditions at the consumer.

4.	The consumer feedbacks the usage result of the analytics to the ADAE Server.

5.	The ADAE Server updates the analytics accuracy based on feedback information from the consumer, and checks the performance of the ML model, which is used to generate the analytics, e.g., performance degradation of the ML model.

NOTE: The capability of ADAES to detect model performance degradation will be revisited during normative phase.

If the AI/ML Enablement Consumer is e.g. VAL Server, similar procedure as shown in Figure 8.12.2-1 be performed for detecting of ML Model performance, by replacing "analytics" with e.g., "operation instructions".

* * * * * * * SECOND CHANGE * * * * * * *

### 8.22.3	Corresponding APIs

Table 8.22.3-1 describes the API for Horizontal FL training subscription request.

Table 8.22.3-1: Horizontal FL training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The VAL server identifier. |
| AIML model and model parameters | M | Information about the AIML model and model parameters to use for HFL training/inferencing. |
| AIML enablement client list | M | A list of AIML enablement clients required for HFL training/inferencing. |
| Dataset requirements | O | Requirements for dataset required for HFL training/inferencing. |
| Number training rounds | O | Number of training rounds for HFL training |
| Number of AIML clients per round | O | Minimum number of AIML clients required for each training round |
| Notification settings | O | Settings for how often to send notifications providing status of HFL training. For example, periodic, event-based, upon completion. |

Table 8.22.3-2 describes the API for Horizontal FL training subscription response.

Table 8.22.3-2: Horizontal FL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request |
| Subscription ID | M | An identifier for the subscription |

Table 8.22.3-3 describes the API for Horizontal FL training subscription response.

Table 8.22.3-3: Horizontal FL training notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request |
| ML model parameters | O (NOTE) | Updated model parameters from training |
| ML model outputs | O (NOTE) | ML model outputs from inferencing |
| Training elapse time | O | Total elapse time of the training |
| Error list | O | A list of errors encountered during training |
| Timestamp | O | A timestamp for the notification |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE: Whether to include Training elapse time and Error list will be discussed during normative phase