3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240343

10th – 18th July 2024	(revision of S6a240240 was S6a240034)

Source:	ETRI

Title:	Spatial map management APIs

Spec:	3GPP TR 23.700-21 v1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	ByungJun AHN (bjahn@etri.re.kr)

1. Introduction

This pCR proposes APIs corresponding to the spatial map management procedures.

2. Reason for Change

Clause 7.8 of 3GPP TR 23.700-21 v1.0.0 addresses the solution to support for spatial map management. Procedures are provided without corresponding APIs. It is required to provide APIs corresponding to the spatial map management procedures.

3. Conclusions

APIs corresponding to the spatial map management procedures are provided.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.0.0.

* * * First Change * * * *

7.8.4	Corresponding APIs

7.8.4.1	Produce spatial map

Table 7.8.4.1-1 shows the produce spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to produce a spatial map.

Table 7.8.4.1-1: Produce spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g. access control rules) |

NOTE 1:	The requested information will be further defined during the normative work phase.

Table 7.8.4.1-2 shows the produce spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the produce spatial map request.

Table 7.8.4.1-2: Produce spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | O | ID of the produced spatial map |
| Spatial map information | O | If Spatial map ID is valid, a collection of information is included. The information corresponds to the produced spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them |

NOTE 2:	Details about Spatial map information is FFS and will be considered during the normative work phase.

7.8.4.2	Update spatial map

Table 7.8.4.2-1 shows the update spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to update a spatial map.

Table 7.8.4.2-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | ID of the spatial map to update |
| Instruction | M | Instruction to update or delete |
| Requested update information | O | Information on what to update/modify or delete, e.g. access control rules or spatial map coverage area, etc. |

NOTE 1:	The CRUD operations for spatial map management APIs will be enhanced in normative work.

NOTE 2:	The Requested update information will be further defined during the normative work phase.

Table 7.8.4.2-2 shows the update spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the update spatial map request.

Table 7.8.4.2-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | M | If the Result is success, ID of the updated spatial map is included. |
| Spatial map information | O | If Spatial map ID is valid, updated spatial map information is included. |

7.8.4.3	Get spatial map

Table 7.8.4.3-1 shows the get spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to get  spatial maps.

Table 7.8.4.3-1: Get spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Spatial map ID(s) | O | ID(s) of the spatial map to get information of it |
| > Area of interest | O | Three-dimensional area information to discover spatial maps |
| > Localization information | O | A known location to discover spatial maps in the three dimensional space, including an orientation |
| > Discovery filters | O | A set of characteristics to search the matching spatial maps |

NOTE:	The Discovery filters will be further defined during the normative work phase.

Table 7.8.4.3-2 shows the get spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the get spatial map request.

Table 7.8.4.3-2: Get spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| List of discovered spatial maps | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial map ID | M | ID of a spatial map discovered |
| > Spatial map information | M | A collection of information that corresponds to the discovered spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them |

7.8.4.4	Subscribe/unsubscribe spatial map

Table 7.8.4.4-1 shows the subscribe spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to subscribe spatial maps.

Table 7.8.4.4-1: Subscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to subscribe |
| > Localization information | O | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3 dimensional space. |
| List of events for discovered target spatial map(s) | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | Type of event to subscribe |
| > Event ID | O | ID of the event to subscribe |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g., added or removed objects, position or direction changes of the objects in the spatial map etc. |
| > Notification intervals | O | Specifies when to notify the detected event, e.g., promptly or periodically with minimum time between consecutive notifications, etc. |

NOTE 1:	Details of how to define events, including by whom and what, will be specified during the normative work phase.

Table 7.8.4.4-2 shows the subscribe spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the subscribe spatial map request.

Table 7.8.4.4-2: Subscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the subscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list subscribed | M | List of the events subscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event subscribed |
| >>> Event ID | M | ID of the event subscribed |
| >>> Result | M | Result of the subscribe request for the event (e.g., success or fail), including cause of failure if needed. |
| >>> Subscription ID | O | Allocated ID for the successful subscription to the event |

Table 7.8.4.4-3 shows the unsubscribe spatial map request sent by VAL server (or SEAL LM client) to SEAL LM server to unsubscribe spatial maps.

Table 7.8.4.4-3: Unsubscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to unsubscribe |
| > List of spatial map IDs | O | ID(s) of the spatial map(s) to unsubscribe |
| > Area of interest | O | Three-dimensional area information to discover spatial map(s) to unsubscribe |
| > Localization information | O | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3 dimensional space. |
| List of events for discovered target spatial map(s) | M | Events to unsubscribe. Each element includes the information described below. |
| > Event type | O | Type of event to unsubscribe |
| > Event ID | O | ID of the event to unsubscribe |
| > Subscription ID | O | Subscription ID to unsubscribe, which was allocated by the SEAL server when the requestor subscribed the event |

Table 7.8.4.4-4 shows the unsubscribe spatial map response sent by SEAL LM server to VAL server (or SEAL LM client) as the response to the subscribe/unsubscribe spatial map request.

Table 7.8.4.4-4: Unsubscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the unsubscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list unsubscribed | M | List of the events unsubscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event unsubscribed |
| >>> Event ID | M | ID of the event unsubscribed |
| >>> Result | M | Result of the unsubscribe request for the event (e.g., success or fail), including cause of failure if needed. |

Table 7.8.4.4-5 shows the spatial map notification sent by SEAL LM server to VAL server (or SEAL LM client) when a subscribed triggering event is detected.

Table 7.8.4.4-5: Notification

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the spatial map that detected the triggering event |
| Event ID | M | ID of the detected triggering event |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |

NOTE 2:	The Notification information will be further defined during the normative work phase.

* * * End of Change * * * *