3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6-240354

10th – 18th July 2024	(revision of S6a240266, S6a240074)

Source:	China Mobile

Title:	Architecture Update

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

It is proposed to update the architecture.

2. Reason for Change

Different architecture options is added for discussion.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

6	Application enablement architecture for XR services

6.1	Application enablement architecture

6.1.1	General

This clause provides the overall architecture description.

6.1.2	Option1: Application enablement architecture for XRAPP function

When the XRApp function is a SEAL layer function, it could be deployment implementation by integrating the related enabler function (e.g. SEALDD server, NSCE server) to support XR application.

The XRApp function provides the service to VAL server over XRApp-S. The XRApp function interact with XRApp client over XRApp-UU. The XRApp client communicates with the XRApp client on the other device (i.e., tethered device) over the XRApp-UUe.

Figure 6.1.2-1: overview of XRApp function

6.1.2.1	Functional Elements

6.1.2.1.1	Server-side XRApp Functionality

The XRApp functional entity provides for the XR service enablement.

6.1.2.1.2	Client-side XRApp Functionality

- The XRApp client functional entity acts as the application client for enabling the XR related service.

6.1.3	Option2: Application enablement architecture based on existing application enablement capabilities

Instead of defining an additional independent architecture, XR related services could be supported by enhancing relevant existing functions(e.g., SEALDD).

6.1.3.1	Application enablement architecture based on SEALDD

To facilitate the specific optimization for XR application provided by 5G network, the application enablement architecture for the XRApp service is based on the generic functional model specified in 3GPP TS 23.434[4] and 3GPP TS 23.433 [6].

The XRApp can be deployed as functionality component of the SEALDD, then the architecture for SEALDD is applicable to XRApp.

Editor's Note: Whether a independent XRAPP architecture is needed is FFS, the gap between the XRApp and SEALDD needs to be identified.

Figure 6.1.3.1-1:Application enablement architecture based on SEALDD

To support the Tethered device, the architecture is discussed in KI#5.

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>