3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240355

10th – 18th July 2024	(revision of S6a240237, S6a240162)

Source:	Huawei, Hisilicon

Title:	Clarification on the service discovery and invocation architecture for XR application

Spec:	3GPP TR 23.700-23 v1.0.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jian Zhang (zhangjian369@huawei.com)

1. Introduction

This paper is proposed to add the related description to clarify how to invoke the required service API (e.g. SEAL service, EDGEAPP service, 5G network service) to support XR application.

2. Reason for Change

For the key issue #4 about the application enablement layer capabilities usage to support the XR services, including the following issues:

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

To solve the above open issues, this paper is to clarify that the border AEF in CAPIF (similar with the API gateway in IT) can be utilized to jointly support the XR service by invoking the application enablement capabilities (e.g. SEAL service, EDGEAPP service, 5G network service). Please find more details in the related discussion paper (S6a240161).

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v1.0.0.

* * * First Change * * * *

6.1.x	CAPIF architecture to support the service discovery and invocation for XR application

The border AEF in existing CAPIF architecture in TS 23.222 [x] can be utilized to support the XR service by invoking the northbound API exposed by the application enablement (e.g. SEAL, EDGEAPP) or the the northbound API exposed by 5GS, as described in Figure 6.1.x-1.

Figure 6.1.x-1: Reuse CAPIF architecture to support the service discovery and invocation for XR application

The XR application acts as API invoker performs the API discovery and receives the details of the service API (i.e. border AEF).  Then the XR application performs service API invocation according to the border AEF address. The border AEF acts as the entry endpoint to route service API request to multiple AEFs (e.g. EDGEAPP service, SEAL service) based on the routing information obtained from CCF.

NOTE 1:  The detailed services of enablement layer need to be invoked depends on the specific scenario and use case for XR application.

NOTE 2: The EDGEAPP service in this part refers to the service discovery and invocation in EDGE-3 interface between EAS and EES.

NOTE 3: The border AEF in this part is the entry point to hide the AEF topology, it is not mandatory depends on the specific deployment for service exposure.

* * * Next Change * * * *

10.X	Conclusions of key issue #4

For the aspect of service discovery and invocation for XR application in key issue #4, the existing CAPIF mechinism can be utilized to support the XR service by invoking the northbound API exposed by the application enablement (e.g. SEAL, EDGEAPP) or the the northbound API exposed by 5GS.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS23.433: "Service Enabler Architecture Layer for Verticals (SEAL);Data Delivery enabler for vertical applications;  Stage 2".

[7]	3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[8]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

[9]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[10]	3GPP TR 22.847: "Study on supporting tactile and multi-modality".

[11]	3GPP TS 23.558:"Architecture for enabling Edge Applications".

[12]	3GPP TS 23.434:"Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[13]	3GPP TR 22.856:"Feasibility Study on Localized Mobile Metaverse Services".

[14]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[15]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[16]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[17]	3GPP TS28.538: "Management and orchestration; Edge Computing Management (ECM)".

[x]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".