3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240356

10th – 18th July 2024	(revision of S6a240253)

Source:	Ericsson

Title:	KI#8 overall evaluation

Spec:	3GPP TS 23.700-23 v1.0.0

Agenda item:	8.5

Document for:	Approval

1. Introduction

This paper introduces overall evaluation for KI#8.

2. Reason for Change

This paper introduces overall evaluation for KI#8.

3. Conclusions

This pCR proposes overall evaluation for KI#8.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23.

* * * First Change * * * *

### 9.3.x	Key issue #8 solutions evaluation

The open issue of KI#8 include:

-	Whether and how SEALDD or other application enabler may be enhanced to facilitate XR communication flows between application clients and application servers involved in the same application service in relation to PDU set (e.g., support for PDU set handling, QoS measurement)?

In the existing solutions, solution#4 addresses QoS measurement for XR flows including PDU set on crossflow RTT. The SEALDD measurement function is enhanced to identify the 1st UL or last DL packet in a PDU set, e.g. for SEALDD client triggered measurement, the SEALDD client needs to identify the last DL packet in a PDU set to complete the measurement for a DL video frame. The crossflow QoS measurement provides added value for the XR service, for example, Motion to Photon can be estimated as crossflow RTT measurement (e.g. {[traffic descriptor 1 for joystick input, UL]; [traffic descriptor 2 for video frame, DL]}).

Editor's note: Whether identifying packet in PDU set is necessary in crossflow RTT measurement is FFS.

In the existing solutions, solution#7 addresses PDU set handling support. The SEALDD layer is enhanced with RTP packetilization and PDU set inclusion.

# *** End of Changes ***