3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240357

10th – 18th July 2024	(revision of S6a240265, S6a240072)

Source:	China Mobile

Title:	Sol #2 update

Spec:	3GPP TR 23.700-01

Agenda item:	8.6

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution proposes to update Solution #2.

2. Reason for Change

Solution evaluation and information element are added to complete solution #2

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01.

* * * First Change * * * *

### 7.2.2	Solution #2: Enhancement for on board EES(s) and service provisioning

#### 7.2.2.1	Solution description

This solution proposes to address the following open issues in KI#5,

-	How the EES(s) are placed on board the Satellite.

-	Whether and how the service provisioning is impacted.​

It is proposed to add the Satellite assistant information in the EES profile to indicating the EES(s) are placed on board the satellite, so that, the ECS could do the service provisioning based on those Satellite assistant information.

##### 7.2.2.1.1	General

When EES is deployed in the satellite, it is proposed to add the Satellite assistant information in the EES Profile to indicating the EES is deployed on-board. The Satellite assistant information could be the satellite ephemeris information(e.g., time slot and spatial location). So that the ECS could obtain the EES’s Satellite assistant information during the EES registration.

Table 7.2.2.1.1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EDN information | O | EDN information where the EES resides. |
| > DNN | M | Data network name to identify the EDN. |
| > DNAI(s) | O | DNAI(s) associated with the EDN. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| >Allowed MNO information | O | Information of the allowed operator as described in EAS profile clause 8.2.4, Only subscribers from these operators can consume the EES services. |
| List of EAS bundle information | O | List of EAS bundles per EASID to which the EAS belongs and related bundling requirements. |
| > Bundle ID (NOTE 2) | O | A bundle ID as described in clause 7.2.10. |
| > List of EASIDs (NOTE 2) | O | List of EASIDs associated with the EAS bundle. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (NOTE 1) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability (e.g. EAS bundle information) of handling bundled EAS ACR. |
| Satellite assistant information | O | Assistant information indicating the EES is on-board, and could be used to calculate the satellite's position and movement. It could be the statistic satellite ephemeris information (e.g.,signal quality metrics and orbital elements) |
| NOTE 1: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". NOTE 2: At least one of the IEs shall be present if EAS bundle information is provided. |  |  |

After that, when UE requests the service provisioning, the following changes may apply.

Pre-condition:

1.	The ECS support the availability information calculation.

Figure 7.2.2.1.1-1: Service provisioning

-	In step2, the ECS calculates the EES availability information based on the Satellite assistant information, and determines the EES based on the EES availability information so that the EES is accessible to the UE. The EES availability information indicates when and how the EES is available for the requesting EEC. It could be a sequence of time durations for each grid point where each time duration includes an indication of coverage availability or unavailability as discussed in Annex Q of 3GPP TS 23.501 [5].

-	In step3, if the EES meet the requirement so that the on-board EES is selected by the ECS, the ECS should send the EES availability information to the EEC. The EEC can select one or more such EES(s) to perform EAS discovery based on EES availability information.

Since the EAS only registers to the EES on the same satellite, the registration of EAS is not affected. Regarding EAS discovery, EES selects the EAS(s) from those deployed on the same satellite. If there is no EAS meets the requirement, the request would fail.

#### 7.2.2.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts (if any) of the solution and possible new SA6 capabilities and interfaces.

#### 7.2.2.3	Corresponding APIs

EDN configuration information in the Service provisioning response needs to be changed as follows.

Table 8.3.3.3.3-2: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| > DNN/APN | M | Data Network Name/Access Point Name |
| > S-NSSAI | O | Network Slice information |
| > EDN Topological Service Area | O | The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| List of EESs | M | List of EESs of the EDN. |
| > EESID | M | The identifier of the EES |
| > EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| > EAS information (NOTE 2) | O | EAS registration and associated bundle information. |
| >> EASID | M | An EASID registered or expected to be registered with the EES. |
| >> List of EAS bundle information | O | List of EAS bundles to which the EAS belongs. |
| >>> Bundle ID (NOTE 3) | O | Bundle ID as described in clause 7.2.10. |
| >>> List of EASIDs (NOTE 3) | O | A list of the EASIDs of the EASs in the bundle. |
| > Application Group ID list (NOTE 5) | O | List of Application Group IDs associated with EAS |
| > Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) |
| >> Instantiation criteria (NOTE 4) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| > ECSP ID | O | The identifier of the ECSP that provides the EES. |
| > EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| > EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| > List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EES. |
| > EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| > Security Credential | O | Indicates the security credential sent by the ECS. The security credential is used by EEC to communicate with the EES as specified in 3GPP TS 33.558 [23], clause 6.3. |
| >EES Availability information (NOTE 6) | O | The available information of the EES, especially for the onboard EES. This information indicates when and where the EES would be available. |
| Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN or serving SNPN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". NOTE 5: "Application Group ID list" IE shall be present when "Application Group profile" is included for "AC profile" in service provisioning request as specified in clause 8.3.3.3.2. NOTE 6: EES Availability information should avoid the need for frequent updates. It could be a sequence of periodical time durations for each grid point where each time duration includes an indication of coverage availability or unavailability as discussed in Annex Q of 3GPP TS 23.501 [5]. |  |  |

* * * Next Change * * * *

* * * Next Change * * * *

<Proposed change in revision marks>