3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240360

10th – 18th July 2024	(revision of S6a240311, S6a240115)

Source:	Samsung

Title:	Pseudo-CR on adding solution#5 evaluation

Spec:	3GPP TR 23.700-01 V1.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

Adding solution evaluation.

2. Reason for Change

Adding solution evaluation for Solution#5 - Application enablers provisioning and expose the S&F events information and other minor edits.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V1.0.0.

* * * First Change * * * *

### 7.2.5	Solution #AE5: Satellite edge computing considering EAS onboard

#### 7.2.5.1	Solution description

##### 7.2.5.1.1	General

Figure 7.2.5.1.1-1: EAS onboard satellite deployment

In this deployment option, ECS and EES are deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access EAS is deployed on satellite. RAN (e.g. gNodeB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF. The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. UPF can be deployed on ground to access the ECS and EES, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space), and EEC (in UE) can reach the EES by EDGE-1 (on ground).

Considering that EAS(s) deployed in multiple MEO or LEO satellites, so that the EAS(s) are mobile as the satellites are moving relative to the Earth's surface. In this case, if the ephemeris information of the EAS is not considered for the discovery, frequent interruption may be experienced when there is a mismatch between the movement of the UE and the movement of the EAS on board satellite.

As for the EAS on MEO and LEO satellites however the EES is on the ground, in the EAS discovery procedure, the EES can determine a list of EAS based on the UE (predicted/expected) location, Prediction expiration time, Satellite ID and EAS ephemeris information and the EES provide list of EAS information in the EAS discovery response message. Upon receiving a list of EAS information, the UE can determine the EAS based on the UE (predicted/expected) location, Prediction expiration time, EAS Satellite ID and EAS ephemeris information, and UE may sequentially connect to corresponding EAS based on ephemeris information of the EASs. When an EAS is about to leave the UE’s location, the UE may connect to a next EAS in advance to obtain an edge computing service, and does not need to perform EAS discovery again. In doing so, the service interruption caused by periodic EAS discovery can be avoided.

Besides, the EEC can send Satellite ID in service provisioning request message to the ECS for EES information.

In addition, when a serving satellite is providing EAS and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EAS (i.e. to which the EEC had previously connected to), without needing to discover EAS endpoints again.

#### 7.2.5.2	Architecture Impacts

This solution has no architecture impacts to EDGEAPP. Under the existing EDGEAPP architecture, the EAS discovery procedure is enhanced to support different cases in discontinuous coverage.

#### 7.2.5.3	Corresponding APIs

##### 7.2.5.3.1	EAS Profile enhancement for EAS onboard

According to clause 7.2.1.1.3 EAS discovery for EAS onboard satellite the following change is needed in TS 23.558 [10].

-	3GPP TS 23.558 [10] Clause 8.2.4 EAS Profile EAS mobility on board LEO or MEO Satellite

3GPP TS 23.558 [10] Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X, UAV, application enabler) |
| EAS description | O | Human-readable description of the EAS |
| … | … | … |
| Satellite ID | O | The Satellite ID corresponding to the EAS. |
| ephemeris information | O | The ephemeris information of the satellite corresponding to the EAS |
| … NOTE 1: The assignment of the trajectory ID for EAS(s) can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculated for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |

#### 7.2.5.4	Solution evaluation

This solution is for KI#2 to addresses the open issue about deployment option for EDGEAPP and related enhancements to the EDGEAPP procedures including service provisioning and EAS discovery. The solution does not required new architecture entity in EDGEAPP.

The solution proposes ECS and EES are deployed on ground while EAS is deployed on satellite. In this solution the EES on the ground determines the list of EAS e.g. corresponding to the UE’s predicted location information then provides to the UE satellite IDs matching the UEs predicted location. EEC then connects to the appropriate EAS corresponding to the location according to the satellite IDs and avoids repeating EAS discovery for each location. Further the solution also avoids repeating EAS discovery by EEC re-connecting to the previously connected EAS associated to a satellite, when the same satellite returns in coverage after discontinuous coverage.

The solution is feasible.

Editor’s note: The procedures and the use of the satellite ID and ephemeris information in the procedures is FFS.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>