3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240362

10th – 18th July 2024	(revision of S6a240312, S6a240116)

Source:	Samsung

Title:	Pseudo-CR on adding solution#7 evaluation

Spec:	3GPP TR 23.700-01 V1.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

Adding solution evaluation.

2. Reason for Change

Adding solution evaluation for Solution#7 - Satellite edge computing considering EAS onboard and other minor edits.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V1.0.0.

* * * First Change * * * *

### 7.2.7	Solution #AE7: Satellite edge computing considering EAS onboard

#### 7.2.7.1	Solution description

##### 7.2.7.1.1	General

Figure 7.2.7.1.1-1: EAS and EES onboard satellite deployment

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. EASs and their registered EES are deployed on one or more satellites.

As for the EAS and EES on MEO or LEO satellites however the ECS is on the ground, in the service provisioning procedure, the ECS can determine a list of EES based on the UE (predicted/expected) location, Prediction expiration time, Satellite ID and EES ephemeris information and ECS provide list of EES information in the service provisioning response message. Upon receiving a list of EES information, the EEC can determine the EAS based on the UE (predicted/expected) location, Prediction expiration time, EAS Satellite ID and EAS ephemeris information, then the UE may sequentially connect to corresponding EESs based on ephemeris information of the EESs. When an EES is about to leave the UE’s location, the UE may connect to a next EES in advance to obtain an edge computing service, and does not need to perform EES discovery again. In doing so, the service interruption caused by periodic EES discovery can be avoided.

Furthermore, the EES can indicate the ECS whether the EAS and EES are deployed in the same Satellite, if the EES and EAS are deployed in the same satellite, then the ECS can provide EAS information (EAS endpoint and EAS ephemeris information) along with the EES information. Upon receiving the EAS information, the UE can connect to the EAS and the EAS discovery procedure does not need to be performed.

In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EES and EAS (i.e. to which the EEC had previously connected to), without needing to discover EES and EAS endpoints again.

Besides, the EEC can send Satellite ID in EAS discovery request message to the EES for EAS information.

#### 7.2.7.2	Architecture Impacts

This solution has no architecture impacts to EDGEAPP. Under the existing EDGEAPP architecture, the service provisioning and EAS discovery procedures are enhanced to support different cases in discontinuous coverage.

#### 7.2.7.3	Corresponding APIs

##### 7.2.7.3.1 	EES Profile enhancement for EES onboard

According to clause 7.2.7.1 service provisioning for EES onboard satellite the following change is needed in 3GPP TS 23.558 [10].

3GPP TS 23.558 [10] Clause 8.2.6 EES Profile EES mobility on board LEO or MEO Satellite

3GPP TS 23.558 [10] Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| … | … | … |
| Satellite ID | O | The Satellite ID corresponding to the EES. |
| ephemeris information | O | The ephemeris information of the satellite corresponding to the EES |
| … NOTE 2: The assignment of the trajectory ID for the EES can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculated for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |

#### 7.2.7.4	Solution evaluation

This solution is for KI#2 to addresses the open issue about deployment option for EDGEAPP and related enhancements to the EDGEAPP procedures including service provisioning and EAS discovery. The solution does not required new architecture entity in EDGEAPP.

The solution proposes ECS is deployed on ground while EES and EAS are deployed on satellite. In this solution the ECS on the ground determines the list of EES e.g. corresponding to the UE’s predicted location information, and then provides to the UE satellite IDs matching the UEs predicted location. EEC then connects to the appropriate EES to determine EAS corresponding to the location according to the satellite IDs and avoids repeating EAS discovery for each location. Further the solution also avoids repeating EAS discovery by EEC re-connecting to the previously connected EAS associated to a satellite, when the same satellite returns in coverage after discontinuous coverage.

The solution is feasible.

Editor’s note: The procedures and the use of the satellite ID and ephemeris information in the procedures is FFS.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>