3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240222_Rev4

10th – 18th July 2024	(revision of S6a240149)

Source:	Samsung

Title:	Pseudo-CR on Solution for spatial map management

Spec:	3GPP TR 23.700-21 v1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Pramodkumar Shah <sapan.shah@samsung.com>

1. Introduction

This pCR proposes new solution to the Key Issue #4 on spatial mapping. The solution proposes to enhance the spatial map with information related to the VAL Ues available in the spatial map along with the related VAL service(s).

2. Reason for Change

New solution for key issue #4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.0.0.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Avatar profile: It is avatar specific configuration and parameters (e.g. allowed locations) applicable to one or more application(s). The avatar profile may be associated with one or more VAL user(s).

Application-Digital Asset Container Management: An entity that securely manages the application related digital assets (e.g. avatars).

Digital Asset Identifier: In the context of this TR, digital asset identifier is used to uniquely identify a digital asset across different mobile metaverse services.

Processed sensor data: data for the Spatial mapping service to estimate the number of objects, type of the object, position, direction and speed of the object, etc.

Spatial map information: information to be included and managed for a Spatial map.

VAL UE augmented spatial map: spatial map augmented with VAL UE information.

For the purposes of the present document, the following terms given in 3GPP TS 22.156 [2] apply:

Avatar

Digital asset

Digital wallet

Spatial anchor

Spatial map

* * * First Change * * * *

## 7.x	Solution #x: Support for VAL UEs in spatial map

### 7.x.1	Solution description

This solution maps to KI#4. This solution enables to support the management of spatial map with VAL UEs information related to the VAL server and its VAL services. During create or update of the spatial map, based on request from the VAL server, this solution  enhances spatial map with information related to the VAL UEs that are available in the area related to the spatial map and are related to the VAL services of the VAL server. The produced spatial map will include VAL UEs information (position in spatial map) association with VAL services.

### 7.x.2	Architecture Impacts

The solution enhances the SEAL LM functionalities to manage the spatial map with VAL UEs information in the area related to the spatial map and that are related to the VAL server’s VAL services. For the spatial map management between the VAL server/SEAL LM client and SEAL LM server, reference points LM-S and LM-UU can be used in this solution.

### 7.x.3	Procedures

#### 7.x.3.1	Create spatial map augmented with VAL UE

Figure 7.x.3.1-1 illustrates the procedure to create a VAL UE augmented spatial map. The request is from VAL server or an SEAL LM client. The service is provided by SEAL LM server.

Editor’s note:	This procedure is to provide the spatial map service with VAL UE, not spatial map. The terminologies will be reconsidered.

Figure 7.x.3.1-1: Create spatial map with VAL UE

1.	The VAL server (or SEAL LM client) sends request to produce the VAL UE augmented spatial map to the SEAL LM server. The request includes area of interest, an indication to identify VAL UEs in the spatial map, VAL service(s) list. The indication informs the SEAL LM server to include (or tag) the information (example, position, location) related to the VAL UEs in the spatial map created, that are available in the area of interest related to the spatial map to be created. VAL Service(s) information indicates to SEAL LM server to tag the VAL UEs information related to these VAL service(s) in the created spatial map.

2.	The SEAL LM server will authenticate and authorize the requesting entity VAL server/SEAL LM client. If the VAL server or SEAL LM client is not authorized then the SEAL LM server sends failure response. If Authentication and authorization is success, then the SEAL LM server fetches the list of VAL UEs in the area of interest related to the spatial map to be created as in the request message, from SEAL LM server using clause 9.3.10 of 3GPP TS 23.434 [8] and/or from NEF as specified in 3GPP TS 23.502 [9]. The SEAL LM server identifies the VAL UEs information related to the VAL service(s) in the request message from the VAL server or SEAL LM client.

3.	The SEAL LM server produces the requested spatial map, using the processed sensor data required to estimate the number of objects, type of the object, position, direction and speed of the object, etc .

4.	The SEAL LM server updates the produced spatial map with the VAL UEs information obtained in step 2, that is, information (example, position, location) of the identified VAL UEs in the spatial map and tag them with the VAL Services(s) they are related to as in the Produce spatial map request message from the VAL server or SEAL LM client.

5.	The SEAL LM server sends the VAL UE augmented spatial map response message including the information related to the created spatial map. The information may include, map identity, VAL UEs position in spatial map, VAL UEs tagged with the related VAL service(s), and stationery / moving objects with attributes related to them.

#### 7.x.3.2	Update spatial map augmented with VAL UEs

Figure 7.x.3.1-1 illustrates the procedure to updating a VAL UE augmented spatial map. The request is from VAL server or an SEAL LM client. The service is provided by SEAL LM server.

Pre-condition:

- spatial map is created and VAL server (or SEAL LM client) is aware of the spatial map identity

- the VAL server (or SEAL LM client) is authorized to perform the update of the spatial map with VAL UEs information.

Figure 7.x.3.2-1: Update spatial map with VAL Ues

1.	The VAL server (or SEAL LM client) sends request to update the VAL UE augmented spatial map to the SEAL LM server. The request includes map identity (identity of the spatial map) that needs to be updated, an indication to identify VAL UEs in the spatial map, VAL service(s) list and may include VAL UEs of interest. The indication informs the SEAL LM server to update the spatial map with the information (example, position, location) related to the VAL UEs that are available in the area/location related to the spatial map to be updated. VAL Service(s) information indicates to the SEAL LM server to update the VAL UEs information related to these VAL service(s) in the updated spatial map.

2.	The SEAL LM server will authenticate and authorize the VAL server or SEAL LM client. If the VAL server or SEAL LM client is not authorized then the SEAL LM server sends failure response. If Authentication and authorization is success, the SEAL LM server determines the area/location information from the target spatial map to be updated using map identity information in the request message and fetches the VAL UEs available in the area/location related to the spatial map, from SEAL LM server using clause 9.3.10 of 3GPP TS 23.434 [8] and/or from NEF as specified in 3GPP TS 23.502 [9]. The SEAL LM server further fetches the VAL service(s) information related to the identified VAL UEs in the area of interest, from the SEAL CM server using clause 11.3.5 of 3GPP TS 23.434 [8]. The SEAL LM server identifies the VAL UEs information related to the VAL service(s) in the request message from VAL server or SEAL LM client.

3.	The SEAL LM server updates the spatial map with the VAL UEs information obtained in step 2, that is, information (example, position, location) of the identified VAL UEs and tag identify them with the VAL Services(s) they are related to as in the update spatial map request message from the VAL server or SEAL LM client.

4.	The SEAL LM server sends the VAL UE augmented spatial map response message including the information related to the updated spatial map. The information may include, map identity, VAL UEs position in spatial map, VAL UEs tags with related VAL service(s), stationery or moving objects with attributes related to them.

### 7.x.4	Corresponding APIs

#### 7.x.4.1	Overview

This clause provides summary of the corresponding APIs for the solution #x.

- Produce spatial map with VAL UEs API (Request / Response, API Provider: SEAL LM server, Known consumers: VAL server/ SEAL LM client, corresponding to step 1 to 5 of clause 7.x.3.1)

- Update spatial map with VAL UEs API (Request / Response, API Provider: SEAL LM server, Known consumers: VAL server/ SEAL LM client, corresponding to step 1 to 4 of clause 7.x.3.2)

#### 7.x.4.2	Information Flows

Table 7.x.4.2-1 shows the information in the request sent by VAL Server/ SEAL LM client to SEAL LM server to produce VAL UE augmented spatial map with VAL UEs

Table 7.x.4.2-1: Produce VAL UE augmented spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL LM client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of Interest | M | Location area of interest related to which the spatial map is to be created. |
| Identify VAL UEs | M | Indication to identify the VAL UEs in the spatial map. |
| > VAL UE of interest | O | The VAL UEs of interest to be identified |
| VAL service(s) | M | The VAL service information |

Table 7.x.4.2-2 shows the VAL UE augmented spatial map response message from SEAL LM server to VAL Server/ SEAL LM client with information about created spatial map

Table 7.x.4.2-2: VAL UE augmented spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Failure cause | O | Indicates reason for the failure |
| Spatial map information | M | Information related to the created spatial map (for successful case) |
| > map identity | M | Identity information of the created spatial map |
| > VAL UEs Information | O | Information (example, position, location) of the identified VAL UEs in the spatial map and tagged with the VAL Services(s) they are related to as in the request message from the VAL server |
| >> VAL UE details | O | Identity and/or other details related to the VAL UE |
| >> Location | M | Location information of the VAL UE in the spatial map |

Table 7.x.4.2-3 shows the request sent by VAL Server/ SEAL LM client to SEAL LM server to udpate VAL UE augmented spatial map with VAL UEs

Table 7.x.4.2-3: Update VAL UE augmented spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., SEAL LM client, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Map identity | M | Identity information of the spatial map to be updated. |
| Identify VAL UEs | O | Indication to identify the VAL UEs in the spatial map. |
| > VAL UE of interest | O | The VAL UEs of interest to be identified |
| VAL service(s) | O | The VAL service information |

Table 7.x.4.2-4 shows the update VAL UE augmented spatial map response message from SEAL LM server to VAL Server/ SEAL LM client with information about updated spatial map

Table 7.x.4.2-4: Update VAL UE augmented spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Failure cause | O | Indicates reason for the failure |
| Spatial map information | M | Information related to the updated spatial map (for successful case) |
| > map identity | M | Identity information of the updated spatial map |
| > VAL UEs Information | O | Information (example, position, location) of the identified VAL UEs in the spatial map and tagged with the VAL Services(s) they are related to as in the request message from the VAL server / SEAL LM client. |
| > VAL UE details | O | Identity related to the VAL UE |
| > Location | M | Location information of the VAL UE in the spatial map |
| > VAL Service(s) | O | VAL Services(s) related to VAL UE. |

### 7.x.5	Solution evaluation

This solution addresses KI#4 by providing the VAL UE augmented spatial map create/update services. The procedures are based on the SEAL-LM architecture and functional model. The solution enhances the existing SEAL LM server by adding new functionalities to manage spatial maps augmented with VAL UE information. This solution enhances the spatial map management with enriched information related to VAL UEs available in the spatial map with the related the VAL service(s) information. This solution is complementary to Solution #8 specified in clause 7.8 and can be combined in normative phase. A consistent way of obtaining processed sensing information for objects by SEAL LMS including objects behaving as VAL UEs is required and will be determined in normative phase.

* * * End of Changes * * * *