3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240372

10th – 18th July 2024	(revision of S6-242049,S6a240188, S6a240302)

Source:	InterDigital Inc.

Title:	Pseudo-CR on New solution for enabling spatial mapping service exposure and discovery

Spec:	3GPP TR 23.700-21 v1.0.0

Agenda item:	8.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a solution to KI#4 for spatial mapping, and addresses the following open issues:

- How to produce or modify a spatial map by the consumer (e.g. UE, VAL server)?

- How to expose a spatial map to authorized third parties?

2. Reason for Change

This paper provides a solution that complements the spatial mapping service offered by a MNO by enabling a 3rd party to offer a spatial mapping service for spatial locations where the MNO cannot.

TR 23.700-21 solution #8 proposes a solution for managing spatial maps within the mobile metaverse application enablement service of a MNO by providing functionalities for managing (i.e. produce, update, get, subscribe) spatial maps.

This paper proposes a complementary solution when the spatial maps are managed outside of the mobile metaverse application enablement service of the MNO. This enables a spatial map owner to provide spatial mapping services (e.g. spatial mapping and spatial localization services) at the VAL layer as a complement to the MNO spatial maps. For example, a factory owner, a mall owner, or a private property owner can offer (e.g., expose) the spatial mapping capability through the MNO service.

In this scenario, the spatial mapping service offered by the MNO enables the discovery of spatial mapping services for spatial locations where the MNO does not or cannot.

This solution proposes spatial mapping service exposure and discovery procedures to allow authorized consumers to discover and use spatial mapping services based on consumer application requirements and capabilitites.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.0.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 | KI #9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |  |  |
| Sol #4 | X |  |  |  |  |  |  |  |  |
| Sol #5 |  |  | X |  |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |  |
| Sol #8 |  |  |  | X |  |  |  |  |  |
| Sol #9 |  |  | X |  | X |  |  |  |  |
| Sol #10 |  |  | X |  | X |  |  |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |  |
| Sol #13 |  |  |  |  |  |  | X |  |  |
| Sol #x |  |  |  | X |  |  |  |  |  |

* * * Next Change * * * *

## 7.x	Solution #x: Spatial mapping service exposure and discovery

### 7.x.1	Solution description

#### 7.x.1.1	General

This solution is for KI#4 and addresses the open issue of how to produce, modify and expose a spatial map.

This solution is presented based on the Mobile Metaverse enablement layer architecture described in clause 6.1 (e.g., option #1); the solution is also applicable if introducing a new SEAL service reusing an existing SEAL service as described in clause 6.3 (e.g., option #3).

In this solution, a VAL server may provide a spatial mapping service; the VAL server may expose the spatial mapping service via MNO APIs for the purposes of complementing the MNO spatial mapping coverage when the MNO does not or cannot produce or maintain a spatial map for a location. For example, a company may provide and maintain spatial maps for their factory and may expose a spatial mapping service to users within the factory via the MNO APIs. A user within the factory may discover and use the spatial mapping service for the factory via the MNO APIs.

In this solution, the mobile metaverse service provides capabilities for exposing a spatial mapping service and for discovering spatial mapping services. Spatial mapping services may be discovered based on consumer application requirements and capabilitites, and mapping service requirements and capabilities.

NOTE: The MMES can integrate CAPIF core function and reuse 23.222 as implementation choice without standard impact.

#### 7.x.1.2	Exposing a spatial mapping service

This procedure allows an authorized VAL server to expose a spatial mapping service to authorized consumers.

Figure 7.x.1.2-1: Exposing a spatial mapping service

1.	A VAL server providing a spatial mapping service sends a spatial mapping service exposure request to the MMES. The request includes a VAL server identifier, spatial mapping service type, spatial mapping service address (e.g., FQDN, URI, IP, endpoint), spatial mapping capabilities (e.g., spatial mapping and/or spatial localization capabilities), and spatial map information (e.g., identifier, coverage area, civic address, latest update time, etc.).

2.	Upon receiving the request, the MMES authorizes the exposure request and stores the spatial mapping service information.

3.	The MMES sends a spatial mapping service exposure response to the spatial mapping service indicating success or failure to expose the spatial mapping service. In the failure case, a reason may be provided.

#### 7.x.1.3	Discovering a spatial mapping service

This procedure allows a consumer application on a UE to discover a spatial mapping service according to provided requirements.

Pre-conditions:

1)	A spatial mapping service is exposed via the MMES.

Figure 7.x.1.3-1: Discovering a spatial mapping service

1.	A VAL client requires spatial mapping services for a specific location.

2.	The VAL client requests a spatial mapping service discovery from the MMEC to discover available spatial mapping services. The request includes discovery filters based on the VAL client requirements (e.g., requested area of interest, etc.).

3.	The MMEC sends a spatial mapping service discovery request to the MMES. The request may include information received from the VAL client and additional UE information (e.g., UE identifier, UE location, UE pose, etc.).

4.	Upon receiving the request, the MMES authorizes the discovery request and if the request is authorized determines a spatial mapping service based on the information provided in the request (e.g., discovery filters, identifiers, etc.).

NOTE 1:	The MMES considers the spatial mapping service offered by the MNO and the spatial mapping service(s) exposed via the MNO to determine a spatial mapping service that meets the UE requirements.

NOTE 2:	The spatial mapping service discovery request  may be sent following a spatial map discovery request that has failed with the MNO spatial mapping service (e.g., MNO indicated no coverage for the area of interest).

The MMES may also obtain location information from the 5GC about the UE by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [7] and TS 23.502 [9], or by invoking the SEAL Location Management APIs as described in TS 23.434 [8].

5. 	The MMES sends a spatial mapping service discovery response to the MMEC. The response may include one or more determined spatial mapping service(s) with the associated information (i.e., address, identifier, capabilities, and other information provided at spatial mapping services exposure in clause 7.x.1.2).

6.	The MMEC informs the VAL client and includes information received from the MMES.

NOTE 3 :	If the information provided to the VAL client includes spatial mapping service(s), the VAL client can obtain spatial map(s) from that service for the area of interest.

#### 7.x.1.4	Service exposure via CAPIF

The mobile metaverse services can support exposing the spatial mapping service offered by the VAL server via CAPIF as specified in 3GPP TS 23.222 [14] by deploying CAPIF core function within the MMES to support publish and discovery of the VAL server service APIs.

NOTE 1:	The MMES supporting the exposure of the spatial mapping service offered by the VAL server using CAPIF is not explicitly depicted in clause 7.x.1.2.

NOTE 2:	Determining if the MNO or if a 3rd party spatial mapping server has a  requested spatial coverage is performed by the MMES.

### 7.x.2	Architecture Impacts

This solution can be realized according to architectures described in clause 6.1 or clause 6.3 or using new SEAL Server.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	Overview

This clause provides a summary of the corresponding APIs for solution #x.

-	Spatial mapping service exposure API (request / response model; API provider: MMES; known consumers: VAL server; corresponding to step 1 and 3 of clause 7.x.1.2).

-	Spatial mapping discovery API (request / response model; API provider: MMEC; known consumer: VAL client; corresponding to step 2 and 6 of clause 7.x.1.3).

-	Spatial mapping discovery API (request / response model; API provider: MMES; known consumer: MMEC; corresponding to step 3 and 5 of clause 7.x.1.3).

NOTE:	The spatial mapping discovery API provided by the MMEC and MMES may be enhanced with the subscribe / notify model in the normative phase.

#### 7.x.3.2	Information flows

Table 7.x.3.2-1 shows the request sent by a VAL server to a MMES for exposing a spatial mapping service.

Table 7.x.3.2-1: Spatial mapping service exposure request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Mapping Service list | M | The list of Spatial Mapping Services to be exposed. Each element includes the information described below. |
| > Identifier | M | The identifier of a spatial mapping service |
| > Type | O | The type of spatial mapping service |
| > address | M | The address to access the spatial mapping service (e.g., FQDN, URI, IP, endpoint) |
| > capabilities | O | The capabilities offered by a spatial mapping service (e.g., spatial mapping, spatial localization capabilities, or both) |
| > Spatial Map information | M | The information about Spatial Map(s) (SM) available at the spatial mapping service (e.g., one or more spatial map may be provided by the service). Each element includes the information described below. |
| >> SM identifier | M | The identifier of a spatial map. |
| >> SM area coverage | M | The spatial area coverage of a spatial map. |
| >> SM location | O | The location of a spatial map (e.g., civic address, etc.). |
| >> SM update time | O | The time when the spatial map was last updated. |
| >> SM update frequency | O | The frequency at which a spatial map is updated. |

Table 7.x.3.2-2 shows the response sent by a MMES to a VAL server when exposing a spatial mapping service.

Table 7.x.3.2-2: Spatial mapping service exposure response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Failure cause | O | Indicates reason for the failure |

Table 7.x.3.2-3 shows the request sent by a MMEC to a MMES for discovering a spatial mapping service.

Table 7.x.3.2-3: Spatial mapping service discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial mapping service discovery filters | M | Set of characteristics to determine spatial mapping service(s). The discovery filters are aligned with the information described in “list of Spatial Mapping Services” described in table 7.x.3.2-1. |

Table 7.x.3.2-4 shows the response sent by a MMES to a MMEC when discovering a spatial mapping service.

Table 7.x.3.2-4: Spatial mapping service discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Spatial mapping services list | O | The list of spatial mapping services determined based on the discovery filters. Each element includes information as described in “list of Spatial Mapping Services” described in table 7.x.3.2-1. |
| Failure cause | O | Indicates reason for the failure |

### 7.x.4	Solution evaluation

This solution is for KI#4 and addresses the open issue of how to produce, modify and expose a spatial map.

This solution can be realized according to architectures described in clause 6.1 or clause 6.3.

This solution proposes that the mobile metaverse service provides capabilities for exposing a spatial mapping service provided by a VAL server for complementing the MNO spatial coverage and proposes that the mobile metaverse service provides capabilities for discovering spatial mapping services offered by the MNO and/or VAL server(s) according to a consumer application requirements.

* * * End of Changes * * * *