3GPP TSG-SA WG6 Meeting #62-Ad Hoc-e	S6a240376

10th – 18th July 2024	(revision of S6a240267, S6a240075)

Source:	China Mobile

Title:	New solution for KI#6: XR EAS selection based on N6 delay measurement

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	zhengshaowen@chinamobile.com

1. Introduction

This paper provides a new solution for KI #6: XR EAS selection based on N6 delay measurement.

2. Reason for Change

The target of KI #6 is to reduce service latency for XR applications. N6 delay measurement has already been studied and concluded in KI#2, TR23.700-49. This paper proposes a new solution for KI #6 to take advantage of the N6 delay measurement mechanism. In this solution, the EES selects the EAS based on the N6 delay of candidate EAS(s) and UPF(s), which can be obtained from 5GC.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change (all new text) * * * *

7.X	Solution #X: XR EAS selection based on N6 delay measurement

7.X.1	Architecture Impacts

This solution is based on architecture of EDGEAPP as described in 3GPP TS 23.558 [11].

7.X.2	Solution description

7.X.2.1	General

In EAS discovery procedure, the EES sends N6 delay measurement request to 5GC and obtains the N6 delay of each pair of discovered XR EAS and UPF for the DNAI of the XR EAS if SA2 can expose N6 delay information to EES.

Editor’s note: whether the N6 delay request indicator is needed for EAS discovery or selection is FFS.

Editor’s note: whether and how the N6 delay measurement could utilized in EAS registration procedure is FFS.

7.X.2.2	Procedure

EAS

UE

ECS

EES

5GC

2. EAS discovery request

6. N6 delay measurement response

1. Service Provisioning Procedure

3. EES discovers the EAS(s)

EAS

4. N6 delay measurement request

5. N6 delay measurement

7. EAS discovery response

Pre-condition:

1.	The EEC obtains the ECS address information.

Procedure:

1.	Service provisioning procedure is performed as described in TS 23.558 clause 8.3.3.2.2.

2.	The EEC send the EAS discovery request message to the EES as described in TS 23.558 clause 8.5.2.2.

3.	Upon receiving the request message from EEC, the EES discovers the candidate XR EAS(s) as described in TS 23.558 clause 8.5.2.2.

4.	The EES sends N6 delay measurement request to the 5GC, including: an indication of considering N6 delay, IP address(es) of candidate EAS(s), DNAI(s) of the candidate EAS(s), the measurement protocol(s) and optionally the protocol-specific configuration parameters.

Editor's note:	The exact information element and service of N6 delay measurement is FFS and depend on SA2 working progress.

5.	The 5GC performs N6 delay measurement for each pair of candidate XR EAS and UPF associated with requested DNAI(s).

6.	The 5GC sends N6 delay measurement response to the EES, including the N6 delay(s) related to each candidate XR EAS.

7.	The EES sends the EAS discovery response message to the EEC as described in TS 23.558 clause 8.5.2.2.

7.X.3	Solution evaluation

This solution addresses KI#6 to support XR EAS selection based on the measurement of N6 delay of XR EAS(s) and UPF(s) performed by 5GC.

Editor's note: Whether the solution can be concluded and whether the solution can be in normative phase depends on whether SA2 expose N6 delay information to AF.

* * * End of Change(s) * * * *