3GPP TSG-SA WG6 Meeting #43-e	S6-21xxxx

e-meeting, 24th – 02th June 2021	(revision of S6-21xxxx)

Source:	at&t

Title:	Pseudo-CR on Message stored and forward

Spec:	3GPP TS 23.554

Agenda item:	7.9

Document for:	Approval

Contact:	Jerry Shih, jerry.shih@att.com

1. Introduction

This pCR proposes the message stored and forward procedure.

2. Reason for Change

The stage 1 requirements in TS 22.262 has the following stored and forward requirment:

[R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available.

A procedure is needed to show how this is supported.

3. Conclusions

The current message delivery procedures in 8.2 include an optional step 4 that the MSGin5G Server might store a message for offline UEs without the details.

The proposal is

- Make the exisit 8.2 procedures for only normal delivery, i.e. the UE is always available for message delivery by removing the optional step 4 in Figure 8.2.2-1, 8.2.2-2 and 8.2.2-3.

- Add a new procedure to handle the MSGin5G Server stored and forward when the UE is not available at delivery.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554.

* * * First Change * * * *

## 8.2	Message delivery procedures into/from MSGin5G Server

### 8.2.1	General

All MSGin5G message traffic is routed via the MSGin5G Server. The present subclause specifies all MSGin5G message traffic towards and from the MSGSin5G Server.

Editor's note:	Alignment of this solution with the architecture and the conclusions from the study is FFS.

### 8.2.2	MSGin5G inbound messages into the MSGin5G Server

Figure 8.2.2-1 shows the procedure for a MSGin5G UE that initiates a MSGin5G message request.

- Figure 8.2.2-1: New MSGin5G Message Request from UE

Figure 8.2.2-2 shows the procedure for an Application Server that initiates an API request for sending a MSGin5G Message to UE.

Editor's note:	The APIs provided by MSGin5G Server to App Server is to be specified in another section of the TS.

- Figure 8.2.2-2: Application Server initiates a request for sending a MSGin5G message

Figure 8.2.2-3 shows the procedure for a Legacy 3GPP Message Gateway or a non-3GPP Message Gateway that sends a new MSGin5G message request to the MSGin5G Server on behalf of a Legacy 3GPP UE or Non-3GPP UE.

- Figure 8.2.2-3: New MSGin5G Message Request sending from Message Gateway

The following procedure applies to the above figures 8.2.2-1, 8.2.2-2 and 8.2.2-3 with the exception that step 1 only applies to figure 8.2.2-1.

1.	The Application Client-1 in the UE sends a request to the MSGin5G Client for invoking the MSGin5G Client to send a new MSGin5G Message to a recipient or to multiple recipients.

Editor's note:	Whether the APIs provided by the MSGin5G Client to the Application Client is to be specified in another section of the TS is FFS.

2.	As shown in figure 8.2.2-1 or 8.2.2-3, the MSGin5G Client or Message Gateway sends the MSGin5G Message Request to the MSGin5G Server and includes the IEs as listed in table 8.2.2-1 in the request; or as shown in figure 8.2.2-2, the Application Server sends an API request to the MSGin5G Server for sending a MSGin5G message, the API request includes the IEs as listed in table 8.2.2-1.

- Table 8.2.2-1: Request to MSGin5G Server for sending MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Recipient MSGin5G Service ID (see NOTE) | O | The service identity of the receiving MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. This IE is mandatory for point-to-point messaging, application-to-point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. |
| Recipient Group Service ID (see NOTE) | O | The service identifier of the target MSGin5G Group. This IE is mandatory for a Group Message and is not present in other message scenarios. |
| Recipient Broadcast Area ID (see NOTE) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic (see NOTE) | O | Indicates which messaging topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application IDs IE may be included when the message is sent to an Application Server or to an Application Client. MSGin5G Server is unaware of the content. |
| Message ID | M | Unique identifier of this message. If the request is a Delivery Report (as shown in step 6) the Message ID IE is copied from the MSGin5G message that is being acknowledged. |
| Security Credentials | O | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |
| Delivery Status Required | O | Indicates if delivery acknowledgement from the recipient is requested. This IE is not present if the MSGin5G message request is a Delivery Report (as shown in step 6). |
| Failure Cause | O | If the request is a Delivery Report (as shown in step 6) the Failure Cause indicates the failure reason, if applicable. |
| Payload | O | Payload of the message. MSGin5G Server/Client is unaware of the content. If the Request is a Delivery Report (as shown in step 6) the Payload IE is not included. If the request is sent from MSGin5G Client or Message Gateway to the MSGin5G server, the maximum size of this IE is a configurable value that shall not exceed 2048 octets. |
| Priority Type | O | Application priority level requested for this message. If the Request is a Delivery Report (as shown in step 6) the Priority Type IE is copied from the message that is being acknowledged. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Segmentation Set Identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages |
| Last Segment Flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| NOTE: One of these IEs need to be included to represent the type of message request. |  |  |

3.	The MSGin5G Server determines if the recipients are available to receive the message or if the message needs to be stored for deferred delivery.

4.

4.	The MSGin5G Server may return a response to the originating entity and includes the IEs as listed in table 8.1.2-2 in the response.

- Table 8.2.2-2: Information Elements in Response

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G service ID | M | The identity of the MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the identity of the Application Server that initiated the previous Request. |
| Message ID | M | Identifier of the initiating Request. |
| Delivery Status | O | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |
| Failure Cause | O | The reason for failure |

5. 	When the MSGin5G Server receives a Message delivery report from the recipient UE it is forwarded to the message originating party.

* * * Next Change * * * *

- 8.X	Message Stored and Forward

Figure 8.X-1 illustrates the message stored and forward procedure. The MSGin5G Server storses a message temporarily when it cannot deliver to the targeted UE due to its unavailability. The MSGin5G Server delivers the stored message as soon as the targeted UE is available again.

Pre-conditions

-	The MSGin5G Server supports stored and forward.

-	The terminating MSGin5G UE is offline.

Figure 8.X-1: Message Stored and Forward

1.	An originating service endpoint (MSGin5G UE, Application Server or Message Gateway) sends a message to the terminating UE. As per 8.2.2 Message delivery procedure into MSGin5G Server. The terminating UE is offline.

2.	As the terminating UE is offline, the MSGin5G Server stores the message and waits for the terminating UE becomes available to deliver.

3.	The terminating UE is online.

4.	the terminating UE registers with the MSGin5G Server to access the MSGin5G service as per 8.Y UE Registration procedure. After the successful registration the MSGin5G Server marks the terminating UE as available.

5.	the MSGin5G Server delivers the stored message to the terminating UE as per 8.2.3 MSGin5G outbound messages from the MSGin5G Server.