## 5.16	Control and management by Mission Critical Organizations

### 5.16.1 	Overview

Clause 5.16 contains general requirements for management of the MCX Service by Mission Critical Organizations sharing the same MCX Service system, and more specific requirements pertaining to management controls and operational visibility, and to management of security services.

### 5.16.2	General requirements

[R-5.16.2-001] The MCX Service shall be able to support multiple Mission Critical Organizations, each with their own MCX Users and MCX Service Groups, on the same MCX Service system.

[R-5.16.2-002] The MCX Service shall provide a mechanism by which Mission Critical Organizations designate and manage (i.e., add, delete, change authorizations, etc.) MCX Service Administrators with authority to manage users, groups, other MCX Service Administrators, security controls, and other mission affecting parameters (e.g., authorizations and priorities) of the MCX Service.

[R-5.16.2-003] The MCX Service shall protect the operational privacy of Mission Critical Organizations by providing effective separation between the administrative and security management (e.g., key) parameters of those organizations except as authorized by the Mission Critical Organizations involved.

[R-5.16.2-004] The MCX Service shall protect the administrative and security management parameters of Mission Critical Organizations from viewing and manipulation by individuals (including those within and outside of the mission critical organization) not explicitly authorized by the Mission Critical Organization.

[R-5.16.2-005] The MCX Service shall provide a mechanism by which Mission Critical Organizations may share subsets of their administrative and security parameters with other Mission Critical Organizations.

NOTE:	The purposes of these requirements protect the operational security of organizations while still allowing for interworking of MCX UE and Users under the control of the Mission Critical Organizations.

### 5.16.3	Operational visibility for Mission Critical Organizations

[R-5.16.3-001] The MCX Service shall provide a means by which an MCX Service Administrator associated with a Mission Critical Organization has visibility into the operational status of the service (e.g., during a natural disaster).

### 5.16.4	Sharing of administrative configuration between Mission Critical Organizations

Mission Critical Organizations can deploy their own 3GPP standardized mobile, local, countrywide or national broadband mission critical communications systems. In order to grant visiting MCX Service Users access to a Partner MCX Service System and its services, a mechanism is required, which allows the exchange of administrative configuration between connected MCX Service Systems.

NOTE:	Administrative configuration is used for adding MCX Service Users into a Partner MCX Service System’s user and group databases, modifying security controls and other mission affecting parameters, and for enabling or disabling certain services for specific users. Relevant information can be exchanged, for example, prior to the visiting MCX Service Users arriving at a Partner MCX Service System or dynamically during events.

[R-5.16.4-001] An MCX Service shall provide a mechanism to allow an authorised MCX User to request MCX User configuration changes in one or more Partner MCX Service Systems (e.g., priorities, authorizations).

[R-5.16.4-002] An MCX Service shall provide a mechanism to allow an authorised MCX User to evaluate and respond to requests for configuration changes from Partner MCX Service Systems.

[R-5.16.4-003] An MCX Service shall provide a mechanism to allow an authorised MCX User to configure automatic responses to categories of requests for configuration changes from Partner MCX Service Systems (e.g., particular users, or groups).

[R-5.16.4-004] An MCX Service shall provide a mechanism to allow an authorised MCX User to request configuration information (e.g., users, groups, security level) from Partner MCX Service Systems.

[R-5.16.4-005] An MCX Service shall provide a mechanism to allow an authorised MCX User to send configuration information to Partner MCX Service Systems.

[R-5.16.4-006] An MCX Service shall provide a mechanism to allow an authorised MCX User to exchange operationally relevant information for specific MCX Users, e.g., MCX User capability information, with Partner MCX Service Systems.

NOTE:	Capability information could include, but not limited to, information such as chemical, biohazard, medical or equipment handling capabilities for a specific MCX User.

[R-5.16.4-007] An MCX Service System shall provide a mechanism to allow secure exchange of administrative and security related information with interconnected MCX Service systems without compromising the integrity and security of either system.

[R-5.16.4-008] An MCX Service System shall provide a mechanism to allow exchange of administrative and security related information with interconnected MCX Service systems without exposing the internal network topology of either system.

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSG SA# | SA Doc. | SA1 Doc | Spec | CR | Rev | Rel | Cat | Subject/Comment | Old | New | WI |
| SP-73 | SP-160543 | S1-162406 | 22.280 | 0001 | 1 | Rel-14 | F | Corrections to referenced sub-clauses in Annex B and Annex C | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162407 | 22.280 | 0003 | 1 | Rel-14 | D | Add an Informative Annex table in MCCoRe TS 22.280 with a List of Variables used | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162403 | 22.280 | 0004 | 1 | Rel-14 | F | Add definitions for participant and participant type to MCCoRe TS 22.280 sub-clause 3.1. | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162019 | 22.280 | 0005 |  | Rel-14 | D | Editorial changes to MCCoRe TS 22.280 Informative Annex D sub-clause D.1 | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162020 | 22.280 | 0006 |  | Rel-14 | F | In MCCoRe TS 22.280 provide changes to References 1, 2, and 3 in clause 2. | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162408 | 22.280 | 0007 | 1 | Rel-14 | D | Provide editorial changes to several sub-clauses in MCCoRe TS 22.280 for consistency and to improve understanding and readability. MCC Note: 25th change (removing Annexes a, B and C) not implemented since contradicting other CRs (i.e. CR#0001r1, 0008r2, 0010r1, 0012r1 and 0021r1) | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162497 | 22.280 | 0008 | 2 | Rel-14 | C | In MCCoRe TS 22.280 provide editorial changes to the service description in sub-clause 6.19.1 Call rejection and queuing. | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162036 | 22.280 | 0009 |  | Rel-14 | B | In MCCoRe TS 22.280 Add a requirement in sub-clause 8.4.2 Requirements for a single group with multiple MCX Services. | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162266 | 22.280 | 0010 | 1 | Rel-14 | B | In MCCoRe TS 22.280 Add sub-clause 7.4 requirements to normative Annex C MCCoRe Requirements for MCData. | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162410 | 22.280 | 0012 | 1 | Rel-14 | F | Insertion of new clause 5.5.2 with requirement [R-5.1.8-002] from clause 5.1.8 | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162411 | 22.280 | 0013 | 1 | Rel-14 | F | Corrections to Clause 4.2 | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162124 | 22.280 | 0014 |  | Rel-14 | F | Correction of inconsistency between header 6.7 and overview text 6.7.1 | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162125 | 22.280 | 0015 |  | Rel-14 | F | Correction to clause 5.1.1 requirement [R-5.1.1-005] | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162412 | 22.280 | 0016 | 1 | Rel-14 | F | Clarification of text in clause 5.4.1 | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162127 | 22.280 | 0017 |  | Rel-14 | F | Clarification of text in clause 5.6.1 | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162413 | 22.280 | 0018 | 1 | Rel-14 | F | Moving of informative text within requirements to NOTEs | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162414 | 22.280 | 0019 | 1 | Rel-14 | F | Correction [R-7.12.3-003] | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162415 | 22.280 | 0020 | 1 | Rel-14 | F | Correction of the entity that affiliates | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162416 | 22.280 | 0021 | 1 | Rel-14 | F | Single requirement label [R-5.6.2.4.1-004] with two requirements | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162132 | 22.280 | 0022 |  | Rel-14 | F | Corrections to definitions | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-73 | SP-160543 | S1-162401 | 22.280 | 0023 | 1 | Rel-14 | F | Corrections to Group Regroup triggered by the need to support of Audio Cut-in capability | 14.0.0 | 14.1.0 | MCImp-MCCoRe |
| SP-74 | SP-160891 | S1-163008 | 22.280 | 0024 |  | Rel-14 | F | Corrections to referenced sub-clauses in Annex C | 14.1.0 | 14.2.0 | MCImp-MCCoRe |
| SP-74 | SP-160891 | S1-163009 | 22.280 | 0025 |  | Rel-14 | D | Editorial changes to 3 sub-clauses | 14.1.0 | 14.2.0 | MCImp-MCCoRe |
| SP-74 | SP-160891 | S1-163010 | 22.280 | 0026 |  | Rel-14 | D | Provide editorial changes to the service description in sub-clause 6.19.1 Communication rejection and queuing for readability. | 14.1.0 | 14.2.0 | MCImp-MCCoRe |
| SP-75 | SP-170150 | S1-170191 | 22.280 | 0027 | 1 | Rel-14 | F | Corrections to requirement references in the applicability annexes | 14.2.0 | 14.3.0 | MCImp-MCCoRe |
| SP-75 | SP-170150 | S1-171025 | 22.280 | 0030 |  | Rel-14 | F | Correct inconsistent use of the term 'one-to-many' | 14.2.0 | 14.3.0 | MCImp-MCCoRe |
| SP-76 | SP-170445 | S1-172428 | 22.280 | 0031 | 7 | Rel-15 | B | FRMCS Role management and presence | 14.3.0 | 15.0.0 | MONASTERY |
| SP-77 | SP-170696 | S1-173361 | 22.280 | 0042 | 1 | Rel-15 | F | Correction to requirement on usage of ulisted functional alias | 15.0.0 | 15.1.0 | MONASTERY |
| SP-77 | SP-170696 | S1-173362 | 22.280 | 0043 | 1 | Rel-15 | C | update requirement for functional alias | 15.0.0 | 15.1.0 | MONASTERY |
| SP-77 | SP-170696 | S1-173510 | 22.280 | 0045 | 2 | Rel-15 | C | update requirement for functional alias | 15.0.0 | 15.1.0 | MONASTERY |
| SP-78 | SP-170985 | S1-174042 | 22.280 | 0046 |  | Rel-15 | F | Updates to the wording in the specification to clarify that the requirements are applicable from LTE onwards | 15.1.0 | 15.2.0 | MCOver |
| SP-78 | SP-170997 | S1-174419 | 22.280 | 0047 | 2 | Rel-16 | B | Priority controlled by MCX User for receiving from Multiple MCX service Communication | 15.1.0 | 16.0.0 | MONASTERY2 |
| SP-78 | SP-170997 | S1-174425 | 22.280 | 0049 | 3 | Rel-16 | B | Changes relating to the User regrouping section of 22.280 | 15.1.0 | 16.0.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180037 | 22.280 | 0053 |  | Rel-16 | B | Enhanced control of User Regroup and Private Communication | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180436 | 22.280 | 0057 | 1 | Rel-16 | F | Clarification of definition of Mission Critical Service in 22.280 | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180439 | 22.280 | 0060 | 1 | Rel-16 | B | Changes related to membership/affiliation section of 22.280 | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180564 | 22.280 | 0054 | 3 | Rel-16 | B | Adding more functionality to interworking between FRMCS and GSM-R | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180565 | 22.280 | 0055 | 5 | Rel-16 | C | Functional Alias enhancements(chapter 5.9a) | 16.0.0 | 16.1.0 | FS_FRMCS2 |
| SP-79 | SP-180134 | S1-180568 | 22.280 | 0056 | 2 | Rel-16 | B | Introduction of railway emergency alert functionality to 22.280 | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180583 | 22.280 | 0061 | 2 | Rel-16 | B | Changes related to administrative sections of 22.280 | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-79 | SP-180134 | S1-180612 | 22.280 | 0059 | 3 | Rel-16 | B | Query of location based on functional alias | 16.0.0 | 16.1.0 | MONASTERY2 |
| SP-80 | SP-180306 | S1-181704 | 22.280 | 0083 | 2 | Rel-16 | A | Generalisation of EPS into 3GPP system | 16.1.0 | 16.2.0 | MCOver |
| SP-80 | SP-180307 | S1-181385 | 22.280 | 0063 | 1 | Rel-16 | A | User profile presentation priority | 16.1.0 | 16.2.0 | MCImp-MCCoRe |
| SP-80 | SP-180310 | S1-181582 | 22.280 | 0071 | 2 | Rel-16 | C | chapter_5_10 MCX User log in to multiple devices | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181588 | 22.280 | 0074 | 2 | Rel-16 | F | Interworking between MCX Service system and GSM-R | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181589 | 22.280 | 0070 | 2 | Rel-16 | B | Addition of location based comunication depending on functional alias | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181590 | 22.280 | 0069 | 2 | Rel-16 | B | Adding authorization for private comunication based on functional alias | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181617 | 22.280 | 0082 | 2 | Rel-16 | B | Add a requirement on 5.11 Location | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181700 | 22.280 | 0073 | 4 | Rel-16 | B | Extension of functional alias chapter 5.9a | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181705 | 22.280 | 0064 | 4 | Rel-16 | B | Addition of location functionality for autonomous train control and operation | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-80 | SP-180310 | S1-181706 | 22.280 | 0065 | 4 | Rel-16 | F | Off-network UE functionality for virtual coupling | 16.1.0 | 16.2.0 | MONASTERY2 |
| SP-81 | SP-180757 | S1-182255 | 22.280 | 0089 |  | Rel-16 | F | Public emergency calls | 16.2.0 | 16.3.0 | MONASTERY2 |
| SP-81 | SP-180757 | S1-182561 | 22.280 | 0084 | 1 | Rel-16 | B | Dynamic Group Participation | 16.2.0 | 16.3.0 | MONASTERY2 |
| SP-81 | SP-180757 | S1-182580 | 22.280 | 0090 | 1 | Rel-16 | F | Clarification of requirement R-6.4.9-001 | 16.2.0 | 16.3.0 | MONASTERY2 |
| SP-81 | SP-180757 | S1-182581 | 22.280 | 0088 | 2 | Rel-16 | B | Selection of Application Layer Priorities by the MCX Service user | 16.2.0 | 16.3.0 | MONASTERY2 |
| SP-81 | SP-180761 | S1-182614 | 22.280 | 0085 | 1 | Rel-16 | B | Application of MCX Services to maritime usage | 16.2.0 | 16.3.0 | MARCOM |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-12 | SP-82 | SP-180999 | 0091 | 1 | B | Requirement to allow for users to automatically be informed when affiliation information changes in a group | 16.4.0 |
| 2018-12 | SP-82 | SP-181000 | 0094 | 1 | B | Move interworking of functional alias from MCPTT to MCCoRe TS, MCCoRe part | 16.4.0 |
| 2018-12 | SP-82 | SP-181000 | 0097 | 1 | B | Interworking among MCX Service systems | 16.4.0 |
| 2018-12 | SP-82 | SP-181000 | 0101 | 1 | F | Dynamic Group Participation | 16.4.0 |
| 2018-12 | SP-82 | SP-181000 | 0099 | 2 | B | Unique addressing of MCX Users when using functional alias | 16.4.0 |
| 2018-12 | SP-82 | SP-181000 | 0098 | 2 | B | Additional requirements for the use of a gateway UE | 16.4.0 |
| 2018-12 | SP-82 | SP-181000 | 0096 | 1 | C | Changes to recording and audit metadata | 16.4.0 |
| 2019-03 | SP-83 | SP-190079 | 0107 | 3 | F | Clarification for discreet listening | 16.5.0 |
| 2019-03 | SP-83 | SP-190084 | 0102 | 1 | C | Group regrouping additional requirements | 16.5.0 |
| 2019-06 | SA#84 | SP-190313 | 0110 | 1 | C | Add new capabilities for late communication entry | 17.0.0 |
| 2019-06 | SA#84 | SP-190313 | 0111 | 2 | C | Deactivation of an unlisted functional alias and deactivation status | 17.0.0 |
| 2019-06 | SA#84 | SP-190313 | 0108 | 2 | C | Enhance metadata to be logged for Railway post-incident analysis | 17.0.0 |
| 2019-09 | SA#85 | SP-190799 | 0133 | 1 | A | Additional broadcast regroup requirements | 17.1.0 |
| 2019-09 | SA#85 | SP-190799 | 0120 | 1 | B | Additional broadcast regroup requirements -> ignored (identical to the CR#0133r1 just above) | 17.1.0 |
| 2019-09 | SA#85 | SP-190799 | 0129 | 2 | A | Clarification of treatment of private call and private emergency calls to functional aliases activated by more than one MCX User | 17.1.0 |
| 2019-09 | SA#85 | SP-190813 | 0122 | 1 | C | Support the FRMCS use case for Arbitration when a user is registered to multiple UEs | 17.1.0 |
| 2019-09 | SA#85 | SP-190813 | 0127 | 1 | F | Extend Functional Alias binding to MCX Service Private Communication | 17.1.0 |
| 2019-09 | SA#85 | SP-190813 | 0131 | 1 | B | Support the location services related use cases while operating in off-network mode | 17.1.0 |
| 2019-09 | SA#85 | SP-190813 | 0121 | 2 | C | Activate Location Information report of a specific FA in a certain area | 17.1.0 |
| 2019-09 | SA#85 | SP-190813 | 0126 | 1 | F | Clarify routing location dependent communications as part of handling MCX Service Private Communication requests | 17.1.0 |
| 2019-09 | SA#85 | SP-190813 | 0132 | 2 | B | Support the recording of communication related use cases while operating in off-network mode | 17.1.0 |
| 2019-09 | SA#85 | SP-190804 | 0134 | 1 | F | New additional broadcast regroup requirements | 17.1.0 |
| 2019-12 | SA#86 | SP-191030 | 0138 | 2 | C | Clarification on interworking between FRMCS System and Public Networks | 17.2.0 |
| 2019-12 | SA#86 | SP-191030 | 0137 | 1 | D | Editorial correction to broadcast regroup requirements | 17.2.0 |
| 2020-07 | SA#88e | SP-200560 | 0140 | 3 | A | Clarification on the maximum number limit of simultaneous log ins of an MCX user | 17.3.0 |
| 2020-07 | SA#88e | SP-200560 | 0141 |  | F | Addition of a per-MCX user log-in limit | 17.3.0 |
| 2020-12 | SA#90e | SP-201029 | 143 | 1 | B | Enhancement to MCX User request queue operation | 17.4.0 |
| 2021-03 | SA#91e | SP-210199 | 144 | 1 | B | Gateway MCX UE identification | 17.5.0 |
| 2021-06 | SA#92e | SP-210499 | 0145 | 1 | C | Enhancement of MCX UE de-affiliation requirements | 17.6.0 |
| 2021-09 | SA#93e | SP-211036 | 0147 |  | F | Correction - Adding missing requirements into tables in the normative Annexes | 17.7.0 |
| 2021-09 | SA#93e | SP-211061 | 0146 | 1 | B | Sharing administrative configuration between Mission Critical Organizations (SACI_MCS) | 18.0.0 |
| 2021-09 | SA#93e | SP-211074 | 0148 | 2 | B | Ad hoc group call requirements (AHGC) | 18.0.0 |
| 2022-03 | SP#95e | SP-220082 | 0149 |  | C | Enhanced MCX Service Ad hoc Group Communication to support Railway needs | 18.1.0 |
| 2022-06 | SA#96 | SP-220434 | 152 | 1 | F | Call restriction based on subparts/elements of functional alias | 18.2.0 |
| 2022-06 | SA#96 | SP-220434 | 153 | 1 | F | Additional parameters for Location Information and clarification of Location terms | 18.2.0 |
| 2022-06 | SA#96 | SP-220434 | 154 | 1 | F | Enhanced MCX Service Ad hoc Group Communication to support Railway needs | 18.2.0 |