3GPP TSG-SA WG6 Meeting #51-e	S6-22xxxx

e-meeting,

Source:	Netherlands Police

Title:	pCR Solution on group configuration change

Spec:	3GPP TR 23.700-v38 v0.3.0

Agenda item:	9.2

Document for:	Approval

Contact:	KeesDotVerweijAtpolitie.nl

1. Introduction

This pCR adds a solution that is designed to solve key issue 4, change group configuration. It also addresses the second pre-condition in 3GPP TS 23.280, clause 10.2.7.2.

-	One or more MC service group members are defined in the partner MC system.

2. Reason for Change

Adding a solution for key issue 4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-38 v0.3.0.

* * * First Change * * * *

## 7.x	Solution #x: Group configuration change

### 7.x.1	Solution description

#### 7.x.1.1	General

This solution addresses the following aspects:

-	Key issue 4 - change group configuration

-	The second precondition in 3GPP TS 23.280, clause 10.2.7.2:

Precondition

-	One or more MC service group members are defined in the partner MC system.

#### 7.x.1.2	Procedure

##### 7.x.1.2.1	Group membership update by authorized user from a partner MC system

The procedure for an authorized MC service user in a partner MC system to request the primary MC system of the MC service group to add group members to an interconnection group is shown in figure 7.x.1.2.1-1.

Pre-conditions

-	MC system A and MC system B are interconnected

-	MC system A and MC system B have implemented an ACMS

-	MC system B is the primary MC system of an interconnection group

-	The authorized MC service user of MC system A (the partner MC system) wants to add a list of MC users to the interconnection group for which MC system B is the primary MC system.

Figure 7.x.1.2.1-1: Group membership update by authorized user from a partner MC system

1.	The ACM client in MC system A sends a group membership update request to the ACM server in MC system A, requesting to add a list of users from MC system A to an interconnection group for which MC system B is the primary MC system.

2.	The ACM server of system A checks whether the MC service user at ACM client A is authorized for the request.

3.	The ACM server of MC system A forwards the group membership update request to the ACM server of MC system B, the primary MC system of the interconnection group.

4a.	The ACM server of MC system B, the primary MC system of the interconnection group, stores the group membership update request.

4b.	The ACM server of MC system B, the primary MC system of the interconnection group, indicates that the group membership update request has been stored.

NOTE 1:	Steps 5, 6, 7 and 8 are optional and subject to configuration by the authorized user of MC system B.

NOTE 2:	There may be a considerable pause between step 4 and 5.

5.	The authorised MC service user of MC system B logs on to the ACM client of MC system B.

6.	The ACM server of MC system B forwards the stored group membership update request to the ACM client of MC system B.

7.	The authorized MC service user of MC system B (manually) screens the content of the group membership update request and decides to approve it.

Editor’s note: 	It is FFS how the group membership update that has been approved by the authorised user of MC system B is applied in MC system B.

8.	The ACM client of MC system B sends a group membership update response to the ACM server of MC system B.

9.	The ACM server of of MC system B forwards the group membership update response to the ACM server of MC system A.

10.	The ACM server of MC system A stores the group membership update response.

NOTE:	There may be a considerable pause between step 10 and 11.

11.	The authorised user of MC system A logs on to the ACM client of MC system A.

12.	The ACM server of of MC system A forwards the group membership update response to the ACM client of MC system A.

### 7.x.2	Solution evaluation

This solution only introduces a procedure for group membership updates by an authorized user from a partner MC system.

Editor’s note:	It is FFS if more procedures are needed to fulfil the group configuration requirements.

* * * End of Changes * * * *