3GPP TSG-SA WG6 Meeting #4	S6-150317

Fukuoka, Japan May 25-29 2015

Title:	MCPTT Group Affiliation Scenario Flows based on OMA XDM

Agenda Item:	6.4

Source:	Kodiak Networks

Contact:	Tom Hiller, thiller@kodiakptt.com,

Abstract: The contribution proposes MCPTT message flow scenarios for group affiliation. It is based on the IETF Common Policy approach outlined in contribution S6-15282.

#### 5.11.1.2 Procedures

Editor’s note: Describes the high-level operation, procedures and information flows for the solution.

##### 5.11.1.2.1  Group Affiliation

###### 5.11.1.2.1.1   User Affiliates to an MCPTT Group

Prerequisites:

- User 1 has not affiliated to the MCPTT group.

- The MCPTT AS does not have a subscription for document changes to the MCPTT group.

- The MCTT AS has a subscription for document changes in the MCPTT User’s Affiliation XDMS.

- Summary:

  - Steps 1&2: MCPTT Client 1 adds an MCPTT Group to their MCPTT Affiliation XDMS.

  - Steps 3&4: The MCPTT AS receives a notification of a change to the Affiliation List XDMS.

  - Step 5&6: The MCPTT AS writes the user’s MCPTT identity into the indicated MCPTT Group XDMS affiliation authorization rule.

  - Step 7&8: The MCPTT AS subscribes to document changes to the indicated MCPTT Group XDMS.

  - Steps 9&10: The MCPTT AS receives a notification of a change to MCPTT Group XDMS.  The notification contains the identities of the group members who are affiliated.

  - Step 11&12: The MCPTT AS updates the user’s MCPTT Affiliation XDMS for the members of the group.

  - Steps 13&14: MCPTT Client 1 receives change notifications from its MCPTT Affiliation XDMS.

## 5.11.1.2.1.2  Another MCPTT Group Affiliates to the Group

Prerequisites:

- MCPTT Client 1 has a subscription to document changes to the authorization rule of the group, and thus can track affiliation status in that group.

- Step 0: A second group member affiliates according to the flow of the previous section

- Summary:

  - Steps 1&2: The MCPTT AS receives a notification of a change to the Affiliation List XDMS.

  - Step 3&4: The MCPTT AS updates the user’s MCPTT Affiliation XDMS for the members of the group.

  - Steps 5&6: MCPTT Client 1 receives change notifications from MCPTT Affiliation XDMS reflecting the group member that are affiliated.