SA WG6 Meeting #4	S6-150470

25 - 29 May 2015, Fukuoka, Japan	(revision of S6-150427)

Source:	Intel, General Dynamics UK Ltd

Title:	Solution for MCPTT operation in IOPS mode

Document for:	Approval

Agenda Item:	6.5

Work Item / Release:	MCPTT / Rel-13

Abstract of the contribution: Based on the incoming LS from SA2 (S6-150349), this contribution proposes a new solution for MCPTT operation in IOPS mode.

# 1	Discussion

The incoming liaison from SA2 (S2-151424/S6-150349) states that as part of their Stage 2 work on Isolated Operation for Public Safety (IOPS) SA2 have decided that the IOPS functionality for the total loss of backhaul case can be supported by means of a “Local EPC” that is co-sited with one of the eNBs in the network, or that is reachable by local network infrastructure (see the candidate solution in TR 23.797).

The term “Local EPC” refers to EPC functionality, consistent with the standard 3GPP EPC architecture, which is sited local to an eNB and which provides the EPC functions in the absence of a normal backhaul.

Additionally, SA2 indicates that provision of application services over an IOPS network will be based upon the LTE-Uu radio interface and EPS bearer services supported by a Local EPC (a new deployment scenario which does not allow access to the centralised MCPTT entities connected to the EPC accessible when not in IOPS mode of operation). An IOPS network basically provides local IP connectivity services.

SA2 requests that SA6 take this into account in their specification of MCPTT services, given the requirements for interoperability with MCPTT services as specified in the IOPS service requirements TS 22.346.

# 2	Proposal

Based on the SA2 request it is proposed to agree the proposed “MCPTT for IOPS” solution for inclusion in TR 23.779.

############################## START OF FIRST CHANGE  ##############################

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

GMF	Group Management Function

IOPS	Isolated E-UTRAN Operation for Public Safety

MCPTT	Mission Critical Push To Talk over LTE

MCPTT AS	MCPTT Application Server

NGCN	Next Generation Corporate Network

OMA	Open Mobile Alliance

P25	Project 25

PCPS	Push to Communicate for Public Safety

POC	Push To Talk Over Cellular

PS-UDF	Public Safety User Data Function

PTT	Push To Talk

TCCE	TETRA and Critical Communications Evolution

TETRA	Terrestrial Trunked RAdio

XDMS	XML Documentation Management Server

############################### END OF FIRST CHANGE  ###############################

############################# START OF SECOND CHANGE  #############################

# 5	Candidate Solutions

## 5.X	MCPTT for Isolated E-UTRAN Operation for Public Safety (IOPS)

### 5.X.1	Solution X-1: Local SIP Core and Local MCPTT server

#### 5.X.1.1	Functional Description

The solution for MCPTT for IOPS described in this clause is straightforward.

Figure 5.x.1.1-1: MCPTT for IOPS with local SIP core and local MCPTT server

As illustrated in Figure 5.x.1.1-1 it is assumed that:

-	In the event of total loss of backhaul connection to the Macro Evolved Packet Core (Macro EPC), the E-UTRAN transitions to IOPS mode by activating a Local EPC (refer to TR 23.797) and establishing the S1 link to the Local EPC; and

-	The Local EPC function provides access to a local SIP core and a local MCPTT server; and

-	The same architecture also applies to a Nomadic EPS (also described in TR 23.797).

Editor’s note:	The method by which the local MCPTT server and SIP core are configured for IOPS mode MCPTT service, and the information required to allow for the continuation of service in IOPS mode is a subject FFS.

#### 5.X.1.2	Procedures

##### 5.X.1.2.1	General

It is expected that the procedures for MCPTT for IOPS will largely follow the procedures for on-network operation. The main impact is expected on security due to the total absence of backhaul connection towards any centralised repository of security credentials.

Editor's note:	The security aspects of MCPTT for IOPS are FFS and need to be defined in SA3.

##### 5.X.1.2.2	High-level call flow for MCPTT for IOPS

Depicted in Figure 5.x.1.2.2-1 is a high-level call flow describing how an IOPS-enabled UE switches from normal mode MCPTT to IOPS mode MCPTT and vice versa, following the underlying EPS transitions between normal and IOPS mode.

Figure 5.x.1.2.2-1: Operation of MCPTT for IOPS

The call flow is based on the call flow in TR 23.797 Figure 6.1.2.1-1.

The only new steps are steps 6, 7 and 11.

Editor’s note:	The exact information transmitted between the Local SIP core and the Local MCPTT server within the third party registration in step 7 to allow for IOPS mode MCPTT service is a subject FFS.

#### 5.X.1.3	Impact on Existing Entities and Interfaces

Editor's note:	Impacts on existing nodes or functionality will be added.

#### 5.X.1.4	Solution Evaluation

Editor’s note:	The fulfilment of requirements will be evaluated.

############################## END OF SECOND CHANGE  ##############################