3GPP TSG-SA WG6 Meeting #8	S6-151433

Anaheim, USA, 16th – 20th November 2015	(revision of S6-151xxx)

Title:	Information elements for configuration requests and responses

Agenda Item:	7.1.1

Source:	Vodafone

Contact:	peter.dawes@vodafone.com

Abstract: The configuration signalling flows in clause 10.1.1 do not contain tables of the parameters passed. Passes parameters are listed in tables, with references to the relevant Annex B sub-clause that describes each configuation data type.

***change***

# 10	Procedures and information flows

## 10.1	MCPTT configuration

Editor's note:	This subclause is a placeholder for description of configuration for both on-network and off-network.

### 10.1.1	General

### 10.1.2	MCPTT UE configuration data

#### 10.1.2.1	General

Editor's note:	The functional description is FFS.

#### 10.1.2.2	MCPTT UE initial configuration

The procedure for initial configuration of the UE is illustrated in figure 10.1.2.2-1.

Pre-conditions:

-	The MCPTT UE has secure access to the configuration management server.

Figure 10.1.2.2-1: MCPTT UE initial configuration (UE initiated)

1,	The configuration management client sends a get UE configuration request to the configuration management server. This message provides the MCPTT user's identity.

2.	The configuration management server obtains the UE configuration information.

3.	The configuration management server sends a get UE configuration response to the configuration management client. This message carries the UE configuration associated with this MCPTT user's identity.

Table 10.1.2.2-1: Get UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user requesting the UE configuration |

Table 10.1.2.2-2: Get UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| UE configuration data | M | UE configuration data for the user identified in the Get UE configuration request as described in B.2 |

### 10.1.3	MCPTT user profile

#### 10.1.3.1	General

#### 10.1.3.2	Default user profile for off-network operation

#### The default user profile for off-network operation is described in Annex B.

#### 10.1.3.3	MCPTT user obtains the user profile (UE initiated)

The procedure for MCPTT user obtaining the user profile that is initiated by MCPTT UE is illustrated in figure 10.1.3.3-1.

Pre-conditions:

-	The MCPTT user has performed user authentication in identity management server.

-	The MCPTT UE has the secure access to the configuration management server.

# Figure 10.1.3.3-1: MCPTT user obtain the user profile (UE initiated)

1.	The configuration management client sends get MCPTT user profile request to the configuration management server. This message provides the MCPTT user's identity information.

2.	The configuration management server obtains the MCPTT user profile information.

3.	The configuration management server sends get MCPTT user profile response to the configuration management client. This message carries the MCPTT user profile information that is associated with this MCPTT user's identity.

Table 10.1.3.3-1: Get MCPTT user profile request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user requesting the user profile |

Table 10.1.3.3-2: Get MCPTT user profile response

| Information element | Status | Description |
| --- | --- | --- |
| user profile | M | user profile of the user identified in the Get MCPTT user profile request as described in B.3 |

#### 10.1.3.4	MCPTT user obtains a user profile update (network initiated)

The procedure for MCPTT user obtaining the user profile update that is initiated by network is illustrated in figure 10.1.3.3-1.

Pre-conditions:

-	The MCPTT user has performed user authentication in identity management server.

-	The MCPTT UE has the secure access to configuration management server.

-	The MCPTT user has previously obtained the MCPTT user profile that is associated with this MCPTT user's identity.

# Figure 10.1.3.4-1: MCPTT user obtain the user profile update (network initiated)

1.	The configuration management server obtains the MCPTT user profile update information.

2.	The configuration management server sends a notification for MCPTT user data update to the configuration management client.

3.	The configuration management client sends get MCPTT user profile request to the configuration management server.

4.	The configuration management server sends get MCPTT user profile response to the configuration management client. This message carries the MCPTT user profile update information requested in step 3.

Table 10.1.3.4-1: Notify MCPTT user profile update

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the target user of the user profile update |

Table 10.1.3.4-2: Get MCPTT user profile request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user requesting the user profile |

Table 10.1.3.4-3: Get MCPTT user profile response

| Information element | Status | Description |
| --- | --- | --- |
| user profile | M | user profile of the user identified in the Get MCPTT user profile request as described in B.3 |

#### 10.1.3.5	MCPTT user upload the user profile data

The procedure for MCPTT user uploading user data that is related to the MCPTT user profile is illustrated in figure 10.1.3.5-1.

Pre-conditions:

-	The MCPTT user has performed user authentication in identity management server.

-	The MCPTT UE has the secure access to configuration management server.

# Figure 10.1.3.5-1: MCPTT user upload the user data

1.	The configuration management client is triggered (e.g. by user interaction operation) to upload the MCPTT user data.

2.	The configuration management client sends an upload MCPTT user data request to the configuration management server. The message carries the user data information to be uploaded.

3.	The configuration management server stores the MCPTT user data to the database.

4.	The configuration management server sends an upload MCPTT user data response to configuration management client to confirm the upload is completed.

Table 10.1.3.5-1: Upload MCPTT user data request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user sending the user profile update |
| user profile | M | user profile of the user identified in the Get MCPTT user profile request as described in B.3 |

Table 10.1.3.5-2: Upload MCPTT user data response

| Information element | Status | Description |
| --- | --- | --- |
| operation result | M | inidication of success or failure of user profile update |

### 10.1.4	MCPTT group configuration management

#### 10.1.4.1	Store group configuration data at the group management server

The procedure for store group configuration data at the group management server is described in figure 10.1.4.1-1.

Pre-conditions:

-	The group management server may have some pre-configuration data which can be used for online group configuration validation;

Figure 10.1.4.1-1: Store group configurations at group management server

1.	The group configuration data is received by the group management client of an authorized user.

2.	The received group configuration data is sent to the group management server for storage using a store group configuration request.

3.	The group management server may validate the group configuration data before storage.

4.	The group management server stores the group configuration data.

5.	The group management server provides a store group configuration data response indicating success or failure. If any validation or storage fails, the group management server provides a failure indication in the store group configuration data response.

Table 10.1.4.1-1: Store group configuration data request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user requesting to store group configuration |
| Group configuration data | M | Group configuration data as described in B.4 |

Table 10.1.4.1-2: Store group configuration data response

| Information element | Status | Description |
| --- | --- | --- |
| operation result | M | inidication of success or failure of operation |

#### 10.1.4.2	Store group configuration data at the group management client

The procedure for store group configuration at the group management client is described in figure 10.1.4.2-1.

Pre-conditions:

-	The group management server may have some pre-configuration data which can be used for online group configuration validation;

Figure 10.1.4.2-1: Store group configuration data at group management client

1.	The group configuration data is received by the group management server.

2.	The group management server may validate the group configuration data before storage.

3.	The group management server stores the group configuration data.

4.	The received group configuration data is sent to the group management client for storage using a notify group configuration request.

5.	The group management client stores the group configuration data.

6.	The group management client provides a store configuration data response indicating success or failure.

Table 10.1.4.2-1: Store group configuration data request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user requesting to store group configuration data |
| Group configuration | M | Group configuration data as described in B.4 |

Table 10.1.4.2-2: Store group configuration data response

| Information element | Status | Description |
| --- | --- | --- |
| operation result | M | inidication of success or failure of store group configuration data |

#### 10.1.4.3	Subscription and notification for group configuration on CSC-2

The procedures for subscription and notification group configuration on CSC-2 are described in figure 10.1.4.3-1 and 10.1.4.3-2 respectively.

Pre-conditions:

-	The group management server has some group configurations stored.

Figure 10.1.4.3-1: Subscription for group configurations

1.	The group management client subscribes to the group configuration information stored at the group management server using the subscribe group configuration request.

2.	The group management server provides a subscribe group configuration response to the group management client indicating success or failure of the request. Also, the response will provide the latest group configuration information to the group management client.

Table 10.1.4.3-1: Subscribe group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | Identity of the user requesting to store group configuration data |

Table 10.1.4.3-2: Subscribe group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| group configuration | M | group configuration data |

Figure 10.1.4.3-2: Notification for group configurations

1.	The group management server will update the group configurations stored as a result of configuration updates for the group by administrators or authorized users.

2.	The group management server provides the notification to the group management client, who previously subscribed for the group configuration information.

Table 10.1.4.3-3: Notify group configuration

| Information element | Status | Description |
| --- | --- | --- |
| group configuration | M | group configuration data as described in B.4 |

## 10.2	MCPTT group and membership configuration data

### 10.2.1	General

Editor's note:	The functional description is FFS.

### 10.2.2	Procedures

Editor's note:	The procedures description is FFS.

### 10.2.3	Structure of group and membership configuration data

The group configuration data includes the following information elements:

-	MCPTT group identity;

-	group owner;

-	group call ongoing or not;

-	group policy;

-	pre-emption capability;

-	pre-empted capability;

-	group priority;

-	associated relay service code (as specified in 3GPP TS 23.303 [8]);

-	priority level of the group;

-	the security level of the group;

-	ProSe Layer-2 Group ID (optional);

-	ProSe Group IP multicast address (optional); and

-	indication whether the UE should use IPv4 or IPv6 for that MCPTT group (optional).

The group membership data includes the following information elements:

-	MCPTT ID;

-	User Info Id (as specified in 3GPP TS 23.303 [8]);

NOTE:	User Info Id is unique (identifying an MCPTT user) to enable mapping by the application from the MCPTT ID to the IP address assigned by the ProSe layer for off-network operation.

-	user priority;

-	participant type (first responder, second responder, dispatcher, dispatch supervisor, MCPTT administrator); and

-	affiliation status.