3GPP TSG-SA WG6 Meeting #8	S6-151468

Anaheim, USA, 16th – 20th November 2015	(revision of S6-151354)

Title:	Adding missing tables to Annexes B and C – data applicable to both on- and off-network

Agenda Item:	7.1.1

Source:	Vodafone

Contact:	peter.dawes@vodafone.com

Abstract: Annex B/C currently contain 30 configuration items, but not all Stage 1 configuration requirements are met. This p-CR adds the missing configuration data to Annex B/C.

This p-CR covers only items that apply to both on- and off-network. Other p-CRs are needed to cover on-network only and off-network only.

Annex B (normative):
MCPTT related data

## B.1	Data classification

Tables B.2-1, B2-2, B.3-1, B3-2, B4-1 and B.4-2 summarise the pieces of data with the reference to the introduction requirements from 3GPP TS 22.179 [2] for on-network use, related respectively to MCPTT UEs, MCPTT users and MCPTT groups. Tables BX-1 relate to the requirements valid both for on-network and for off-network (clause 5 of 3GPP TS 22.179 [2]) and tables BX-2 relate to the requirements valid for on-network (clause 6 of 3GPP TS 22.179 [2]). The various columns in the tables have the following meanings:

-	Reference: Is the reference of the corresponding requirement in 3GPP TS 22.179 [2], leading to the introduction of a given piece of data. Only one reference if given for each piece of data, even if several requirements in 3GPP TS 22.179 [2] mandates the same data or a slight variant of it.

-	Definition: A short definition of the semantics of the corresponding piece of data.

-	UE provisioned: The piece of data has to be known by the UE for the fulfilment of the corresponding stage 1 requirement. The rate of update in the UE may be low or moderate, meaning that a quasi-real time notification of a change of state to the UE is not required.

-	UE notified: The piece of data has also to be known by the UE, but a quasi-real time notification is required.

NOTE:	UE notification is assumed to use SIP signalling.

-	Network managed/enforced: This piece of data has to be known by the network. It is either been known only by the network, or it may also be known by the UE but its use is controlled and/or enforced by the network.

Editor's note: it is FFS if another SIP reference point is needed.

Editor's note: Following tables are incomplete.

## B.2	MCPTT UE related data

Table B.2-1 summarises the data related to UE. The only information which is assumed to be promptly notified to the MCPTT UE is its disabling/enabling status.

Table B.2-1: MCPTT UE related data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.5.2-005 | Max number of simultaneous group calls (N4) | X |  | X (CSC-4) |
| 5.5.2-006 | Max number of transmissions (N5) in a group | X |  | X(CSC-4) |
| 5.5.2-007 | Max number of private calls(N10) | X |  | X(CSC-4) |

Table B.2-2: MCPTT UE related data (on network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified () | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 6.13.4-001 | Enabling/disabling of UE |  | X | X(CSC-4) |

## B.3	MCPTT user related data

Table B.3-1 summarises the data related to an MCPTT user.

Table B.3-1: MCPTT user related data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.5.2-008 | Max number of simultaneously received group calls (N6) | X |  | X(CSC-2) |
| 5.5.2-009 | Max number of transmissions (N7) | X |  | X CSC-2) |
| 5.6.5-001 | Authorised to make private call with manual commencement | X |  | X(CSC-4) |
| 5.6.5-003 | User which can be called in private call | X |  | X(CSC-4) |
| 5.6.5-002 | Authorised to make private call with automatic commencement | X |  | X(CSC-4) |
| 5.6.5-004 | Max private call duration | X |  | X(CSC-4) |
| 5.6.5-005 | Max hang time for private calls | X |  | X(CSC-4) |
| 5.6.5-006 | Restriction in notification of call failure reason for private call | X |  | X(CSC-4) |
| 5.7.2.2.2-002 | Authorisation for imminent in- peril cancelation | X |  | X(CSC-4) |
| 5.7.2.3.1-001 | Authorisation to make an emergency private call | X |  | X(CSC-4) |
| 5.7.2.3.2-001 | Authorisation to cancel emergency priority in a private emergency call | X |  | X(CSC-4) |
| 5.7.2.4.1-002 | Authorisation to activate emergency alert | X |  | X(CSC-4) |

Table B.3-2: MCPTT user related data

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |

## B.4	Group related data

Tables B.4-1 and B4-2 summarise the data related to an MCPTT group.

Table B.4-1: MCPTT group related data (both on network and off network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.7.2.1.2-002 | Timeout value for the cancellation of an in progress emergency for a group call | ? |  | X(CSC-2) |
| 5.7.2.2.2-004 | Timeout value for the cancellation of an in progress imminent-peril group call | ? |  | X(CSC-2) |

Table B.4-2: MCPTT group related data (on network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 6.2.4-003 6.4.9-002 | Group call Hang timer |  |  | X(CSC-4) |

Table B.4-3: MCPTT related data (off network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-7.4-002] [R-7.4-003] | Group call Hang timer |  |  | X(CSC-4) |

## B.5	Service related data

Tables B.5-1, B.5-2 and B.5-3 summarise the data related to an instance of an MCPTT service.

Table B.5-1: Service related data (both on-network and off-network)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| 5.7.2.3.2-002 | Timeout value for the cancellation of an in progress emergency for a private call | X |  | X(CSC-2) |

Table B.5-2: Service related data (on-network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
|  | Hang timer for private calls |  |  |  |

Table B.5-3: Service related data (off-network only)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-7.4-002] [R-7.4-003] | Hang timer for private calls in off-network |  |  |  |

Annex C (informative):
Local UE settings for MCPTT

## C.1	Local UE settings for MCPTT

Table C.1-1 details local UE settings for MCPTT that represent important functionalities that may be required by MCPTT UEs, however these functionalities do not require central configuration like the parameters in annex B, so these are detailed in this annex for information to UE vendors interested in producing MCPTT UEs. The various columns in the tables have the following meanings:

-	Reference: Is the reference of the corresponding requirement in 3GPP TS 22.179 [2].

-	Definition: A short definition of the local UE setting.

Table C.1-1: Local UE settings for MCPTT

| Reference | Definition |
| --- | --- |
| 5.1.1-003 | Call reception signalling method |
| 5.1.1-004 | Disabling of call reception signalling |
| 5.8-003 | Configuration of display of User ID (on/off) |
| 5.8-003 | Configuration of display of aliases associated with the User ID (on/off) |
| 5.8-003 | Configuration of display of Selected MCPTT Group (on/off) |
| 5.8-003 | Configuration of display of Mission Critical Organization name (on/off) |
| 6.8.7.4.1-003 | Configuration of the notification of an emergency alert |

# Additional tables – on- and off-network (to be re-located in spec.)

Table B.X-n: Group/status information

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.1.3-001] | who can be a member of specific MCPTT Groups, so that those MCPTT Groups shall be inaccessible to other users, including dispatchers or supervisors. |  |  | X (CSC-4) |

Table B.X-n: Membership/affiliation

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.1.5-004] | limit the total number (N2) of MCPTT Groups that an MCPTT User can be affiliated to simultaneously. | X |  | X (CSC-4) |

Table B.X-n: Group Call administration

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.1.6-001] | maximum duration for MCPTT Group Calls for MCPTT Users within MCPTT administrator authority. | X |  | X (CSC-4) |

Table B.X-n: Group-Broadcast Group (e.g., announcement group)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] | Levels of group hierarchy for Group-Broadcast Groups (B1) |  |  | X |

Table B.X-n: User-Broadcast Group (e.g., System Call)

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.2.3-001] | Levels of user hierarchy for User-Broadcast Groups (B2) |  |  |  |

Table B.X-n: MCPTT Emergency Group Call

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.7.2.1.1-013] | MCPTT Group (i.e., user’s selected group or dedicated MCPTT Emergency Group) that is used for the MCPTT Emergency Group Call by an MCPTT User. |  |  |  |
| [R-5.7.2.1.2-005] | Authorize an MCPTT User to cancel in-progress Emergencies. |  |  |  |
| [R-5.7.2.2.1-009] | MCPTT Group (i.e., user’s selected group or dedicated imminent peril group) that is used for the Imminent Peril communications |  |  |  |

Table B.X-n: MCPTT Emergency Alert

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.7.2.4.1-005] | configure a geographical area for an MCPTT Emergency Alert to send a notification to MCPTT Users of the MCPTT User entering the MCPTT Emergency State, independent of the MCPTT Group Membership. |  |  |  |
| [R-5.7.2.4.1-005] | Authorized to configure a geographical area for an MCPTT Emergency Alert to send a notification to MCPTT Users of the MCPTT User entering the MCPTT Emergency State |  |  |  |
| [R-5.7.2.4.1-008] | Group (i.e., user’s selected group or dedicated MCPTT Emergency Group) or MCPTT User that is used for the MCPTT Emergency communications. |  |  |  |
| [R-5.7.2.4.1-010] [R-5.7.2.4.1-011] | MCPTT User configures, subject to MCPTT Service policy, how they are notified of an incoming MCPTT Emergency Alert (e.g., visual, audio). |  |  |  |
| [R-5.7.2.4.1-012] | MCPTT Group (i.e., user’s selected group or dedicated MCPTT Emergency Group) or MCPTT User (e.g., dispatcher) that is used for the MCPTT Emergency Alert by an MCPTT User. |  |  |  |

Table B.X-n: User ID

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.8-001] | creation and deletion of aliases for an MCPTT User and its associated user profiles by authorized parties. |  |  |  |
| [R-5.8-002] | Minimum length (N3) of an alphanumeric identifier (i.e., alias) assigned by an MCPTT Administrator. |  |  |  |

Table B.X-n: Security

| Reference | Definition | UE provisioned (CSC-4) | UE notified | Network enforced/managed |
| --- | --- | --- | --- | --- |
| [R-5.13-012] | pre-provisioning and over-the-air provisioning of cryptographic keys |  |  |  |

Additional tables – on-network only (to be re-located in spec.)

User configuration data / MCPTT user profile data (on network)

| Reference | Parameter description | Is offline configuration possible? |
| --- | --- | --- |
| [R-6.1-007] | Geographic areas associated to a dispatcher for the purpose of routing Location dependent calls and alerts (MCPTT Private Call requests, MCPTT Group Calls) |  |
| [R-6.2.1-003] | MCPTT user acknowledgement: direct interaction/automatically executed |  |
| [R-6.2.1-005] | MCPTT users must acknowledge receipt of the MCPTT Group call setup request before the audio transmission proceeds, regardless of affiliation state. |  |
| [R-6.2.3.1-003] | Number (maximum of N9) of simultaneous audios received by MCPTT user in a single MCPTT group. |  |
| [R-6.2.3.2-001] | Authorized to request to transmit to an MCPTT group or an individual participant. |  |
| [R-6.2.3.3.3-001] | Authorized to listen to both the overriding and overridden participant transmissions (applies to MCPTT group configured to allow both overriding and overridden transmitting participants). |  |
| [R-6.4.5-002] | Authorized to request the list of currently affiliated members on an MCPTT group. |  |
| [R-6.4.6.1-001] | Authorized to change an on-network MCPTT user's selected MCPTT group(s). The MCPTT service sends a notification to the on-network MCPTT user. |  |
| [R-6.4.6.2-001] | Authorized to propose that another on-network MCPTT user should affiliate to a specific MCPTT group. |  |
| [R-6.4.6.2-002] | Authorized to propose that multiple other on-network MCPTT users should affiliate to a specific MCPTT group. |  |
| [R-6.4.6.2-003] | Authorized to propose that another on-network MCPTT user should select a specific MCPTT group. |  |
| [R-6.4.6.2-004] | Authorized to propose that multiple on-network MCPTT users should select a specific MCPTT group. |  |
| [R-6.6.1-004] | Authorized to perform dynamic regrouping operations. |  |
| [R-6.7.1-003] | Authorized to place a private call (without floor control). |  |
| [R-6.7.1-011] | Geographic areas associated to a dispatcher for the purpose of routing a private call to a local dispatcher based on the MCPTT user's current location. |  |
| [R-6.17-001] | Authorized to make and/or receive telephony calls. |  |
| [R-6.17-002] | Interaction between telephony calls and MCPTT calls |  |
| [R-6.17-003] | Authorized to use telephony services to block incoming telephony calls. |  |

Group configuration data (on network)

| Reference | Parameter description | Is offline configuration possible? |
| --- | --- | --- |
| [R-6.2.1-011] | Call setup timeout applicable when an MCPTT group call setup request requires acknowledgement from affiliated MCPTT group members. |  |
| [R-6.2.3.2-006] | Depth of the floor control queue |  |
| [R-6.2.3.2-012] | Parameter(s) of the floor control queue (i.e., timer). |  |
| [R-6.2.3.3.1-003] | Allow only the overriding participant to transmit/allow both the overriding and overridden participant to transmit. |  |
| [R-6.2.4-001] | Terminate/continue the call if a participant of an MCPTT group call is pre-empted (an indication is sent to the transmitting participant that one or more receiving participants was pre-empted). |  |
| [R-6.4.9-003] | Maximum allowed time for MCPTT group calls to remain active. |  |
| [R-6.4.9-004] | Number of MCPTT users remaining for an MCPTT group call to remain active. |  |
| [R-6.6.4.2-003] | Parameters for a particular preconfigured user regroup operation, e.g. activate with minimal delay. An example is to communicate with particular other MCPTT users within a predefined radius for urgent calls such as MCPTT emergency group calls. |  |
| [R-6.8.1-004] | Pre-emption hierarchy for MCPTT group transmissions and their associated users. |  |

Service configuration data (on network)

| Reference | Parameter description | Is offline configuration possible? |
| --- | --- | --- |
| [R-6.2.3.3.1-004] | For MCPTT private calls (with floor control) allow only the overriding participant/both the overriding and overridden participant to transmit. |  |
| [R-6.4.9-001] | Conditions under which MCPTT calls shall be terminated (e.g., last participant leaving, second last participant leaving, initiator leaving). |  |
| [R-6.7.2-001] | Presence of a particular MCPTT user is available/not available |  |
| [R-6.8.7.4.1-003] | User emergency notification (e.g., visual/audio). |  |
| [R-6.12-003] | User restricted dissemination of location information. |  |