3GPP TSG-SA WG6 Meeting #14	S6-161379

Reno, Nevada, USA, 14th – 18th November 2016	(was S6-161187,160964)

Source:	Motorola Solutions

Title:	pCR TR 23.781 – solution for user configuration and authorization for interconnection

Specification:	TR 23.781 v0.2.0

Agenda Item:	7.2  FS_MCSMI

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

Abstract: Provides a solution to user configuration and authorization for key issues 1.1, 1.3 and 1.4.

This pCR proposes adding a solution that adds authorization for interconnection calls with users and groups in listed partner systems to user configuration tables.

***************************************** Change 1 **************************************************

## 6.x	Solution #: User authorization and configuration for interconnection

### 6.x.x	Description

This solution applies to aspects of the following key issues:

-	1.1  MC system authorization to support interconnection calls.

-	1.3 Authorization for interconnection private calls.

-	1.4: Affiliation and authorization for interconnection MC service group calls.

A set of configuration information will be added to user configuration data (on-network) to provide a set of permissions for the different services available to an MC user for interconnection calls.  The list will contain information on a per-partner MC system basis, to allow the service that is available for interconnection calls with one partner MC system to be different to that available for a different partner MC system.

Editor's note: the procedure for managing the configuration information is FFS. Group related information may need to include information provided by the partner system.

For each partner MC system with which the MC user is permitted some interconnection functionality, the configuration parameters should include:

-	Application identity of that MC system

NOTE:	The application identity of the MC system is the domain portion of the MC service IDs for whom the system is primary.

-	Addressing information for relevant entities that enable communication with that partner MC system, This may include addressing information for signalling plane and application plane as needed.

NOTE:	It is possible that no entities in another system are directly addressable in an interconnection scenario, and the addresses may refer to local entities by which users and groups in the partner MC system can be reached.

-	Whether MCPTT private calls are permitted with users in that partner MC system.

-	List of MC users in that partner MC system to whom private calls may be made.

-	List of MC users in that partner MC system from whom private calls may be received.

-	Indication whether private calls can be made to users of that partner MC system who are not included in the list.

-	Indication whether private calls can be received from users of that partner MC system who are not included in the list.

-	List of groups in that partner MC system of which the MC user is a member

-	Address information for the group management server from which the group profile may be obtained.

-	List of MC services for which the group applies (MCPTT, MCVideo, MCData).

-	Whether MCVideo calls are permitted with users in that partner MC system.

-	List of MC users in that partner MC system to whom MCVideo calls may be made.

-	List of MC users in that partner MC system from whom MCVideo calls may be received.

-	Indication whether MCVideo calls can be made to users of that partner MC system who are not included in the list.

-	Indication whether MCVideo calls can be received from users of that partner MC system who are not included in the list.

-	Whether MCData transactions are permitted with users in that partner MC system.

-	List of MC users in that partner MC system to whom MCData transactions may be initiated.

-	List of MC users in that partner MC system from whom MCData transactions may be initiated.

-	Indication whether MCData transactions can be initiated to users of that partner MC system who are not included in the list.

-	Indication whether MCData transactions can be initiated from users of that partner MC system who are not included in the list.

### 6.x.x	Impacts on existing nodes and functionality

User configuration data will need to be extended to incorporate the information listed in the previous subclause.

Editor's note: this may affect configuration servers within each of the three services, and/or the common functional architecture

### 6.x.x	Solution Evaluation

Editor's Note: Use this section for evaluation at solution level.

***************************************** End change 1 ************************************************