3GPP TSG-SA WG6 Meeting #15	S6-170016

Jeju Island, South Korea, 13th – 17th February 2017	(was S6-161381,161188,160965)

Source:	Motorola Solutions

Title:	pCR TR 23.781 – Solution for user profile management during migration

Specification:	TR 23.781 v0.2.0

Agenda Item:	8.2  FS_MCSMI

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

Abstract: Provides a solution to allow user profile management between primary and partner MC system during migration.

This solution addresses aspects of key issues 2.1 and 2.7.

The primary MC system of the user needs to authorize the services available to an MC user who migrates. Similarly, a partner MC system to which the user migrates will also want to apply local authorizations to that user, which may override some of the permissions requested by the primary system.

When the MC user migrates, the user must register for service with nominated MC service servers in the partner system. These must be pre-configured in the user's profile(s) for migration before migration takes place.

Following the request for registration, the partner MC system will need to obtain a profile for the MC user from the primary MC system of that user, and may then apply local configuration requirements for that user. These local configuration requirements may be dependent only on the primary system of that user (i.e. all users from the same MC system may have the same local rule set applied), or may be more granular for example related to the organisation of which the MC user is a member, provided that the structure of the MC service ID of the user allows the partner MC system to determine the organisation of the MC user.

To achieve this, the partner MC system uses a template which can be applied to the profiles obtained from the primary MC systems of the migrating MC users. The resulting profile, based on the profile received from the primary system and the template, is then provided to the MC service servers in the partner MC system, and downloaded to the migrating MC client.

***************************************** Change 1 **************************************************

## 6.x	Solution #: User profile management for migration

### 6.x.x	Description

This solution applies to aspects of the following key issues:

-	2.1: MC authorization for migration.

-	2.7: Authorization, configuration and affiliation for MC group calls during migration

The principles of this solution apply to all three services MCPTT, MCVideo and MCData.

Figure 6.x.x-1 below illustrates the entities involved in the solution:

Figure 6.x.x-1: Entities and reference points for migrated user profile configuration

Prior to migration, the primary MC system will be configured with at least one profile for the MC service user to use during migration.

When the MC service user migrates to a partner MC system, the migrating MC service user will first need to authenticate and obtain a suitable credential to authorize access to services in the partner MC system. This authentication process is not described within this solution. The authentication process may also provide the MC service user with a temporary identity for use within the partner MC system. Following authentication, for access to MC services, the migrating MC service client registers for service with the partner MC service server. It will present both the temporary identity (if provided) and the MC service ID of the client.

NOTE:	In order to gain access to the partner system, the MC service client will be configured with the information needed to gain access to the partner  MC system, such as appropriate PDN, SIP core addressing and credentials for access.  This information is described in more detail in solution xx.

Editor's note:	The authentication and authorization mechanisms needed to allow the MC service client to gain access to the servers in the application plane of the partner MC system are within the remit of SA3.

When the MC service client registers, the MC service server needs to retrieve the MC service user profile(s) for that MC service user, and so the MC service server requests the profile(s) from the MC service user database using the identity presented by the MC service client, which may be a temporary identity provided by the authentication process as well as the MC service ID of the client.

NOTE:	It is FFS whether the MC service user database can cache a user profile for that migrating MC service user, if the MC service user has previously migrated to that partner MC system, for example within a defined time window. The MC service database could retain a cached profile, and only delete this when it receives an indication that the MC service user is no longer needing service from this MC system, for example following the MC service user registering on either its primary MC system, or another partner MC system. While the MC service user is receiving service or considered to need service within this MC system, the cached profile could be updated as needed by changes in the local template or in the profile received from the primary system of the MC user.

The MC service user profile that is provided to the partner MC service server and to the migrating MC service client needs to contain a combination of authorizations from the primary system of the MC service user, and the authorizations granted by the partner MC service system. The partner system configuration management server will request an MC service user profile for the MC service user from the primary system configuration management server. The reference point on Figure 1 above is shown as CSCM-x (CSC for Migration x). The primary system configuration management server will request profile(s) for the user from the MC service user database in the primary MC service system, and will then provide one or more user profiles for that MC service user to the partner MC system configuration management server.

Editor's note: 	The interaction between the configuration management server in the partner system, the configuration management server in the primary system and the MC service server in generating the combiner user profile is FFS

It is expected that there will be specific profiles for migration defined for each MC service user, and these will be different to the profiles generally used by the MC service user when receiving service in the primary MC system. For example, the list of MC service groups accessible may be different, such that some of the MC service groups available to the MC service user when in the primary system may not be available to that MC service user in the partner MC system. Authorizations to carry out operations such as group regrouping may not be permissible in the partner MC system.

The partner MC system configuration management server will receive the profile(s) from the primary MC system configuration management server.

The partner system MC service user database will be configured with templates which are to be applied to the user profiles for migrating MC service users, and will provide these templates to the partner MC system configuration management server. There may be one template per MC system with which the partner MC system has agreements to support migrating MC users, or multiple templates for visitors from the same MC system, e.g. based on the organizations of the MC users. To support multiple templates for migrating MC service users from the same MC system, the partner MC system must be able to derive an organizational structure from the MC service ID of the visiting MC user. The partner MC system configuration management server will combine the user profile(s) from the primary system of the migrating MC user with the appropriate template to produce the user profile(s) that will be used by the MC user while migrating. The combined user profile(s) will be sent to the MC service user database.

Following registration with the partner MC service server, the MC service client requests and is provided with the MC service user profile(s) for use in that partner system during migrated service from the partner MC system configuration management server.

The template applied in the partner system to modify the user profile received from the primary system of the MC user may include modifications such as:

-	Restriction for access to certain MC service groups

-	Addition of further MC service groups provisioned with the partner MC system for specific operational functions such as mutual aid

-	Modification or replacement of configured user priority with an appropriate priority for use within the partner system.

-	Restriction of use of certain functions based on security policy, e.g. authorisation to obtain presence list, authorisation to override current talking party, authorisation to invoke enable/disable.

-	Limitations on geographical area of operation

### 6.x.x	Impacts on existing nodes and functionality

The configuration management server will need to support an additional reference point, CSCM-x, for communication with configuration management servers in partner MC systems to support migrated MC user profiles.

The MC service user database (or relevant user databases in common services core and other MC services) will need to support templates to modify the user profiles of migrated users from partner MC systems.

The configuration management server will need to support the generation of profiles for migrated MC users from user profiles provided from the primary systems of those MC users and the modification templates.

### 6.x.x	Solution Evaluation

Editor's Note: Use this section for evaluation at solution level.

***************************************** End change 1 ************************************************