3GPP TSG-SA6 Meeting #15	S6-170073

# Jeju Island, Korea (Republic Of), 13th Feb 2017 - 17th Feb 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0012 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | URL query |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hytera Communications Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCImp-MCData |  |  |  |  |  | Date: |  |  | 2017-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 3GPP SA1 TS 22.282-e10 has the following requirements related to URL. [R-5.4.2-002] The MCData data streaming capability shall provide an option that allows each recipient to choose to receive the data stream or not (e.g. by sending a link (URL) to all relevant members). [R-5.3.2-003] The MCData file distribution capability shall provide an option for each recipient to choose to receive the file or not (e.g. by storing the file and sending a link (URL) to all relevant members). But how to query the URL of existed files needs to be addressed in TS 23.282. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding URL query procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is hard to get the URL of resource in MCData server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

7.5.2.x URL query

7.5.2.x.1	General

Editor's Note: General functional description should be described here.

7.5.2.x.2	Information flows

7.5.2.x.2.1	Information flows for URL query request

Table 7.5.2.x.2.1-1 describes the information URL query request from the MCData client to the MCData server.

Table 7.5.2.x.2.1-1 MCData URL query request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the calling party |
| Query criteria | M | The query criteria, for example, file name/file size etc. |

7.5.2.x.2.2	Information flows for URL query response

Table 7.5.2.x.2.2-1 describes the information URL query reponse from the MCData server to the MCData client.

Table 7.5.2.x.2.2-1 MCData URL query response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| Result | M | Result indicates success or failure of URL query. |
| URL List | O | list which meet the criteria |

7.5.2.x.3	Procedure

The procedure in figure 7.5.2.x.3-1 describes the case where an MCData user is querying URL stored in MCData server.

Pre-conditions:

1.	The MCData user on the MCData client 1 is registered for receiving MCData service.

Figure 7.5.2.x.3-1: URL query

1.	The user at the MCData client 1 initiates a URL query request with search criteria.

2.	MCData client 1 sends the query to MCData server, who will generate a URL list based on the filter criteria.

3.	The MCData server replies the URL list to MCData client 1.

* * * End of the Change * * *